3GPP SA4 #123-e	S4-230458

17-21 April 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.806 | CR | pseudo | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_SmarTAR] RTP header extension for in-band delay measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SmarTAR |  |  |  |  |  | Date: |  |  | 17/04/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RTP header extension for supporting in-band delay measurement, as proposed in S4aI230087, was agreed at the post SA4 122 meeting MBS Telco on March 30th, 2023, but the proposal lacks important details. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of details on how to use RTP header extension to do in-band delay measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Add the following references to clause 2:

===== CHANGE #1  =====

[14] D. Singer, H. Desineni and R. Even, "RFC 8285: A General Mechanism for RTP Header Extensions", 2017.

[15] D. Mills, et al, “RFC5905: Network Time Protocol Version 4: Protocol and Algorithms Specification”, 2010.

===== END of CHANGE #1  =====

At the end of clause 6.2.3, add the following:

===== CHANGE #2  =====

RTP header extensions defined in RFC8285 [14] can be used to do timestamp piggybacking. There are two formats of RTP header extension, namely the one-byte and the two-byte formats [14].

Figure 6.2.3-3 shows the timestamp piggybacking in the RTP packet payload that allows for measuring the one-way delays in both directions as well as the round-trip time.

Fig. 6.2.3-3 In-band end-to-end delay measurement by piggybacking timestamps to an RTP packet.

The one-byte RTP header extension for piggypacking timestamps is shown in Figure 6.2.3-4. The header extension consists of a single RTP header extension element, the ID of which is set to 1, and the length L field of which is set to 2 (which indicates 3 bytes rather than 2 bytes in the one-byte format according to RFC8285 [14]), and the data of which is labeld “#timestamps, start, size” occupying 24 bits. Specifically,

- The “#timestamps” field specifies the number of timestamps. In the case of piggybacking the originate timestamp (T1), this field is set to 1; in the case of piggybacking the originate timestamp (T1), the receive timestamp (T2) and the transmit timestamp (T3), this field is set to 3.

- The “start” field specifies whether the timestamps are at the beginning of the RTP payload or at the end of the RTP payload.

- The “size” field specifies the size of the timestamps. The size depends on the timestamp formats. Considering the granularity and overhead, the 32-bit short NTP timestamp format [15] or a truncated version of it seems a good choice.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length=1            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID=1 | L=2   |     #timestamps, start, size                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Fig. 6.2.3-4 The one-byte RTP header extension for piggybacking timestamps.

The two-byte RTP header extension for piggypacking timestamps is shown in Figure 6.2.3-5. The difference is that the ID and the L fields together occupy 2 bytes rather than 1 byte as in the one-byte format. Note that the value in the L field indicates the size of the data of the RTP header extension element literally, i.e., L=2 means 2 bytes.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length=1            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  ID=1         |         L=2   | #timestamps, start, size      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Fig. 6.2.3-5 The two-byte RTP header extension for piggybacking timestamps.

For both the one-byte format and the two-byte format, the timestamp(s) are put in the RTP payload according to the respective “#timestamps, start, size” fields.

===== END OF CHANGE #2  =====