3GPP TSG-WG SA4 Meeting # 123e	S4-230501

E-meeting, April 17 – 21, 2023

Agenda Item:	10.8

Source:		Huawei, HiSilicon, KDDI

Title:		Guidelines for PDU Set identification based on existing RTP/SRTP headers, header extensions and/or payloads

Document for:	Discussion and Agreement

# Introduction

Since SA4#121, it has been agreed that SA4 will develop a solution based on a new header extension for the RTP/SRTP protocols to support the PDU Set feature. Besides, in the LS S2-2303849 [1] as shown below, SA2 also asks SA4 to provide how the PDU Set information can be derived from the existing RTP/SRTP headers, header extensions and/or payloads. It’s essential for SA4 to define the guidelines on how to identify and map to the PDU Set information. This paper intends to provide the guidelines on the identification and mapping based on the existing RTP headers, header extension and/or media payloads.

- provide how the parameters listed above (or a subset thereof) can be derived from existing RTP/SRTP headers, header extensions and/or payloads.

# Guidelines for PDU Set Information derivation

## RTP/SRTP header

Figure 1 RTP header fields as defined in RFC 3550 [2]

If the PDU Set represents a video frame, the video frame may be identified based on the RTP header fields as following:

-	With the "M" bit and the sequence number in RTP header, the Indication of End PDU of a PDU Set and PDU SN within a PDU Set/frame can be derived.

-	With the "timestamp" field and the sequence number in RTP header, the Indication of End PDU of a PDU Set and PDU SN within a PDU Set/frame can be derived.

NOTE:	When multiple RTP streams multiplexed over a single RTP session, the “M” bit may not be used to indicate the end of the video frame.

The frame size and the importance cannot be derived by the RTP header fields.

## Existing RTP header extension

### Frame Marking RTP header extension

Figure 2 Frame Marking RTP header extension fields [3]

If the PDU Set represents a video frame, the video frame may be identified based on the Frame Marking RTP Header Extension as following:

-	With the "S" and "E" bits in RTP header extension and the sequence number in the RTP header, the indication of the End PDU of a PDU Set and PDU SN within a PDU Set can be derived.

-	With the "I" bit or "D" bit in the RTP header extension, the PDU Set importance can be derived.

The PDU Set Size in bytes cannot be derived based on the Frame Marking RTP header extension fields.

NOTE: The Frame Marking RTP header extension only supports H.264/AVC codec in current release.

## RTP payload

### RTP payload for H.264/AVC codec

For a video slice with H.264 RTP payload, the PDU Set Information can be realized by following approach.

According to RFC 6184 [4], within the RTP packet, the NAL unit type in the NAL unit header can indicate the content of NAL unit, e.g. coded slice of an I frame, coded slice of a P frame, and also the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet and fragmentation unit (FU).

- For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 28.

- When NAL unit type is 28 or 29, one NAL unit is carried over multiple RTP packets. In this case, the first byte of RTP payload is also named the fragmented unit (FU) indicator and the following byte is the FU header. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the PDU Set can be identified.

Figure 3 NAL unit header format for H.264 [4]

With the RTP payload (i.e. NAL unit header and optionally FU header) and the sequence number in the RTP header, the indication of the End PDU of the PDU Set and the PDU SN within a PDU Set can be derived.

The NRI value in the NAL unit header indicates the relative transport priority. It can be used to be mapped to the PDU Set importance. Besides, different NRI values can also indicate different QoS requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

The PDU Set Size in bytes cannot be derived based on the RTP payload for a H.264/AVC codec.

### RTP payload for H.265/HEVC codec

For a video slice with H.265 RTP payload, the identification of the PDU Set can be realized by following approach.

According to RFC 7798 [5], within the RTP packet, the NAL unit header can indicate the content of NAL unit. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet (APs), fragmentation unit (FUs) and Payload Content Information (PACI) carrying RTP packet.

-	For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 49.

-	When NAL unit type is 49, one NAL unit is carried over multiple RTP packets. In this case, the first two-byte of RTP payload is also named the payload header (denoted as NAL U header) and the following byte is the FU header. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the start/end of the PDU Set can be identified.

-	When NAL unit type is 50, this is a PACI packet which may carry a single NAL unit packet or FU. In this case, the first two-byte of RTP payload is also named as the PACI header (denoted as NAL Unit header). In the following two bytes, the "A" bit is the copy of "F" bit and cType field is the copy of Type field in the PACI payload NAL unit. Then the following is the PHES field, whose length is determined by the PHSize. Finally, the following is the PACI payload NAL unit, during which the first byte is FU header when cType is 49. Therefore, based on the PACI header and PACI payload NAL unit, the start/end of the PDU Set can be identified.

Figure 4 The Structure of the HEVC NAL Unit Header [5]

Figure 5 The Structure of FU Header

With the RTP payload (i.e. NAL unit header and optionally FU header) and the sequence number in the RTP header, the indication of the End PDU of the PDU Set and the PDU SN within a PDU Set can be derived.

The Type field in the NAL unit header contains the VCL (video coding layer) NAL unit. In the VCL NAL unit, three types of IRAP pictures, namely IDR, CRA, and BLA pictures do not refer to any pictures other than themselves for inter prediction in its decoding process, and they are considered as the most important pictures, other pictures are encoded based on these pictures. Besides, for the TID (TemporalId) field in the NAL unit header when the temporal scalability streams are used, the TID value indicates (among other things) the relative importance of an RTP packet.

The above Type field and/or the TID field in the NAL unit header indicates the relative transport priority. They can be used to be mapped to the PDU Set importance. While they can also indicate different QoS requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

The PDU Set Size in bytes cannot be derived based on the RTP payload for a H.264/AVC codec.

### RTP payload for H.266/VVC codec

If the PDU Set represents a video slice with H.266 RTP payload, the identification of the PDU Set can be realized by following approach.

According to IETF RFC 9328 [6], within the RTP packet, the NAL unit header can indicate the content of NAL unit. Besides, it can also indicate the possible structures of the RTP payload, e.g. single NAL unit packet, aggregation packet (APs) and fragmentation unit (FUs).

-	For single NAL unit packet and aggregation packet, it can be easily detected that each RTP packet carries one NAL unit when the NAL unit type is less than 29.

-	When NAL unit type is 29, one NAL unit is carried over multiple RTP packets. In this case, the first two-byte of RTP payload is also named the payload header (denoted as NAL U header) and the following byte is the FU header. In the FU header, the "S" bit and "E" bit separately represents the start and end of the NAL unit. Therefore, based on the NAL unit type (also known as FU indicator for fragmented unit) and the FU header, the the UPF can identify the start/end of the same NAL unit.

Figure 6 The Structure of the VVC NAL Unit Header [6]

Figure 7 The Structure of the FU header

With the RTP payload (i.e. NAL unit header and optionally FU header) and the sequence number in the RTP header, the indication of the End PDU of the PDU Set and the PDU SN within a PDU Set can be derived.

The Type field in the NAL unit header contains the VCL (video coding layer) NAL unit. In the VCL NAL unit, three types of IRAP pictures, namely IDR, CRA, and BLA pictures do not refer to any pictures other than themselves for inter prediction in its decoding process, and they are considered as the most important pictures, other pictures are encoded based on these pictures. Besides, the LayerID field identifies the layer a NAL unit belongs to, wherein a layer may be, e.g., a spatial scalable layer, a quality scalable layer, a layer containing a different view, etc. And the TID value indicates the temporal sublayers when temporal scalability is used. The LayerID and/or TID values indicate the relative importance of an RTP packet.

The above Type field and/or the LayerID/TID field in the NAL unit header indicates the relative transport priority. They can be used to be mapped to the PDU Set importance. While they can also indicate different QoS requirements, which can be used to provide different protects against transmission losses, e.g. reliabilities (tolerable frame/slice error rate), priorities.

The PDU Set Size in bytes cannot be derived based on the RTP payload for a H.264/AVC codec.

# Proposal

It’s proposed to agree Clause 2 as guidelines for deriving the PDU Set Information and add it into the PD.

# Reference

[1]	S2-2303849, Reply LS on N6 PDU Set Identification

[2]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications", STD 64, July 2003

[3]	draft-ietf-avtext-framemarking: "IETF experimental draft-ietf-avtext-framemarking-13 - Frame Marking RTP Header Extension".

[4]	IETF RFC 6184: "RTP Payload Format for H.264 Video", May 2011.

[5]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC)", March 2016.

[6]	IETF RFC 9328: "RTP Payload Format for Versatile Video Coding (VVC) ", December 2022.