Source:	VoiceAge Corporation, Fraunhofer IIS

Title:	Test plan proposal for IVAS Selection experiment P800-7, 2 Objects

## Document for:	Discussion and Agreeement

## Agenda Item:	7.5

Introduction

EVS-8b SWB clean speech experiment [1] was used as a template for this Experiment setup proposal, which was adapted to reflect the IVAS Test Plan [2].

Proposal

Include the text below into Annex E.7 of IVAS-8a.

References

S4-141319: EVS Permanent Document EVS-8b: Test plans for selection phase including lab task specification, v.1.2.

S4-230453: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4.

S4aA230050: IVAS-7a: Processing plan for selection phase, v.0.8.1.

---------------

Annex E: P.SUPPL800 Experiments

E.7	Experiment P800-7: 2-Objects clean speech test

The purpose of this experiment is to evaluate the performances of the IVAS candidate for encoding, decoding, and rendering of two simultaneous objects with metadata. The metadata determines the position of the objects around the listener in a 3-dimensional space as described in the Scene description section below. The audio is rendered binaurally via headphones.

E.7.1	Database

- 48 kHz sampling rate

- The listening database consists in artificially created spatial audio samples from monophonic clean speech recordings where always 1 female and 1 male talker are combined in conversation-like scenarios following the Scene descriptions below.

- A leading and trailing silence is assumed for each artificially created spatial audio sample. The metadata corresponds to the whole duration of the sample. This means that for moving objects, only a part of the trajectory corresponds to active speech.

- In half of the samples, the 2nd talker’s utterance follows the 1st talker’s utterance simulating natural conversation. In the 2nd half of the samples, the situation is similar, but the utterances partially overlap. The targeted overlap should be approximately [30] %. Non-overlapping sentence pairs are used for Scenes a., c., and e. as described below. Overlapping sentence pairs are used for Scenes b., d., and f.

- The length of the created spatial audio samples is 6 s.

- Level adjusted as specified in IVAS-7a (Processing Plan) [3].

E.7.2	Descriptions of different scenes for metadata creation

    - Two talkers sitting at a table (elevation 0°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |
| 50° | 350° |

    - Two standing talkers (elevation 35°), at different azimuths. To increase positional variation, both the absolute azimuths and the difference of the azimuths of both talkers vary for each sentence pair:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |
| 50° | 350° |

    - One talker sitting at a table (elevation 0°), second talker standing beside the table (elevation 45°). Non-overlapping utterances:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 1st talker | 2nd talker |
| 0° | 50° |
| 10° | 110° |
| 20° | 170° |
| 30° | 230° |
| 40° | 290° |

    - One talker sitting at a table (elevation 0°), second talker walking around the table (elevation 45°). The azimuth of the 2nd talker varies continually, positive sense is counterclockwise:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 50° | 180° : +1° : 120° |
| 100° | 130° : +1° : 70° |
| 150° | 80° : +1° : 20° |
| 200° | 30° : -1° : -270° (90°) |
| 250° | -20° : -1° : -320° (40°) |
| 300° | -70° : -1° : -10° |

    - Two talkers walking side-by-side around the table (elevation 45°). The azimuth is the same for both talkers and varies continually:

Azimuth of the talkers

180° : +1° : 120°

130° : +1° : 70°

80° : +1° : 20°

30° : -1° : -270° (90°)

-20° : -1° : -320° (40°)

-70° : -1° : -10°

    - Two talkers walking around the table in opposite directions (elevation 30°), starting at the same position. Azimuths of both talkers vary continually:

| Azimuth of 1st talker | Azimuth of 2nd talker |
| --- | --- |
| 180° : +1° : 120° | 180° : -1° : -120° |
| 240° (-120°) : +1° : 180° | 240° (-120°) : -1° : -60° |
| 300° (-60°) : +1° : 240° | 300° (-60°) : -1° :0° |
| 0° +1° : 300° | 0° : -1° : 60° |
| 60° : +1° : 0° | 60° : -1° : 120° |
| 120° : +1° : 60° | 120° : -1° : 180° |

E.7.3	Allocation of scenes to talkers to each listening panel

Each of the sentences uttered by a certain talker is encoded using different scene. To balance the test, listeners of each panel listen to all talkers, and all scenes are covered in each panel.

Table E.7.1: Allocation of scenes and talkers to each listening panel

| Panel / Categories | G1 | G2 | G3 | G4 | G5 | G6 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |

E.7.4	Experiment setup

Tables E.7.2 to E.7.4 show conditions to be used for this experiment, list of preliminaries and full list of conditions, respectively.

The test Categories correspond to different talkers.

Table E.7.2: Factors and conditions for Experiment P800-7

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Candidate | 1 | CuT |
| Bitrates | 4 | 16.4, 24.4, 32, and 48 kbps |
| DTX | 2 | DTX ON and OFF |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Codec references |  |  |
| Codec references | 1 | EVS |
| Bitrates | 4 | 2 x 8.0, 2 x 13.2, 2 x 16.4, 2 x 24.4 kbps |
| DTX | 2 | DTX ON and OFF |
| Input frequency mask | 1 | HP50 |
| Noise | 0 | No noise |
| Error Conditions | 2 | 0%, 3% FERs |
| Other references |  |  |
| Direct | 1 | -26 LKFS |
| MNRU | 5 | Q= 15, 23, 31, 39, 47 dB |
| ESDRU | 4 | 0.1, 0.3, 0.5, 0.7 |
| Input frequency mask | 1 | HP50 |
| Common Conditions |  |  |
| Number of talkers (categories) | 6 | 3 male and 3 female |
| Number of speech samples | 7 | 6 + 1 (preliminaries) sentence pairs per talker. |
| Listening Level | 1 | 73 dB SPL |
| Listeners | 30 | Naïve Listeners |
| Randomizations (panels) | 6 | 6 panels of 5 listeners |
| Rating Scale | 1 | P.SUPPL800 |
| Listening System | 1 | Headphones |

Table E.7.3: Preliminaries for Experiment P800-7

| Trial | Label | Sample | Condition | Bitrate [kb/s] | DTX | FER [%] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | EVS | 2 x 16.4 | Off | 3 |
| 2 |  |  | MNRU 31 | - | - | - |
| 3 |  |  | EVS | 2 x 8.0 | On | 0 |
| 4 |  |  | ESDRU 0.5 | - | - | - |
| 5 |  |  | EVS | 2 x 24.4 | Off | 0 |
| 6 |  |  | MNRU 15 | - | - | - |
| 7 |  |  | EVS | 2 x 13.2 | Off | 0 |
| 8 |  |  | Direct | - | - | - |
| 9 |  |  | EVS | 2 x 8.0 | Off | 3 |
| 10 |  |  | MNRU 47 | - | - | - |
| 11 |  |  | EVS | 2 x 16.4 | On | 0 |
| 12 |  |  | ESDRU 0.1 | - | - | - |

Table E.7.4 Test conditions for Experiment P800-7:

| Label | Condition | Bitrate [kb/s] | DTX | FER [%] | Ref condition | ToR |
| --- | --- | --- | --- | --- | --- | --- |
| C01 | Direct | - | - | - | - | - |
| C02 | MNRU 15 | - | - | - | - | - |
| C03 | MNRU 23 | - | - | - | - | - |
| C04 | MNRU 31 | - | - | - | - | - |
| C05 | MNRU 39 | - | - | - | - | - |
| C06 | MNRU 47 | - | - | - | - | - |
| C07 | ESDRU 0.1 | - | - | - | - | - |
| C08 | ESDRU 0.3 | - | - | - | - | - |
| C09 | ESDRU 0.5 | - | - | - | - | - |
| C10 | ESDRU 0.7 | - | - | - | - | - |
| C11 | EVS | 2 x 8.0 | Off | 0 | - | - |
| C12 | EVS | 2 x 13.2 | Off | 0 | - | - |
| C13 | EVS | 2 x 16.4 | Off | 0 | - | - |
| C14 | EVS | 2 x 24.4 | Off | 0 | - | - |
| C15 | EVS | 2 x 8.0 | Off | 3 | - | - |
| C16 | EVS | 2 x 13.2 | Off | 3 | - | - |
| C17 | EVS | 2 x 16.4 | Off | 3 | - | - |
| C18 | EVS | 2 x 24.4 | Off | 3 | - | - |
| C19 | EVS | 2 x 8.0 | On | 0 | - | - |
| C20 | EVS | 2 x 13.2 | On | 0 | - | - |
| C21 | EVS | 2 x 16.4 | On | 0 | - | - |
| C22 | EVS | 2 x 24.4 | On | 0 |  |  |
| C23 | CuT | 16.4 | Off | 0 | C11 | NWT |
| C24 | CuT | 24.4 | Off | 0 | C12 | NWT |
| C25 | CuT | 32 | Off | 0 | C13 | NWT |
| C26 | CuT | 48 | Off | 0 | C14 | NWT |
| C27 | CuT | 16.4 | Off | 3 | C15 | NWT |
| C28 | CuT | 24.4 | Off | 3 | C16 | NWT |
| C29 | CuT | 32 | Off | 3 | C17 | NWT |
| C30 | CuT | 48 | Off | 3 | C18 | NWT |
| C31 | CuT | 16.4 | On | 0 | C19 | NWT |
| C32 | CuT | 24.4 | On | 0 | C20 | NWT |
| C33 | CuT | 32 | On | 0 | C21 | NWT |
| C34 | CuT | 48 | On | 0 | C22 | NWT |