Source:	Fraunhofer IIS

Title:	Proposed experiments for IVAS Codec selection testing with Stereo content and BS.1543 MUSHRA tests

Document for:	Discussion & Agreement

Agenda Item:	7.5

Introduction

In Table 5 of Section 5 in the IVAS-8a Test Plan [1] there is a high-level overview of the experiments to be held using BS.1543 MUSHRA tests. Experiments BS1534-1a and BS1534-1b give an overview of the experiments for Stereo input as can be seen in the respective rows from Table 5 below.

| Exp | Input format | Source material | Listening environment | Bitrates kbps | FER/jitter | DTX | Headtracking |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS1534-1a | Stereo | Generic Audio | Headphones |  | ≤ x% | N | No |
| BS1534-1b | Stereo | Generic Audio | Headphones |  | ≤ x% | N | No |

This contribution aims to give a detailed description of the experiments to be held for the selection phase for this input format.

Stereo BS.1543 MUSHRA experiments BS1534-1a, BS1534-1b

Experimental general considerations

Below are proposed outlines for experiments BS1534-1a, BS1534-1b with Stereo input/source material. Main factors to be varied in the tests are:

Source material

The source material should meet the requirements defined in Section 4.5.4 for critical generic audio material in [1]

Bit rates

The bitrate range for the Stereo input format that is not being covered from the respective P.SUPPL800 would be 64-256kbps. However, the source finds that the performance for 48kbps with generic audio is also critical and therefore, it is suggested that experiment 1a covers the mid bitrate range namely 48 and 64kbps and experiment 1b the higher range, i.e., 96 and 128 kbps.

Audio bandwidth

IVAS operation is required for WB, SWB and FB audio input-output. It is not possible to cover all audio bandwidths in the selection tests. In line with a corresponding editor’s note in Pdoc IVAS-3, it is rather suggested to test at the maximum available bandwidth, which is FB.

Reference condition

For the experiments BS1534-1a/BS1534-1b with stereo input format, apart from the reference and hidden reference, one 3.5 kHz low-pass anchor condition is considered to be sufficient.

Specifics for Experiments BS-1534-1a: Stereo Input

Factors and conditions (BS1534-1a Generic Audio)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated with stereo audio input at 48 and 64 kbps DTX off at 0% FER |
| Codec references |  |  |
| Codec references | 3 | Dual-mono EVS 2*24.4 kbps, 2*32 kbps, 2*48 kbps DTX off at 0% FER |
| Other references |  |  |
| Reference | 1 | Direct signal, Nominal input level |
| Hidden Reference | 1 | Direct signal, Nominal input level |
| LP3.5 anchor | 1 | 3.5kHz lowpass filtered signal, nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Audio sampling frequency/bandwidth | 2 | 48 kHz/FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12 - 16] | Experienced Listeners |
| Randomizations | 10 | Individual per listeners |
| Rating Scale | 1 | Continuous Mushra scale from 0-100 [ITU-R BS.1534] |
| Replications | 2 |  |
| Listening System | 1 | High-quality headphone for diotic presentation |
| Listening Environment | 1 | No room noise |

Specifics for Experiments BS-1534-1b: Stereo Input

Factors and conditions (BS1534-1b Generic Audio)

| Main Codec Conditions |  |  |
| --- | --- | --- |
| Codec under Test (CuT) | 2 | IVAS candidate operated with stereo audio input at 96 and 128 kbps DTX off at 5% FER |
| Codec references |  |  |
| Codec references | 3 | Dual-mono EVS 2*48 kbps, 2*64 kbps, 2*96 kbps DTX off at 5% FER |
| Other references |  |  |
| Reference | 1 | Direct signal, Nominal input level |
| Hidden Reference | 1 | Direct signal, Nominal input level |
| LP3.5 anchor | 1 | 3.5kHz lowpass filtered signal, nominal level |
| Common Conditions |  |  |
| Test item generation | 1 | According to material collection procedure for IVAS selection MUSHRA tests. |
| Audio sampling frequency/bandwidth | 2 | 48 kHz/FB |
| Input frequency mask | 1 | Flat |
| Nominal output loudness | 1 | -26 LKFS ([ITU-R BS.1770-4]) |
| Listening Level | 1 | Adjusted by listener |
| Listeners | [12 - 16] | Experienced Listeners |
| Randomizations | 10 | Individual per listeners |
| Rating Scale | 1 | Continuous MUSHRA scale from 0-100 [ITU-R BS.1534] |
| Replications | 2 |  |
| Listening System | 1 | High-quality headphone for diotic presentation |
| Listening Environment | 1 | No room noise |

# References

S4-230025: IVAS Permanent Document IVAS-8a: Test Plan for Selection Phase, v.0.7.4.