Agenda item: 	10.7

Source: 	InterDigital Communications

Title: 	[GA4RTAR] Call Flows and Procedures for Collaboration Scenario (CS) 3

Document for:	Discussion and Agreement

# Introduction

In this contribution, we discuss the call flows and procedures for MNO facilitated 5G-RTC sessions (Collaboration Scenario 3).

# Call Flows and Procedure for CS 3

  - Client-driven Management of 5G RTC Edge Processing

In collaboration scenario 3, MNO hosts the WebRTC sessions by providing a trusted WebRTC signalling server in the 5G-RTC AS. In addition, a trusted media server is also present in 5G-RTC AS to support SFU and MCU functionality.

The call flows for CS 3 are as shown in Figure 1.

Figure 1. Call flows for MNO facilitated 5G-RTC sessions (collaboration scenario 3)

The 5G-RTC Application Provider creates a Provisioning Session with the 5G-RTC AF and starts provisioning the usage of the RTC Streaming session between two endpoints. During the establishment phase, the used features such as content consumption measurement, logging, collection and reporting; QoE metrics measurement, logging, collection and reporting; dynamic policy; network assistance; are negotiated and detailed configurations are exchanged.

The 5G-RTC Application Provider Provisioning Session phase is performed prior to the establishment of any related WebRTC sessions by the 5G-RTC Application Provider. Subsequent updates to the provisioning session are possible.

- Create Provisioning Session: In this step, the 5G-RTC Application Provider creates a new provisioning session.

- Provision 5G-RTC features: In this step, the 5G-RTC Application Provider may create different configurations such as content consumption measurement, logging, collection and reporting; QoE metrics measurement, logging, collection and reporting; dynamic policy; network assistance; charging, offering STUN/TURN servers, WebRTC signalling servers, edge processing, etc.

The ICE candidate discovery phase is performed with the following steps in an MNO-facilitated 5G-RTC system:

- Configuration information: The 5G-RTC AF uses the RTC-5 interface to provide the MSH with a list of trusted STUN/TURN servers, trusted WebRTC signaling and data channel servers and their capabilities. The UE may use this configuration information for establishing RTC sessions.

- ICE Servers request: The application queries the MSH for the list of trusted ICE servers.

- ICE candidate validation: The UE discovers and tests the ICE candidates to validate that they are suitable for the connection.

The WebRTC session establishment phase is performed with the following steps in an MNO-facilitated 5G-RTC system:

- Query configuration information: The WebRTC framework queries the MSH for the WebRTC signaling server information. In some cases where the signaling is not handled by WebRTC framework, the native WebRTC application queries the MSH for the WebRTC Signaling server information.

- Configuration information: MSH sends the WebRTC signaling and data channel servers and their capabilities information to WebRTC framework or in some cases with native WebRTC application.

In SDP exchange phase, the WebRTC Framework or native WebRTC Application exchanges signaling information related to the WebRTC session such as ICE candidates, SDP offer/answer between two or more WebRTC endpoints using the trusted WebRTC signaling server.

- SDP offer: The WebRTC Framework or native WebRTC Application creates a request with SDP offer which includes the ICE candidates and sends it to the WebRTC signaling server.

- Covert SDP offer: The WebRTC signalling server converts the SDP offer to match with the remote WebRTC endpoint referred as UE 2.

- SDP offer: The WebRTC signaling server forwards the request to UE 2.

- SDP answer: Upon accepting the offer, UE 2 responds to signalling server with SDP answer.

- SDP answer: WebRTC signalling server sends the SDP answer to the UE1.

With this, a WebRTC session is established between WebRTC endpoints using the most suitable ICE candidate and the WebRTC signaling server.

If the Network support function feature is supported in the 5G-RTC AF, then the Network Support Function AF (NS-AF) shall offer the bitrate recommendation request API based on existing policy templates, through the usage of either the Npcf_PolicyAuthorization API over N5 interface, or the Nnef_AFSessionWithQoS over N33 interface to the PCF. If no corresponding AF application session context already exists, the NS-AF shall use the Npcf_PolicyAuthorization_Create method over N5 interface with the appropriate service information to create and provision an application session context. The Network assistance phase performed with the following steps in an MNO-facilitated 5G-RTC system.

- QoS information: The WebRTC signaling server, upon reception of the QoS allocation request by the application (or WebRTC framework) may extract the 5-Tuple information for each of the media sessions and convey the information to the NS-AF in 5G-RTC AF.

- QoS allocation: The NS-AF uses the N5 interface to request QoS allocation from PCF or SMF.

- Confirmation: PCF or MSF confirms the successful allocation of network support or QoS allocation.

- Subscribe to QoS events: The NS-AF will also subscribe to events related to the QoS flows of the WebRTC session with the PCF and SMF.

- QoS events: The NS-AF receives notifications about any changes to the QoS flows of the WebRTC session from the PCF or the SMF.

- QoS notifications/ Bitrate recommendations: The NS-AF may send notifications to the MSH about the changes in QoS flow. When the allocated session is active, the MSH forwards the bitrate recommendation to the application.

- Adjust media bitrate: The WebRTC application may act on adjusting the bitrate recommendation, e.g., by reducing the uplink media bitrate.

After successful creation of a WebRTC session and the bitrate negotiations, the actual WebRTC media streaming session over 5G may start:

- Media transfer:

  - The WebRTC Application may connect to the selected TURN server and/or Media Function in the “5G-RTC AS” through the RTC-4m interface and real-time streaming starts, and the media is delivered to the remote endpoint.

  - In some cases, a peer-to-peer connection is also possible and the media is delivered directly to the remote endpoint.

- Method calls and notifications: Supporting information about the WebRTC session is passed from the WebRTC framework to the Media Session Handler.

- Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the WebRTC session with the 5G-RTC AF.

- End session: The WebRTC Application informs the WebRTC framework that the RTC session has ended.

- Session ending event: The WebRTC framework informs the Media Session Handler about the end of the RTC session.

- Final reporting: The Media Session Handler performs any final reporting to the 5G-RTC AF.

# Proposal

We propose to agree the proposed call flow and procedure text in CS 3 and document it in the clause 5 of GA4RTAR TS (26.506).