3GPP TSG-SA4 Meeting #125	S4-231197

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.506 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RTC Functions are general Media Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | GA4RTAR |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was identfiied that RTC has many commonalities with 5GMS and hence a generalized Media Architecture representation is preferred. This alignment is documented |  |  |  |  |  |  |  |  |
| Summary of change: |  | Integration of RTC in Media Architecture Bug Fixes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-3 specifications are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 1, 2, 3.1, 3.2, 3.3, 4.1, 4.1.2 (new), 4.2.1, 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# =====  CHANGE =====

# Introduction

The present document specifies an architecture for real-time media communication integrated into the 5G System. To support MNO and third-party services for real-time media, essential functionalities and interfaces are specified.

# =====  CHANGE =====

# 1	Scope

The present document specifies an architecture for real-time media communication integrated into the 5G System. To support Mobile Network Operator (MNO) and third-party services for real-time media, essential functionalities and interfaces are specified. The primary scope of this Technical Specification is the documentation of the following aspects:

-	The definition of a real-time media communication architecture mapped to the 5GS architecture, with relevant core building blocks, reference point, and interfaces to support modern operator and third-party media services, based on the 5GMS architecture

-	Definition all relevant reference points and interfaces to support different collaboration scenarios between 5G System operator and third-party media communication service provider, including but not limited to an Augmented Reality (AR) media communication service provider.

-	Call flows and procedures for different real-time communication service types,

-	Specification to support AR relevant functionalities such as split-rendering or spatial computing on top of a 5G System based on this architecture

# =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[3]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[4]	3GPP TS 26.113: "Enabler for Immersive Real-time Communication".

[5]	3GPP TR 26.930: "Study on the enhancement for Immersive Real-Time communication for WebRTC".

[6]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[7]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[8]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[9]	3GPP TS 36.321: "LTE; Medium Access Control (MAC) protocol specification".

[10]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[11]	3GPP TS 23.501: " System architecture for the 5G System (5GS)"

# =====  CHANGE =====

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

# =====  CHANGE =====

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

# =====  CHANGE =====

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

EAS	Edge Application Server

ECS	Edge Configuration Server

EEC	Edge Enabler Client

EES	Edge Enabler Server

IETF	Internet Engineering Task Force

ICE	Interactive Connectivity Establishment

IMS	IP Multimedia Subsystem

MCU	Multi-point Control Unit

MNO	Mobile Network Operator

MR	Mixed Reality

MSH	Media Session Handler

MTSI	Multimedia Telephony Service for IMS

NAT	Network Address Translation

RTC	Real-Time Media Communication

SDP		Session Description Protocol

SFU	Selective Forwarding Unit

STUN	Session Traversal Utilities for NAT

TURN	Traversal Using Relays around NAT

W3C	World Wide Web Consortium

WebRTC	Web Real-Time Communication

# =====  CHANGE =====

# 4	Real-time media Communication Architecture

## 4.1	Overall architecture for Real-Time media Communication (RTC)

## 4.1.1	Definition of RTC Architecture

Real-Time media Communication (RTC) over 5G system in the context of this specification is defined as the delivery of delay-sensitive media from one peer to another with support of 5G network. AR conversational service described in TR 26.998 [2] is a typical use cases for RTC, which enables end-users to directly communicate real-time media including AR/MR media content as specified in TS 26.119 [3]. As identified in clause 8.4 of TR 26.998, there may be different options to enable such AR conversational service, for example re-use of parts of MTSI as defined in TS 26.114 such as the IMS data channel or 5G Media Streaming for managed services.

The overall RTC architecture is shown in Figure 4.1.1-1 as below.

Figure 4.1.1-1: Real-time media communication (RTC) in 5G System

NOTE:	The functions indicated by the yellow filled boxes are in scope of the present document for RTC. The functions indicated by the grey boxes are defined in 5G System specifications. The functions indicated by the blue boxes are neither in scope of 5G RTC nor 5G System specifications.

The media data is exchanged between two or more RTC endpoints over a 5G System as defined in TS 23.501 [11]. The RTC endpoint is an endpoint configured by RTC architecture in the present document. It is typically a UE, but an edge computing server can also be the RTC endpoint. The Application Provider provides a RTC Aware-Application on the UE to make use of RTC endpoint and network functions using interfaces and APIs. RTC architecture provides the core functions and entities to support WebRTC-based service over 5G System, two main functions are defined in the trusted DN.

-	RTC AF: An Application Function similar to 5GMS AF as defined in TS 26.501 [6], but dedicated to real-time media communication

-	RTC AS: An Application Server dedicated to real-time media communication

NOTE:	If both, RTC AF and RTC AS are in an external DN, this is  out of scope of the present specification.

The detailed RTC architecture mapping to the overall high-level architecture in Figure 4.1.1-1 is shown in Figure 4.1.1-2 below. Note that Figure 4.1.1-2 illustrates only the half portion of Figure 4.1.1-1 (the link from one RTC endpoint to RTC AF and RTC AS), as the rest of portion is symmetric.

Figure 4.1.1-2: RTC General Architecture

NOTE1:	Some of functions may not be required depending on the collaboration scenario. Description of collaboration scenario and its architecture variant are specified in Annex A.

NOTE2:	The WebRTC framework is a WebRTC protocol stack and its implementation, defined in W3C and IETF.

NOTE3:	Red ovals indicate API provider functions.

The subfunctions inside RTC AF, RTC AS, and RTC endpoint are defined in clause 4.2 and the interfaces shown in Figure 4.1.1-2 are defined in clause 4.3.

# =====  CHANGE =====

### 4.1.2	Generalized Integrated Media Architecture

Due to the similarity of 5GMS as defined in TS 26.501 [6] and RTC as defined in the present document, the RTC functions and 5GMS functions may share or may make use of many common functionalities, both media session handling as well as for media delivery. While a full integration of 5GMS and RTC is not addressed in this specification, a generalized 5G Media Architecture that integrates 5G Media Streaming and RTC can be envisioned as shown in Figure 4.1.2-1.

Figure 4.1.2-1 – Generalized Media Support within the 5G System

In this case, the RTC AF is integrated into the Media AF, the RTC AS is integrated in the Media AS, and the RTC client is part of the Media Client.

A functional description with additional details as well as reference points is provided in Figure 4.1.2-2.

Figure 4.1.2-2 Generalized 5G Media Delivery Architecture

Functional definitions may be generalized as follows:

-	Media AF: An Application Function similar to that defined in TS 23.501 [11], clause 6.2.10, dedicated to 5G Media Delivery.

-	Media AS: An Application Server dedicated to 5G Media Delivery.

-	Media Client: A UE internal function dedicated to 5G Media Delivery.

-	Media Session Handler: A function on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: A UE internal function A function on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

The following interfaces and APIs may be defined for 5G Media Delivery:

-	M1 (Provisioning API): External API, exposed by the Media AF which enables the Media Application Provider to provision the usage of the 5G Media Delivery and to obtain feedback.

-	M2 (User Plane interface): External interface provided by the Media AS and used when the Media AS in the trusted DN to exchange data media data with the application service provider.

-	M3: (Server Configuration API): API used to exchange information between Media AF and Media AS for configuration purposes.

-	M4 (Media Delivery Interface): Interface and reference point between media access function and Media AS in order to exchange media content.

-	M5 (Session Handling API): APIs exposed by a Media AF to the Media Session Handler for media session handling, control, reporting and assistance that also include appropriate security mechanisms, e.g. authorization and authentication.

-	M6 (Client Configuration APIs): APIs exposed by a Media Session Handler to the Application and media access function for client-internal communication.

-	M7 (Media Access APIs): APIs exposed by a Media Access function to configure and communicate with the Media access function.

-	M8 (Application reference point): application interface used for information exchange between the Media Application and the Media Application Provider.

The mapping of RTC functions and reference points to Generalized Media Architecture is provided in Table 4.1.2-1. It can be assumed that the respective function and reference points fully integrate the all functionalities for RTC.

Table 4.1.2-1 Mapping of RTC functions and reference points to Generalized Media Architecture

| Generalized Media Architecture Function/Reference Points | RTC Function/Reference Points |  |  |
| --- | --- | --- | --- |
| Media AF | RTC AF |  |  |
| Media AS | RTC AS |  |  |
| Media Client | RTC endpoint |  |  |
| M1 | RTC-1 |  |  |
| M2 | Not existing in case of RTC |  |  |
| M3 | RTC-3 |  |  |
| M4 | RTC-4 |  |  |
| M5 | RTC-5 |
| M6 | RTC-6 |
| M7 | RTC-7 |
| M8 | RTC-8 |

# =====  CHANGE =====

### 4.2.1	General

This clause defines minimal and essential functions as well as extra functions and entities that may appear in certain deployment or collaboration scenarios. The definitions of extra functions and entities are specified in TS 26.113 [4] and TR 26.930 [5].

# =====  CHANGE =====

### 4.2.2	Provisioning function

The provisioning function may enable an application provider to perform provisioning of the following functionalities:

-	QoS support provisioning for WebRTC sessions

-	Charging provisioning for WebRTC sessions

-	Collection of consumption and QoE metrics data provisioning related to WebRTC sessions

-	Offering Interactive Connectivity Establishment (ICE) functionality provisioning to support Network Address Traversal /NAT) such as Session Traversal Utilities for NAT (STUN) and Traversal Using Relays around NAT (TURN) servers

-	Offering WebRTC signalling servers provisioning, potentially with interoperability to other signalling servers.

The provisioning function may not be relevant to all collaboration scenarios and some of the 5G support functionality may be offered without application provider provisioning.

NOTE:	The integration/collocation of this RTC AF and WebRTC signalling function is possible. Co-located WebRTC signalling function is able to act as a RTC AF which is accessible to 5GC, and replace some of this RTC AF’s interfaces and APIs with WebRTC signalling function. For example, interfaces and APIs between this RTC AF and UE will be replaced to avoid concurrent/redundant requests from UE.