Source:	HEAD acoustics GmbH

Title:	Custom format for delay/error profile used for JBM performance testing

Document for:	Agreement

# Introduction

For the work item eUET [1], the delay & error profiles for testing performance of the jitter buffer management (JBM) in 3GPP TS 26.131/132 [2] [3] are planned to be replaced or extended by more realistic ones, which are preferably based on real network captures. In several contributions [4] [5] [6], new profiles were proposed and discussed, which also contain packet duplications.

When delay & error profiles were introduced in 3GPP TS 26.131/132 [2] [3], the format for describing delay and losses was adapted from clause 8.2.3.3 of 3GPP TS 26.114 [7]. Five profiles were actually completely taken over from this specification, see Annex F of TS 26.132 [3]. This rather simple format provides delay values (in ms) as raw text with one entry per line, packet losses are indicated as "-1". Packet duplications are not considered at all in this profile format, most likely due to the JBM specifications made clause in clause 8.2.2 of the same document [7], which states that …

A JBM implementation used in MTSI shall support the following requirements [...]. Speech JBM used in MTSI shall:

[…]

-	be able to receive duplicate speech frames and only present unique speech frames for decoder consumption;

[…]

Even though UEs that show any other JBM behaviour than discarding duplicated packets would already violate the requirements of 3GPP TS 26.114, testing within the scope of 26.131/132 would require a new convention, how duplicated packets are specified in the text-based format currently used.

# Proposal

As an extension for the existing text-based format for describing delay & error profiles, it is proposed to use "+" (plus sign) as a suffix to a delay value to indicate a duplicated packet. Note that it is not sufficient to just have a unique flag (like "-1" for packet loss) for duplication, as the first/regular packet still must have a valid delay value. An example of the proposed extension is shown in Figure 1.

Figure 1: Example for indication of packet duplication

# Conclusion

For the description of new delay & error profiles that will also include packet duplication, a new convention is needed to extend the current format. To proceed with the actual work, the source proposes to agree on a certain extension, preferably the one suggested in the present document.

Since this (or any other) extension would become incompatible with the profiles defined in 3GPP TS 26.114 [7], the revised Annex F of TS 26.132, which will specify the new delay & error profiles, should clearly indicate this deviation.

# References

| [1] | 3GPP SP-220610, „New WID on Enhancements to UE Testing,“ Orange, HEAD acoustics GmbH, Fraunhofer IIS, ROHDE & SCHWARZ, Amazon, 2022. |
| --- | --- |
| [2] | 3GPP TS 26.131, „Terminal acoustic characteristics for telephony; Requirements,“ Release-17.1. |
| [3] | 3GPP TS 26.132, „Speech and video telephony terminal acoustic test specification,“ Release-17.1. |
| [4] | 3GPP TR 26.114, „IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction,“ Release-18.3. |
| [5] | 3GPP S4-230236, „Proposed updates on JBM performance,“ Orange. |
| [6] | 3GPP S4-230617, „Follow-up on profiles for JBM behaviour evaluation,“ Orange. |
| [7] | 3GPP S4-230941, „Proposed changes for JBM performance testing,“ Orange. |