3GPP TSG SA WG4 #125 meeting	S4-231286

Gothenburg, Sweden, 21st – 25th of August 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119 | CR | pseudo | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] MIV Operation Points and Decoding Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation, Philips, Interdigital Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 15/08/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Define MIV operating points and enable the support of V3C (MIV) bitstream and receiver decoding capabilities to address volumetric media for MeCAR and RTC use cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | References, New clause 7.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S4-230878-> S4aV230032 |  |  |  |  |  |  |  |  |

Start of Change 1

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[3]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[4]	3GPP TR 26.857: "5G Media Service Enablers".

[5]	Khronos, "The OpenXR Specification", https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html…

[6]	ISO/IEC 23090-12:2023, Information technology — Coded Representation of Immersive Media — Part 12: MPEG immersive video

End of Change 1

Start of Change 2 NEW CLAUSE

## 7.1.3 	Volumetric decoding Capabilities

This clause applies to device type 3 “XR phone” and device type 4 “XR HMD”.

The underlying 2D video codecs are specified in 7.1.1.

#### 7.1.3.1 MIV Operation Point

This clause adds V3C restrictions to the functionalities offered in the V3C and MIV standard [6].

3GPP MIV defines the simplest operation point giving operators the opportunity to target different applications and devices including performance compromises and adding tools that will preserve the creative intent of the content creator.

A Bitstream conforming to the 3GPP MIV operation point shall conform to the MPEG AVC Progressive High MIV extended Unconstrained or MPEG HEVC Main10 MIV extended Unconstrained profile with the following additional restrictions and requirements:

- The ptl_tier_flag shall be set to 0.

- The value of ptl_profile_codec_idc shall be either 0 or 1 with the codec level profiles defined in section 7.1.1.

- The value of ptl_profile_toolset_idc shall be 65.

- The value of ptl_level_idc shall be 75 or lower.

- The value of ptl_max_decodes_idc shall be 0, 1, or 2.

NOTE: The video sub-bitstreams are constrained by the video decoder capabilities (7.1.1).

A Bitstream conforming to the 3GPP MIV operation point shall contain attribute of type 0 (ATTR_TEXTURE) and may contain attribute of type 2 (ATTR_TRANSPARENCY).

A bitstream conforming to this operation point shall include full range for the geometry video component, i.e., the video full range code point as specified in ISO/IEC 23091-2 shall be set to 1.

In addition, the following also applies to a bitstream conforming to the 3GPP MIV operation point:

- The value of ptc_max_atlas_count_minus1 equal to 0

- The value of ptc_multiple_map_streams_constraint_flag equal to 1

- The value of ptc_max_map_count_minus1 equal to 0

- The value of ptc_attribute_max_dimension_minus1 equal to 2

- The value of ptc_attribute_max_dimension_partitions_minus1 equal to 0

- The value of ptc_restricted_geometry_flag equal to 0 or 1

- When value of vps_extension_type is present and equal to 1 (VPS_EXT_PACKED) the value of vps_occupancy_video_present_flag, vps_geometry_video_present_flag, and vps_attribute_video_present_flag shall be 0 and ptl_max_decodes_idc shall be 0.

Type 3 and Type 4 receivers conforming to the 3GPP MIV operation point shall support decoding 3GPP MIV operation point bitstreams. A device that supports 3GPP MIV operation point should be able to render it. The rendering process is not in the scope of this document.

Type 3 and Type 4 receivers conforming to the 3GPP MIV operation point shall support all Receiver requirements in this clause.

End of Change 2