3GPP TSG SA WG4 Meeting #125	S4-231351

# Gothenburg, SE, 21 – 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR |  | rev |  | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP] Updates to Guidelines for PDU Set Importance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP |  |  |  |  |  | Date: |  |  | 15 August 2023 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA4#124, the document S4-231028 was agreed into TS 26.522, providing an initial version of the guidelines for PDU set marking. However, for marking of the PDU set importance (PSI) field, alignment is needed between the codec-specific guidelines and the guidelines that consider the dependencies across PDU sets within a stream and across multiple streams. This pCR provides an alignment between the clauses and improves the guidelines. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Assign PSI ranges derived from codec-specific guidelines - Further edits for improving the text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

#### 4.4.2.6	Guidelines for PDU Set Marking

##### 4.4.2.6.1	End of Data Burst Field

NOTE: These detailed guidelines are FFS.

##### 4.4.2.6.2	PDU Set Importance Field

NOTE:	The following aspects need to be further defined:

-	Default value for importance when the sender cannot define importance

-	Codec level aspect:

-	video: importance when PDU set is i) slice, ii) frame iii) parameter sets iv) tile set v) other?

-	audio: when and if to use PDU set marking HE in an audio frame.

-	text/metadata: when and if to use PDU set marking HE in text/metadata

-	image: a frame is a PDU set and the importance for all frames are i) same ii) set based on application aspects.

-	Importance across bitstreams

-	Multiplexed streams: importance marking when a 5-tuple corresponds to more than one bitstream

-	Importance marking considerations for non-multiplexed bitstreams

4.4.2.6.2.1        General

The PDU sets that contain audio data shall be set with highest importance compared with other media PDU sets.

NOTE: PDU sets that carry immersive audio data are not set with highest importance compared with other media PDU sets. The importance value of immersive audio PDU sets is FFS.

The PDU sets that contains the reference frames present in the video bitstream shall be set with higher importance compared with PDU sets that contain non-reference frames.

The following clauses provides the guidelines on setting the PSI field in a PDU set RTP header extension for the 3GPP video codecs. For specific PSI value ranges, refer to the Clause 4.4.2.6.2.5.

4.4.2.6.2.2        H.264 Codec

In an H.264 bitstream, NAL units with the nal_unit_type field assigned the value 5 (refer to Table 7.1 in AVC specification [2]) are Instantaneous Decoding Refresh (IDR) pictures. When the Type field value in the NAL Unit header of an RTP packet is 5, then the corresponding PDUs in that PDU set shall be set with higher importance.

The parameter set NAL units such as Sequence Parameter Set (SPS) and Picture Parameter Set (PPS) are important for decoding the bitstream. Therefore, PDU sets with a Type field value equal to 7, 8, 13 or 15 (refer to Table 7.1 in AVC specification [2]) in the NAL Unit header of the RTP packet shall be set with higher importance.

+---------------+

|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+

|F|NRI|  Type   |

+---------------+

Figure X. NAL unit type octet in an RTP packet payload

The NAL unit type octet contains the NRI (nal_ref_idc) field highlighted in Figure X. The NRI field indicate the relative transport priority.  A value of b00 indicates that the content of the NAL unit is not used to reconstruct reference pictures for inter picture prediction. Such NAL units can be discarded without risking the integrity of the reference pictures. Values greater than b00 indicate that the decoding of the NAL unit is required to maintain the integrity of the reference pictures. The highest transport priority is b11, followed by b10, and then by b01; finally, b00 is the lowest.

PDU sets with an NRI value b00 shall be set with lower importance relative to the PDU sets with other NRI values. PDU sets with an NRI value 0x11 shall be set with higher importance relative to the  PDU sets with other NRI field values.

Type and NRI field can be used to set the PDU Set importance. The PDU set importance value assignment based on the Type and NRI field values is for further study.

4.4.2.6.2.3        HEVC Codec

Different from H.264 (AVC), H.265 (HEVC) NAL unit header (shown in Figure Y) is two bytes, contains a 6-bit Type field and no NRI field. NAL unit types 0–31 indicate Video Coding Layer (VCL) NAL unit types; 32–40 indicate non-VCL NAL unit types. NAL unit types 41–47 are reserved, and types 48–63 are unspecified.

+---------------+---------------+

|0|1|2|3|4|5|6|7|0|1|2|3|4|5|6|7|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|   Type    |  LayerId  | TID |

+-------------+-----------------+

Figure Y.  The Structure of the HEVC NAL Unit Header

All VCL NAL units of the same access unit must have the same NAL unit type which defines the type of the access unit and its coded picture. There are three basic classes of pictures in H.265 (HEVC): intra random access point (IRAP) pictures, leading pictures, and trailing pictures.

In an HEVC bitstream, NAL units with the nal_unit_type field assigned a value in the range 16 to 23 (inclusive) (refer to Table 7.1 in HEVC specification [3]) are Intra Random Access Pictures (IRAP) pictures. This includes IDR, CRA, and BLA picture types as well as types 22 and 23, which currently are reserved for future use.

When the Type field value in the NAL Unit header of RTP packet is in the range 16 to 23 (inclusive), then the corresponding PDUs in that PDU set shall be set with higher importance.

The parameter set NAL units such as Sequence Parameter Set (SPS), Picture Parameter Set (PPS), Video Parameter Set (VPS) are important for decoding the bitstream. Therefore, PDU sets with payload Type field value in the NAL Unit header of RTP packet in the range 32 to 34 (inclusive) shall be set with higher importance.

RFC 7798 specifies Aggregation Packets (APs) to enable the reduction of packetization overhead for small NAL units, such as most of the non-VCL NAL units, which are often only a few octets in size. An AP aggregates NAL units within one access unit. Each NAL unit to be carried in an AP is encapsulated in an aggregation unit. An AP consists of a payload header (denoted as PayloadHdr) followed by two or more aggregation units. In an AP, the Type field in the PayloadHdr MUST be equal to 48. APs are typically used to aggregate parameters sets (VPS, SPS, PPS) into a single packet.

When APs are used, the sender should consider the NAL unit types of the aggregation units while assigning the PSI value. For example, if the aggregation unit contains parameter sets, PDU sets containing those shall be set with higher importance.

It could be that there are PDUs with different NAL unit types in a PDU set. For example, if the first PDU in PDU set is a prefix SEI message or Access Unit Delimiter (AUD), it would be misleading if the sender looked only at the first PDU of the PDU set to determine the PSI value.

The sender should ignore the NAL units with non-VCL NAL unit types 35 and 39 and instead consider NAL unit types of the subsequent VCL NAL units while determining the PSI value for such PDU sets.

A leading picture is a picture that follows a particular IRAP picture in decoding order and precedes it in output order. There are two types of leading pictures in H.265 (HEVC): Random access decodable leading (RADL) pictures and Random access skipped leading (RASL) pictures. A RADL picture is a leading picture that is guaranteed to be decodable when random access is performed at the associated IRAP picture. Therefore, RADL pictures are only allowed to reference the associated IRAP picture and other RADL pictures of the same IRAP picture. A RASL picture is a leading picture that may not be decodable when random access is performed from the associated IRAP picture. Only other RASL pictures are allowed to be dependent on a RASL picture.

Hence, in HEVC bitstreams, RASL pictures can be discarded during random access. HEVC provides mechanisms to enable specifying the conformance of a bitstream wherein the originally present RASL pictures have been discarded. Consequently, system components can discard RASL pictures, when needed, without worrying about causing the bitstream to become non-compliant.

PDU sets with Type field value equal to 6 or 7 (refer to Table 7.1 in HEVC specification [3]) in the NAL Unit header of RTP packet are RADL pictures. PDU sets with Type field value equal to 8 or 9 (refer to Table 7.1 in HEVC specification [3]) in the NAL Unit header of RTP packet are RASL pictures. PDU sets that contain RADL pictures shall be set with higher importance relative to the IRAP pictures but lower importance relative to the RASL pictures in the bitstream.

In video coding, temporal scalability is the option to decode only some of the frames in a video stream instead of the whole stream. This enables a media server to reduce the bitrate sent towards viewers that don’t have enough bitrate or CPU to handle the whole stream. In HEVC, pictures with lowest temporal identifier value (TID) are used as reference pictures in the bitstream and are important for decoding the dependent frames.

PDU sets with TID value 1 (lowest possible value) shall be set with higher importance relative to PDU sets that have a higher TID value. The PSI value for such pictures should be lower for IRAP pictures and slightly higher for non-IRAP pictures compared to the pictures with higher TID values.

Pictures with highest TID value cannot be used as reference pictures and can be discarded at the network level when the throughput is not good, or network conditions are unstable. PDU sets with higher TID values shall be set with lower importance compared with the PDU sets with lower TID values.

In H.265 (HEVC), each leading picture and trailing picture type has two type values. The even picture type numbers indicate sub-layer non-reference pictures and odd picture type numbers indicate sub-layer reference pictures. An encoder can use the sub-layer non-reference picture types for pictures that are not used for reference for prediction of any picture in the same temporal sub-layer. Note that a sub-layer non-reference picture may still be used as a reference picture for prediction of a picture in a higher temporal sub-layer.

PDU sets that contain sub-layer reference picture types shall be assigned a lower PSI value compared to the PDU sets with the corresponding sub-layer non-reference picture types.

End of First Change

Second Change

4.4.2.6.2.5        PSI mapping based on PDU set dependencies

Senders should consider that multiplexed RTP streams are treated as a single QoS flow and set the PSI field accordingly, i.e., the PSI field for one bitstream will affect the PDU sets in other multiplexed streams as well.

In some cases, dependencies may exist across bitstreams even when they are not multiplexed, particularly for XR services.

In case of such dependencies, it may not be enough to have PSI values based on codecs and media types alone. PSI values shall be set in this case based on the following criteria, which are listed in an increasing order of importance i.e., decreasing order of PSI values.

- The PDU set is not necessary for the processing of any other PDU set. Such PDU sets can be discarded, if needed and shall be assigned the highest PSI values 14-15. When multiplexing, discardable packets of all streams should be given value 15 to prevent unfair treatment. If interdependency is known, e.g., in stereo streams (left eye is more important than right eye), then the more important stream can be given the PSI value 14.

  - In H.264, these include the PDU sets with an NRI value equal to b00 in the NAL unit header.

  - In HEVC, the NAL unit header does not contain a field like NRI that indicates the relative transport priority. Hence, it is up to the application to identify such PDU sets.

-	The PDU set is necessary for the processing of some PDU sets of the stream to which it belongs. Such PDU sets shall be assigned a PSI value in the range 9-13 (inclusive). Lower end of the range should be used for IDR/IRAP pictures since they are more important for the bitstream.

  - In H.264, these include:

    - IDR pictures with nal_unit_type equal to 5

    - Non-IDR pictures with nal_unit_type in the range 1 to 4 (inclusive)

  - In HEVC these include:

    - IRAP pictures with nal_unit_type field assigned a value in the range 16 to 23 (inclusive)

    - RADL or RASL pictures with nal_unit_type in the range 6 to 9 (inclusive)

- The PDU set is necessary for the processing of all the other PDU sets of the stream to which it belongs.

  - In H.264, these include:

    - SPS, PPS, i.e., NAL units with the nal_unit_type field equal to 7, 8, 13 or 15

  - In HEVC these include:

    - SPS, PPS, VPS, i.e., NAL units with the nal_unit_type field in the range 32 to 34 (inclusive)

-	The PDU set is necessary for the processing of some PDU sets of the stream to which it belongs and also necessary for the processing of some PDU sets of some other streams to which it does not belong.

NOTE: Values in this and lower range shall be used for assigning PSI values to PDU sets in multiplexed streams or if dependencies exist across non-multiplexed bitreams. Use cases for those cases are FFS. For single streams, the ranges provided by the previous bullet points shall be used.

NOTE: Considerations for multiplexed audio streams are FFS.

-	The PDU set is necessary for the processing of all PDU sets of the stream to which it belongs and also of some other streams to which it does not belong.

-	The PDU set is necessary for the processing of all PDU sets of all streams.

[Editor’s Note1] Alignment between all the clauses in Guidelines section is required.

End of Second Change