Agenda item: 	10.7

Source: 	InterDigital Communications

Title: 	[5G_RTP] Transmission of timing information data for QoE measurements

Document for:	Discussion and Agreement

# Introduction

In the SA4 122nd meeting, S4-230359 [1] and S4-230390 [2] are discussed and agreed in 5G_RTP and MeCAR WI discussions respectively. The RTP header extension for rendered pose and timestamp are described in [1] while various timestamps capturing events occurred in device and split rendering function are described in [2]. In the SA4 123rd meeting, S4-230711 [3] was discussed and it was agreed that transmitting the timestamps estimated or measured by the device, e.g., estimated-at-time (T1), and the time when the rendering started (T3) at the split rendering function should be done using an RTP HE.

In the SA4 123rd meeting, S4-230708 [4] discussed the use of time information recorded at the output of the Split Rendering Server (T5), to measure the server processing delay and the overall application delay excluding the server processing delay. This was agreed in S4-230738 (MeCAR PD).

In the SA4 124th meeting, S4-230903 proposed the transport of timing information recorded in the Split Rendering Server (SRS) such as the output time of the SRS, SceneUpdateTime to the UE, to measure the server processing delay, user interaction delay, age of content, and the round-trip interaction delays using the “Rendered Pose” RTP header extension. It was commented that RTCP messages may be more appropriate to transfer such data compared to RTP HE methods as the timing information is more statistical rather than operational. The operational information such as pose information may be carried over RTP HE and statistical information such as QoE timing information which may be sent at a lower frequency may be carried over RTCP messages.

Based on the discussions in the SA4 124th meeting, this contribution proposes an RTCP XR message-based method to transmit the timing information recorded in the Split Rendering Server (SRS) to the UE.

# Background

# User interaction QoE metrics

The user interaction QoE metrics are defined in MeCAR PD v8.0.0 as below.

In use cases for shared interactive immersive services, the user interaction information is sent from a UE to a server. The server handles the user’s request to the immersive media scene (e.g., changing the context such as translation, rotation, and scaling or adding a new object in the scene). In the case of the edge-assisted UE type, the UE offloads the scene rendering to the Split Rendering Server and the server rasterizes the XR viewport and does pre-rendering to generate the XR media which is encoded and delivered to the UE.

In the context of interactive immersive services, one important parameter to estimate the user quality of experience is the roundtrip interaction delay which is defined in the TR 26.928 section 4.2.2.

The roundtrip interaction delay is defined as the sum of the age of content and the user interaction delay.

The user interaction delay is defined as the time duration between the moment at which a user action is initiated and the time such an action is taken into account by the content creation engine. It is impacted by the uplink latency of the wireless network.

The age of content is defined as the time duration between the moment the content is created and the time it is presented to the user. It is impacted by the downlink latency of the wireless network.

- Action and timestamp information from the device:

  - Action information: the user action information which are grouped into action sets. Each action has a unique identifier of the action

  - lastChangeTime: the time when the user action is made. It corresponds to the lastChangeTime field in the action information defined as the timestamp of the last change to the state of the action.

- Action and timestamp information associated with rendered media frame from the Split Rendering Server:

  - Action identifiers: The identifiers of the actions which are handled by the scene manager and rendered in the associated media frame

  - sceneUpdateTime: the time when the Scene manager processes the interaction task according to the actions in the action message from the UE and updates the scene.

The procedures of the interactivity pipeline are shown in Figure 1.

Figure 1 - User action call flow

Using all the timestamps from the SRS and the UE, the application can calculate the interaction delays as below:

- User-interaction-delay = sceneUpdateTime - lastChangeTime

- Age-of-content = T2.actual – sceneUpdateTime

- Roundtrip-interaction-delay = T2.actual – lastChangeTime

# RTCP messages for transmission of control information

There are number of possible ways to carry a variety of control information using RTCP packets. This includes:

- profile-specific extensions to the sender (PT=200) and receiver report (PT=201) [7],

- application-defined RTCP packet with payload type equal to 204 (PT=204) [7],

- Generic RTP Feedback reports with payload type equal to 205 (RTPFB; PT=205) [9], and

- payload-specific RTCP feedback messages with payload type equal to 206 (PSFB; PT= 206) [9].

- extended reports (XR) with payload type equal to 207 (PT=207) [8],

The guidelines for extending the RTCP messages (as mentioned in RFC 5968) are as follows:

- New RTP profiles may define a profile-specific extension to RTCP SR and/or RR packets, to give additional feedback (see [RFC3550], Section 6.4.3). It is important to note that while extensions using this mechanism have low overhead, they are not backward compatible with other profiles. Where compatibility is needed, it's generally more appropriate to define a new RTCP XR block or a new RTCP packet type instead.

- A new RTCP application-defined, or APP, packet is appropriate for private use by applications that don't need to interoperate with others, or for experimentation before registering a new RTCP packet type ([RFC3550], Section 6.7). It is not appropriate to define a new RTCP APP packet in a standards document: use one of the other extensions points, or define a new RTCP packet type instead.

- New RTCP AVPF (Audio-Visual Profile with Feedback) transport-layer feedback messages should be used to transmit general-purpose feedback information that will be generated and processed by the RTP transport. Examples include (negative) acknowledgements for particular packets, or requests to limit the transmission rate. This information is intended to be independent of the codec or application in use.

- New RTCP AVPF payload-specific feedback messages should be used to convey feedback information that is specific to a particular media codec, RTP payload format, or category of RTP payload formats. Examples include video picture loss indication or reference picture selection, which are useful for many video codecs.

- A new RTCP XR block type is appropriate for transporting new metrics regarding media transmission or reception quality.

Based on the above information, the RTCP XR block type mechanism is best suited for the transmission of the timing information required for measurement of QoE metrics.

# Proposed changes

# Timing information required for measurement of QoE metrics

The estimated-at-time (T1) and start-to-render-at-time (T3) provide the times when the pose was estimated and when the split rendering server (SRS) started to render the rendered frame, respectively. The split-renderer-output-time (T5) provides the time when the output of the SRS for a rendered frame is available. This T5 information can be used to measure the server processing delay and the overall application delay excluding the server processing delay. The SRS processes the interaction according to the actions in the action message from the UE and updates the scene. The Scene Manager records the sceneUpdateTime (T6) timestamp when it starts to process the actions. The sceneUpdateTime is used to measure the user interaction delay, age of content and the roundtrip interaction delay. The details of sceneUpdateTime, measurement of User-interaction-delay, Age-of-content and Roundtrip-interaction-delay QoE interaction metrics are defined in clause 8.3 and 8.6 of MeCAR PD.

TR 26.928 defines the user interaction delay, age of content, and round-trip interaction delay measurements as quality of experience metrics for XR content. These delay measurement metrics need to be calculated at the UE for providing better QoE to the user. Also, the server processing delay measurement helps the UE in the adaptation process with the split rendering server for achieving better QoE. It is expected that the time synchronization between the UE and SRS is performed before the start of media delivery.

Based on the discussion in SA4#124 meeting, this contribution proposes an RTCP message-based mechanism to transmit the pose-estimation-time (T1), start-to-render-at-time (T3), split-rendering-server-output-time (T5) and the sceneUpdateTime (T6) timing information to UE.

# RTCP message-based transmission of timing information

# RTCP XR packet-based transmission.

The timing information data recorded at the SRS or the RTP sender can be transmitted to the UE by enhancing the RTCP XR packets, which are specified in IETF RFC 3611. The RTCP XR report is identified by payload type (PT) equal to 207, which refers to an extended report block message. For transmission of timing information data using RTCP XR messages, the block type (BT) defined RFC 3611 can be extended with a value 40.

# RTCP XR packet format

The format of an RTCP XR packet is as shown in Figure 2.

0                       1                      2                       3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P|reserved |   PT=XR=207    |                length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                     SSRC                                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

:                               report blocks                             :

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 2 – RTCP XR packet format

An XR packet consists of a header of two 32-bit words, followed by a number, possibly zero, of extended report blocks. This type of packet is laid out in a manner consistent with other RTCP packets, as concerns the essential version, packet type, and length information. The semantics of the RTCP XR packet is as below.

- version (V) [2 bits]: Identifies the version of RTP. This specification applies to RTP version two.

- padding (P) [1 bit]: If the padding bit is set, this XR packet contains some additional padding octets at the end. The semantics of this field are identical to the semantics of the padding field in the SR packet, as defined by the RTP specification RFC 3550.

- reserved [5 bits]: This field is reserved for future definition. In the absence of such definition, the bits in this field MUST be set to zero and MUST be ignored by the receiver.

- packet type (PT) [8 bits]: Contains the constant 207 to identify this as an RTCP XR packet. This value is registered with the Internet Assigned Numbers Authority (IANA).

- length [16 bits]: The length of this XR packet in 32-bit words minus one, including the header and any padding.

- SSRC [32 bits]: The synchronization source identifier for the originator of this XR packet.

- report blocks [variable length]: Zero or more extended report blocks. In keeping with the      extended report block framework defined below, each block MUST consist of one or more 32-bit words.

# Extended Report Block Framework

Extended report blocks are stacked, one after the other, at the end of an XR packet. The value of BT (block type) field identifies the block format, and its name space is managed by IANA:

https://www.iana.org/assignments/rtcp-xr-block-types/rtcp-xr-block-types.xhtml. An individual block's length is a multiple of 4 octets. The XR header's length field describes the total length of the packet, including these extended report blocks. Each block has block type and length fields that facilitate parsing. A receiving application can demultiplex the blocks based upon their type and can use the length information to locate each successive block, even in the presence of block types it does not recognize.

RFC 3611 defines seven extended report blocks: block types for reporting upon received packet losses and duplicates, packet reception times, receiver reference time information, receiver inter-report delays, detailed reception statistics, and voice over IP (VoIP) metrics.

An extended report block has the following format:

0                       1                       2                      3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      BT          | type-specific |         block length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

:                    type-specific block contents                      :

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 3 – RTCP XR block format

- block type (BT) [8 bits]: Identifies the block format. Seven block types are defined in Section 4 of RFC3611.  Additional block types may be defined in future specifications. This field's name space is managed by the Internet Assigned Numbers Authority (IANA).

- type-specific [8 bits]: The use of these bits is determined by the block type definition.

- Block length [16 bits]: The length of this report block, including the header, in 32-bit words minus one. If the block type definition permits, zero is an acceptable value, signifying a block that consists of only the BT, type-specific, and block length fields, with a null type-specific block contents field.

- type-specific block contents [variable length]: The use of this field is defined by the particular block type, subject to the constraint that it MUST be a multiple of 32 bits long. If the block type definition permits, It MAY be zero bits long.

# Extended Report block for QoE timing information

This extended report block type permits detailed reporting of timing information recorded at the split-rendering-server (SRS). These reports can be used, for example, for calculating the QoE metrics such as round-trip delay, server processing delay, user interaction delay, age of content and the round-trip interaction delay at the UE.

The timing information required for measuring QoE metrics may be expressed in the same units as in the RTP timestamps of RTP data packets. This is so that, for each packet, one can establish the relation between the media data flowing and the corresponding QoE timing information recorded at the SRS for a specific media frame.

For optimum use of the RTCP bandwidth, the RTCP XR block payload may contain the whole or part of the timing information required to calculate the QoE metrics. time_info field present in Figure 4 represents the timing information present in an RTCP XR block report with BT value equal to 40. When a bit is set to ‘ONE’ in time_info field the respective timing information shall be present in the payload. When a bit is set to ZERO in time_info field, the respective time information shall not be present in the payload. For e.g., when the sender like to transmit only T1 and T3 information, the time_info field is set to b0011 and only T1 and T3 information is present in the message payload.

The identifiers of all actions that were processed for the rendering of a frame at a specific time are reported in the “Rendered Pose” RTP header extension. This header extension is identified using the “a=extmap” attribute URN: “urn:3gpp:xr-rendered-pose”. The synchronization between the various timing information present in the below XR report and the action identifiers present in the “Rendered Pose” RTP HE messages is performed using the RTP timestamp information present in the RTP header of the packet containing the “Rendered Pose” RTP HE and the RTP timestamp field present in the below XR report block.

The QoE timing information Report Block with BT value equal to 40 has the following format:

0                       1                       2                      3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     BT=40     | resv  |time_info|         block length                |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             SSRC of source                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                             RTP time stamp                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           estimated-at-time (T1)                      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          start-to-render-at-time (T3)                |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        split-rendering-server-output-time (T5)     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        scene-update-time (T6)                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 4 –RTCP XR block format for QoE timing information data

The semantics of the fields in QoE time information Extended Report (XR) block is as follows

- block type (BT) [8 bits]: A QoE time information Report Block is identified by the constant 40.

- time_info [4 bits]: This field bits represent the time stamps that are present in the XR report block. When T1 is present in the XR report, 1st bit (least significant bit) is set to 1. When the LSB is set to 0, T1 information shall not be present. When T3, T5 and T6 are present in the RTCP XR block data, bits 2, 3 and 4 are set to 1 respectively. When T1, T3, T5 and T6 are present in an RTCP XR block data, the time_info field value shall be b1111. The timing information when present shall follow the order T1, T3, T5 followed by T6. For example, when the time_info field value is b0101, the XR block carries the T1 information followed by T5. T3 and T6 timing information will not be present in that XR block content.

  - The transmission frequency of T1, T3, T5 and T6 time information in XR report block can be negotiated during the configuration phase.

- resv [4 bits]: This field is reserved for future definition. In the absence of such definition, the bits in this field MUST be set to zero and MUST be ignored by the receiver.

- block length [16 bits]: The length of this report block, including the header, in 32-bit words minus one.

- SSRC of source [32 bits]: The SSRC of the RTP data packet source being reported upon by this report block.

- RTP timestamp [32 bits]: This field represents the RTP time stamp of the media frame at which the corresponding QoE timing information date was recorded at the SRS. This correspondence may be used for synchronization between the media data and the QoE timing information measurements recorded at the SRS for a specific media frame.

- estimated-at-time (T1) [32 bits]: This field represents the time when the pose estimation was made. This time information is expressed in the same units and with the same random offset as the RTP timestamps in data packets.

- start-to-render-at-time (T3) [32 bits]: This field represents the time when the renderer in the split rendering server started to render the associated media frame. This time information is expressed in the same units and with the same random offset as the RTP timestamps in data packets.

- split-rendering-server-output-time (T5) [32 bits]: This field represents the recorded time at the output of the split rendering server. This time information is expressed in the same units and with the same random offset as the RTP timestamps in data packets.

- scene-update-time (T6) [32 bits]: This field represents the time when the Scene manager processes the interaction task according to the actions in the action message from the UE and updates the scene. This time information is expressed in the same units and with the same random offset as the RTP timestamps in data packets.

# SDP signaling and attributes

RFC 3611 defines the SDP attribute "rtcp-xr" that can be used to signal participants in a media session that they should use the specified XR blocks. This attribute is extendable with new parameters to cover any new type of XR report blocks.

The extended RTCP XR block with QoE timing information SDP attribute is defined as below in Augmented Backus-Naur Form (ABNF).

rtcp-xr-attrib = "a=" "rtcp-xr" ":" [xr-format *(SP xr-format)] CRLF

xr-format = qoe-timing-info

/ format-ext

qoe-timing-info= "qoe-timing-info" ["=" max-size]

max-size       = 1*DIGIT ; maximum block size in octets

DIGIT          = %x30-39

format-ext     = non-ws-string

non-ws-string  = 1*(%x21-FF)

CRLF           = %d13.10

The "rtcp-xr" attribute contains zero, one, or more XR block related parameters. Each parameter signals functionality for an XR block, or a group of XR blocks. The attribute is extensible so that parameters can be defined for any future XR block.

The parameters are extended to support delivery of QoE timing information data over RTCP XR packets with block type equal to 40.

The "qoe-timing-info", parameter MAY specify an integer value. This value indicates the largest size the whole report block SHOULD have in octets.

# Proposal

We propose to agree the proposed changes in clauses 2 and 3 of this document and to document them into a new clause 4.4 in the 5G_RTP permanent document.

# References

- S4-230359, “Signalling the render pose and other related information”.

- S4-230390, “[MeCAR] On pose information”

- S4-230711, “RTP header extension for pose information.”

- S4-230708, “Pose information QoE”

- S4-230966, “MeCAR Permanent Document v8.0.0”

- 3GPP TR 26.928, “Extended Reality (XR) in 5G”

- RFC 3550, “RTP: A Transport Protocol for Real-Time Applications.”

- RFC 3611, “RTP Control Protocol Extended Reports (RTCP XR).”

- RFC 4585, “Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).”