3GPP TSG SA WG4 125 Meeting						   S4-231405

Goteborg, Sweden, 21st – 25th August 2023

Source:	Dolby Laboratories Inc., Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Title:	Proposed Update on IVAS Source Code

Document for:	Agreement

Agenda Item:	7.5

Introduction

The IVAS floating-point C source code has been made available as electronic attachment to [1] by the proponent companies.  However, as code verification and testing is an ongoing process, certain corrections have only become available after the SA4#125 document upload deadline. The source companies propose to include these corrections into the adopted IVAS C source code.

The electronic attachment to this contribution contains an updated version of the IVAS source code. All changes relative to the version provided in [1] are outlined in the following section. None of the proposed corrections affect bit-exactness of any condition tested in the IVAS selection tests.

Proposed Corrections

Free of unallocated memory in decoder

It was found that the decoder might crash upon a free or unallocated memory operation. Such a situation might happen especially when the decoder operation could not be performed at all, e.g. by an invalid command line given by the user. The problem was removed by making sure the involved pointer is set correctly to NULL at initialization.

Wrong metadata update in the object renderer in case of JBM operation

It was found that in the JBM path the metadata update in the object renderer was not performed in the correct subframe. The proposed correction resolves this problem.

Update of 5ms rendering API

The IVAS codec features a 5ms update interval for head-tracking data, the API however had a design flaw that forced it to 20ms framing. Unfortunately the API and supporting functions couldn’t be fully tested and completed for the package in [1] without introducing issues for regular 20ms operation, and was thus excluded to avoid adverse effects on the 20ms operation. In the meantime, the remaining issues were resolved and the corresponding code added back to allow binaural rendering with 5ms granularity.

Conclusion

This document describes additional corrections to the IVAS source code relative [1]. These corrections fix important issues in the codec and the source companies ask to include these corrections into the IVAS floating point C code version being adopted by 3GPP.

References:

[1] S4-231246: DRAFT TS 26.258 (Codec for Immersive Voice and Audio Services; C code (floating-point))