3GPP TSG-SA4 Meeting #125	S4-231428

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023					revision of S4-231205

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.247 | CR | 0181 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of HTTPList from 3GPP Metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2023-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue identified in 5G-MAG Reference Tool implementations https://github.com/5G-MAG/Standards/issues/79 - TS 26.512 clause 11.4.3 defines the metrics reporting format to be used for 5G Media Streaming referencing the scheme urn:3GPP:ns:PSS:DASH:QM10 and TS 26.247 clause 10.6.1 and 10.6.2. - TS26.247 clause 10.4 lists all the relevant quality metrics for DASH. HTTPList is not included in this list but does appear in the schema definition in clause 10.6.2 despite not being referenced from anywhere TS 26.247. It was digged deeper into this and found that HTTPList has been removed from TS 26.247 V14.0.0 by CR0093r1 (agreed at SA4#92 as S4-170227 and approved at SA#75 as SP-170027). Summary of change: Remove the HttpList metric from the QoE reporting. Reason for change: The HttpList metrics report includes the media URL used, which can be seen as sensitive information for the end-user. As the HttpList metric is not necessary for almost any QoE-related use-case, this privacy breach is not motivated. Consequences if not approved: Privacy breach, or possibly we need to add active end-user consent to (at least parts of) the QoE reporting concept. |  |  |  |  |  |  |  |  |
| Summary of change: |  | HTTPList and all the related definitions are removed from the schema definition in clause 10.6.2. schema name space identifier changed for compatibility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing for implementors |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# =====  CHANGE =====

### 10.6.2	Report Format

The QoE report is formatted as an XML document that complies with the following XML schema:

<?xml version="1.0"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2017:HSD:receptionreport" xmlns:sup="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric" xmlns:sv="urn:3gpp:metadata:2016:PSS:schemaVersion" xmlns="urn:3gpp:metadata:2017:HSD:receptionreport" elementFormDefault="qualified"> <xs:element name="ReceptionReport" type="ReceptionReportType"/> <xs:complexType name="ReceptionReportType"> <xs:choice> <xs:element name="QoeReport" type="QoeReportType" minOccurs="0" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="contentURI" type="xs:anyURI" use="required"/> <xs:attribute name="clientID" type="xs:string" use="optional"/> </xs:complexType> <xs:complexType name="QoeReportType"> <xs:sequence> <xs:element name="QoeMetric" type="QoeMetricType" minOccurs="1" maxOccurs="unbounded"/> <xs:element ref="sup:supplementQoEMetric" minOccurs="0" maxOccurs="1"/> <xs:element ref="sv:delimiter"/> <xs:any namespace="##other" processContents="skip" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:attribute name="periodID" type="xs:string" use="required"/> <xs:attribute name="reportTime" type="xs:dateTime" use="required"/> <xs:attribute name="reportPeriod" type="xs:unsignedInt" use="required"/> <xs:attribute name="qoeReferenceId" type="xs:hexBinary" use="optional"/> <xs:attribute name="recordingSessionId" type="xs:hexBinary" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="QoeMetricType"> <xs:choice> <xs:element name="RepSwitchList" type="RepSwitchListType"/> <xs:element name="AvgThroughput" type="AvgThroughputType" maxOccurs="unbounded"/> <xs:element name="InitialPlayoutDelay" type="xs:unsignedInt"/> <xs:element name="BufferLevel" type="BufferLevelType"/> <xs:element name="PlayList" type="PlayListType"/> <xs:element name="MPDInformation" type="MpdInformationType" maxOccurs="unbounded"/> <xs:element name="PlayoutDelayforMediaStartup" type="xs:unsignedInt"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepSwitchListType"> <xs:choice> <xs:element name="RepSwitchEvent" type="RepSwitchEventType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepSwitchEventType"> <xs:attribute name="to" type="xs:string" use="required"/> <xs:attribute name="mt" type="xs:duration" use="optional"/> <xs:attribute name="t" type="xs:dateTime" use="optional"/> <xs:attribute name="lto" type="xs:unsignedInt" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="AvgThroughputType"> <xs:attribute name="numBytes" type="xs:unsignedInt" use="required"/> <xs:attribute name="activityTime" type="xs:unsignedInt" use="required"/> <xs:attribute name="t" type="xs:dateTime" use="required"/> <xs:attribute name="duration" type="xs:unsignedInt" use="required"/> <xs:attribute name="accessbearer" type="xs:string" use="optional"/> <xs:attribute name="inactivityType" type="InactivityType" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="InactivityType"> <xs:restriction base="xs:string"> <xs:enumeration value="Pause"/> <xs:enumeration value="BufferControl"/> <xs:enumeration value="Error"/> </xs:restriction> </xs:simpleType> <xs:complexType name="BufferLevelType"> <xs:choice> <xs:element name="BufferLevelEntry" type="BufferLevelEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="BufferLevelEntryType"> <xs:attribute name="t" type="xs:dateTime" use="required"/> <xs:attribute name="level" type="xs:unsignedInt" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListType"> <xs:choice> <xs:element name="Trace" type="PlayListEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListEntryType"> <xs:choice> <xs:element name="TraceEntry" type="PlayListTraceEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="start" type="xs:dateTime" use="required"/> <xs:attribute name="mstart" type="xs:duration" use="required"/> <xs:attribute name="startType" type="StartType" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="PlayListTraceEntryType"> <xs:attribute name="representationId" type="xs:string" use="optional"/> <xs:attribute name="subrepLevel" type="xs:unsignedInt" use="optional"/> <xs:attribute name="start" type="xs:dateTime" use="required"/> <xs:attribute name="sstart" type="xs:duration" use="required"/> <xs:attribute name="duration" type="xs:unsignedInt" use="required"/> <xs:attribute name="playbackSpeed" type="xs:double" use="optional"/> <xs:attribute name="stopReason" type="StopReasonType" use="optional"/> <xs:attribute name="stopReasonOther" type="xs:string" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="StartType"> <xs:restriction base="xs:string"> <xs:enumeration value="NewPlayoutRequest"/> <xs:enumeration value="Resume"/> <xs:enumeration value="OtherUserRequest"/> <xs:enumeration value="StartOfMetricsCollectionPeriod"/> </xs:restriction> </xs:simpleType> <xs:simpleType name="StopReasonType"> <xs:restriction base="xs:string"> <xs:enumeration value="RepresentationSwitch"/> <xs:enumeration value="Rebuffering"/> <xs:enumeration value="UserRequest"/> <xs:enumeration value="EndOfPeriod"/> <xs:enumeration value="EndOfContent"/> <xs:enumeration value="EndOfMetricsCollectionPeriod"/> <xs:enumeration value="Failure"/> <xs:enumeration value="Other"/> </xs:restriction> </xs:simpleType> <xs:complexType name="MpdInformationType"> <xs:choice> <xs:element name="Mpdinfo" type="RepresentationType" maxOccurs="unbounded"/> </xs:choice> <xs:attribute name="representationId" type="xs:string" use="required"/> <xs:attribute name="subrepLevel" type="xs:unsignedInt" use="optional"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="RepresentationType"> <xs:attribute name="codecs" type="xs:string" use="required"/> <xs:attribute name="bandwidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="qualityRanking" type="xs:unsignedInt" use="optional"/> <xs:attribute name="frameRate" type="xs:double" use="optional"/> <xs:attribute name="width" type="xs:unsignedInt" use="optional"/> <xs:attribute name="height" type="xs:unsignedInt" use="optional"/> <xs:attribute name="mimeType" type="xs:string" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:simpleType name="DoubleVectorType"> <xs:list itemType="xs:double"/> </xs:simpleType> <xs:simpleType name="StringVectorType"> <xs:list itemType="xs:string"/> </xs:simpleType> <xs:simpleType name="UnsignedIntVectorType"> <xs:list itemType="xs:unsignedInt"/> </xs:simpleType> </xs:schema>

The following schema is an extension to allow additional QoE metrics.

<?xml version="1.0" encoding="UTF-8"?> <xs:schema xmlns="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric" elementFormDefault="qualified"> <xs:element name="supplementQoEMetric" type="SupplementQoEMetricType"/> <xs:complexType name="SupplementQoEMetricType"> <xs:sequence> <xs:element name="deviceinformation" type="DeviceInformationType" minOccurs="0"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> </xs:complexType> <xs:complexType name="DeviceInformationType"> <xs:choice> <xs:element name="Entry" type="DeviceInformationEntryType" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="DeviceInformationEntryType"> <xs:attribute name="start" type="xs:dateTime" use="required"/> <xs:attribute name="mstart" type="xs:duration" use="required"/> <xs:attribute name="videoWidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="videoHeight" type="xs:unsignedInt" use="required"/> <xs:attribute name="screenWidth" type="xs:unsignedInt" use="required"/> <xs:attribute name="screenHeight" type="xs:unsignedInt" use="required"/> <xs:attribute name="pixelWidth" type="xs:double" use="required"/> <xs:attribute name="pixelHeight" type="xs:double" use="required"/> <xs:attribute name="fieldOfView" type="xs:double" use="required"/> <xs:anyAttribute processContents="skip"/> </xs:complexType> </xs:schema> <?xml version="1.0" encoding="UTF-8"?> <xs:schema xmlns="urn:3gpp:metadata:2016:PSS:schemaVersion" xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2016:PSS:schemaVersion" elementFormDefault="qualified"> <xs:element name="schemaVersion" type="xs:unsignedInt"/> <xs:element name="delimiter" type="xs:byte"/> </xs:schema>

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

xmlns="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2016:PSS:SupplementQoEMetric"

elementFormDefault="qualified">

<xs:element name="supplementQoEMetric" type="SupplementQoEMetricType"/>

<xs:complexType name="SupplementQoEMetricType">

<xs:sequence>

<xs:element name="deviceinformation" type="DeviceInformationType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>
    </xs:complexType>

<xs:complexType name="DeviceInformationType">
        <xs:choice>
            <xs:element name="Entry" type="DeviceInformationEntryType" maxOccurs="unbounded"/>
        </xs:choice>
        <xs:anyAttribute processContents="skip"/>
    </xs:complexType>

<xs:complexType name="DeviceInformationEntryType">

<xs:attribute name="start" type="xs:dateTime" use="required"/>
        <xs:attribute name="mstart" type="xs:duration" use="required"/>
        <xs:attribute name="videoWidth" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="videoHeight" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="screenWidth" type="xs:unsignedInt" use="required"/>
        <xs:attribute name="screenHeight" type="xs:unsignedInt" use="required"/>

<xs:attribute name="pixelWidth" type="xs:double" use="required"/>
        <xs:attribute name="pixelHeight" type="xs:double" use="required"/>

<xs:attribute name="fieldOfView" type="xs:double" use="required"/>
        <xs:anyAttribute processContents="skip"/>

</xs:complexType>

</xs:schema>

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema	xmlns="urn:3gpp:metadata:2016:PSS:schemaVersion"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:metadata:2016:PSS:schemaVersion"

elementFormDefault="qualified">

<xs:element name="schemaVersion" type="xs:unsignedInt"/>

<xs:element name="delimiter" type="xs:byte"/>

</xs:schema>

If a supplementQoEMetric needs to be sent when no ordinar QoEMetric are due, a dummy MPDInformation metric shall be sent with codecs="none", bandwidth=0, mimeType="none", representationId="none".

If the attribute qoeReferenceId was defined in the QMC configuration (see clause L.2), the value shall be copied into each QoE report, to facilitate network-side correlation (see [63]). If this attribute was defined the attribute recordingSessionId shall also be returned for each QoE report. The recordingSessionId is a two-byte octet defined by the client. It shall remain the same for all QoE reports belonging to the same streaming session, and it should be different for QoE reports belonging to different streaming sessions.