3GPP TSG-SA4 Meeting #126	S4-231974

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023	revision of S4-231778

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.131 | CR | 0088 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SWB frequency masks for headset UE, desktop hands-free UE, and handheld hands-free UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUET |  |  |  |  |  | Date: |  |  | 2023-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For headset UE (receive only), desktop hands-free UE, and handheld hands-free UE, currently no tolerance masks are defined for frequency response measurements in SWB mode. Defining these performance requirements is part of the work item eUET. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Limits for frequency masks in SWB mode are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For headset UE (receive only), desktop hands-free UE, and handheld hands-free UE are lacking performance requirements for frequency response measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.4.2.2	Headset UE receiving

The sensitivity/frequency characteristics shall be as follows:

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction, shall be within a mask, which can be drawn with straight lines between the breaking points in table 21 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 21: Headset receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [5…6] | [TBD] |
| 200 | [5…6] | [-8…-10] |
| 250 | [5…6] | [-5…-6] |
| 5000 | [5…8] | [-5…-6] |
| 12500 | [5…8] | [-11...-12] |
| 16000 | [5…8] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is recommended as a performance objective that the receiving sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 22.

Table 22: Headset receiving sensitivity/frequency objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3…4] |  |
| 200 | [3…4] | [-4…-6] |
| 250 | [3…4] | [-4…-3] |
| 5000 | [3…4] | [-4…-3] |
| 12500 | [3…4] | [-6…-7] |
| 16000 | [3…4] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

TBD

Figure 17: Headset receiving sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.4.3	Desktop hands-free UE sending

The sending sensitivity frequency response from the MRP to the SS audio output (digital output of the reference speech decoder of the SS) shall be as follows:

The sending sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 23 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 23: Desktop hands-free sending sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [0...5] |  |
| 200 | [3...5] | [-3...-5] |
| 5000 | [3...5] | [-3...-5] |
| 12500 | [3...5] | [-5...-10] |
| 16000 | [3...5] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is recommended as a performance objective that the sending sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 24.

Table 24: Desktop hands-free sending sensitivity/frequency objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3] |  |
| 200 | [3] | [-3] |
| 5000 | [3] | [-3] |
| 12500 | [3] | [-5] |
| 16000 | [3] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

TBD

Figure 18: Desktop hands-free sending sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.4.4	Desktop hands-free UE receiving

The receiving sensitivity frequency response from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the free-field shall be as follows:

The receiving sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 25 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 25: Desktop hands-free receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| [125 | [8] |  |
| [200] | [8] | [-12] |
| [250] | [8] | [-9] |
| [315] | [7] | [-6] |
| [400] | [6] | [-6] |
| [5 000] | [6] | [-6] |
| [12 500] | [6] | [-11] |
| [16 000] | [6] |  |
| NOTE: The limits for intermediate frequencies lie on a straight line drawn between the given values on a linear (dB) - logarithmic (Hz) scale. All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

It is recommended as a performance objective that the receiving sensitivity frequency response be within the mask which can be drawn with straight lines between the breaking points in table 26 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 26: Desktop hands-free receiving sensitivity/frequency objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| [100] | [0] | [] |
| [200] | [0] | [-18] |
| [250] | [0] | [-15] |
| [315] | [0] | [-12] |
| [12 500] | [0] | [-12] |
| [16 000] | [0] |  |

TBD

Figure 19: Desktop hands-free receiving sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.4.5	Hand-held hands-free UE sending

The sending sensitivity frequency response from the MRP to the SS audio output (digital output of the reference speech decoder of the SS) shall be as follows:

The sending sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 27 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 27: Hand-held hands-free sending sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3...5] |  |
| 200 | [3...5] | [-3...-5] |
| 5000 | [3...5] | [-3...-5] |
| 12500 | [3...5] | [-5...-10] |
| 16000 | [3...5] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale |  |  |

It is recommended as a performance objective that the sending sensitivity/frequency response be within the mask which can be drawn with straight lines between the breaking points in table 28.

Table 28: Hand-held hands-free sending sensitivity/frequency objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | [3] |  |
| 200 | [3] | [-3] |
| 5000 | [3] | [-3] |
| 12500 | [3] | [-5] |
| 16000 | [3] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

TBD

Figure 20: Hand-held hands-free sending sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.

### 7.4.6	Hand-held hands-free UE receiving

The receiving sensitivity frequency response from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the free-field shall be as follows:

The receiving sensitivity frequency response shall be within the mask which can be drawn with straight lines between the breaking points in table 29 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 29: Hand-held hands-free receiving sensitivity/frequency requirement mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| [100-315] | [6] |  |
| [630] | [6] | [-12] |
| [800] | [6] | [-6] |
| [4 000] | [6] | [-6] |
| [12 500] | [6] | [-12] |
| [16 000] | [6] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

TBD

It is recommended as a performance requirement that the receiving sensitivity frequency response be within the mask which can be drawn with straight lines between the breaking points in table 30 on a logarithmic (frequency) - linear (dB sensitivity) scale.

Table 30: Hand-held hands-free receiving sensitivity/frequency objective mask

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| [100-315] | [6] | [] |
| [400] | [6] | [-12] |
| [500] | [6] | [-6] |
| [4 000] | [6] | [-6] |
| [12 500] | [6] | [-12] |
| [16 000] | [6] |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |

TBD

Figure 21: Hand-held hands-free receiving sensitivity/frequency masks

Compliance shall be checked by the relevant test described in TS 26.132.