3GPP TSG SA WG4 Meeting #126 	S4-231654

# Chicago, US, 13 - 17 Nov 2023	revision of S4-231472

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0051 | rev | 2 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_Pro_Ph2] Uplink Streaming and multi-entry downlink streaming: unified media entry point |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-18 |
| Reason for change: |  | Defines the media entry points for multi-entry downlink streaming as well as uplink streaming in a unified way: 1. Enables multi-entry downlink streaming 2. Enabes multi-entry uplink streaming 3. Uses the same property for downlink resource URL and the uplink base URL |  |  |  |  |  |  |  |
| Summary of change: |  | - Streaming Access in Service Access Information |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of support in both cases of downlink and uplink streaming. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 8.1, 10.3 (new) |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-231255: initial version. S4-231472: endorsed in SA4#125 S4-231654: 10.3: contribution protocols |  |  |  |  |  |  |  |

FIRST CHANGE

# 8	Media ingest and publish (M2) protocols

## 8.1	General

The set of content protocols supported by the 5GMS AS is listed in table 8.1-1 below:

Table 8.1-1: Supported content protocols

| Description | Term identifier | Clause |
| --- | --- | --- |
| Content ingest protocols at reference point M2d |  |  |
| HTTP pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 8.2 |
| DASH-IF push-based content ingest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 8.3 |
| Content egest protocols at reference point M2u |  |  |
| HTTP pull-based content egest protocol | urn:3gpp:5gms:content-protocol:http-pull-egest | 8.4 |
| DASH-IF push-based content egest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 8.5 |

-

NEXT CHANGE

## 10.3	Contribution protocols

### 10.3.1	General

The contribution protocols supported by the 5GMSu AS at reference point M4u are listed in table 10.3-1 below:

Table 10.3.1-1: Supported contribution protocols at reference point M4u

| Description | Term identifier | Clause |
| --- | --- | --- |
| DASH-IF push-based content ingest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 10.3.1 |

### 10.3.2	DASH-IF push-based contribution protocol

If streamingAccess.profiles is set to urn:3gpp:5gms:content-protocol:dash-if-ingest in the Service Access Information, media resources shall be streamed to the 5GMSu AS as specified by the DASH-IF Live Media Ingest specification [3]. The properties of the content shall be specified in streamingAccess.contentType using one or more of the values specified in table 6 of clause 7.1.2 of [3]. The content shall conform to at least one of the conformance profiles listed in streamingAccess.profiles, if any.

END OF CHANGES