3GPP TSG-SA4 Meeting # 126	S4-231713

# Chicago, US,  13-17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | pseudo | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Device capabilities metadata |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adds the metadata to convey the device capabilities to the split rendering server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A JSON object that includes the following parameters: - The device profile if available |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | As indicated in 8.4.1, the split-rendering server needs to know the capabilities of the device to define the split-rendering process on the edge. This needs to be achieved either by high-level signaling of the device profile (e.g. pixel streaming profile) and/or by listing the detailed capabilities of the devices. Currently, such a feature is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 8.4 	Split Rendering Formats for Session Setup and Negotiation

### 8.4.1 	General

In Figure 5.2.1-1 and 5.2.1-2, step 5 defines the negotiation between the SRC and SRS for the split-rendering configuration. In most simple case, the SRC provides SRS the capabilities of the device and if SRS can accommodate the split-rendering processing that addresses the device, it confirms the configuration. In such scheme, the SRS is responsible to make the decision and no back-and-forth negotiation occurs.

### 8.4.2 	Device capabilities

The device capabilities shall be in JSON format. It shall have the following information.

Table 8.4.2-1: Device capabilities object

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| splitRenderingProfile | URL | 0..N | Device split-rendering profile identifier. The profile identifiers are listed in Annex C for each profile. |

### 8.4.3 	Split Rendering Configuration Format