3GPP TSG-S4 Meeting # 126	S4-231791

Chicago, US,  13th - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 061 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Defining the Notification Channel to the MSH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 7th November 2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure for notifications between the MSH and the 5GMS AF is underspecified and may not be implemented in an interoperable manner. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to the MQTT specification. Generalize the notification procedure to carry different types of notifications and to clearly identify the related session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of the notification procedure will remain ambiguous and interoperability cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

…

[30]		OASIS: "MQTT Version 5.0",
https://docs.oasis-open.org/mqtt/mqtt/v5.0/mqtt-v5.0.html.

2nd Change

### 4.7.6	Procedures for network assistance

These procedures are used by the 5GMS Client to request Network Assistance from one of the 5GMS AF instances listed in the networkAssistanceConfiguration.serverAddresses property of the Service Access Information resource retrieved using the procedure in clause 4.7.2.3. Details of the APIs supporting these procedures are specified in clause 11.6.

The 5GMS Client first creates a Network Assistance Session with its chosen 5GMS AF instance. It provides information that will later be used by the 5GMS AF to request a particular network QoS to be applied by the PCF to one or more application data flows, and to recommend a bit rate to the 5GMS Client. This procedure is further specified in clause 11.6.4.1.

The NetworkAssistanceSession resource may be retrieved by the Media Session Handler using the procedure specified in clause 11.6.4.2.

When a Network Assistance Session is created, the responding 5GMS AF instance may nominate an MQTT [30] endpoint URL in the NetworkAssistanceSession.notificationURL property. The Media Session Handler may subscribe to the MQTT channel provided at this endpoint and receive notifications with an up-to-date bit rate recommendation whenever this changes. The usage and message formats for MQTT are described in clause 11.7.

At any time after the Network Assistance Session resource is created, the 5GMS Client may use the Network Assistance Session identifier to explicitly request a bit rate recommendation by invoking a remote procedure call provided for this purpose by the 5GMS AF. This procedure is further specified in clause 11.6.4.3.

Using the Network Assistance identifier, the 5GMS Client may also request a delivery boost to be provided by the 5G System at any time by invoking a remote procedure call provided for this purpose by the 5GMS AF. This procedure is further specified in clause 11.6.4.4.

The information provided when first creating a Network Assistance Session may be modified subsequently by the Media Session Handler using the session modification procedure specified in clause 11.6.4.5.

In order to terminate a Network Assistance Session, the 5GMS Client deletes the Network Assistance session resource. This procedure is further specified in clause 11.6.4.6.

3rd Change

## 11.7	Notification Channel

### 11.7.1	General

The Media Session Handler and the 5GMS AF shall support the usage of an MQTT notification channel that is used by the 5GMS AF to notify the Media Session Handler about updates to the media streaming session or any related configuration information.

For each ongoing media streaming session that uses any of the 5GMS procedures, the Media Session Handler shall subscribe to the associated MQTT notification channel.

For each ongoing media streaming session that the 5GMS AF is managing, it shall offer a notification channel for the Media Session Handler to subscribe to.

In addition, the 5GMS AF shall offer a generic notification channel that is dedicated to all sessions of a specific Application Service Provider with an active Provision Session a the 5GMS AF. This channel may be used to share notifications about updates to Service Access Information, Background Data Transfer, or any other media session handling information.

The 5GMS AF shall use a trusted MQTT broker for the exchange of these notifications. The MQTT broker shall use appropriate client authentication mechanisms, such as a token-based authentication mechanism (JWT) to secure access to the notification channel.

The MQTT Topics shall be structured as defined in clause 11.7.2.

The MQTT Messages may be formated according to clause 11.7.3.

### 11.7.2	Usage of MQTT Topics

The Topic shall be formatted according to the following ABNF syntax:

Topic= provisioningSessionId “/” media_session_id

Where:

- provisioningSessionId is the Provisioning Session identifier associated with the notifications over this notification channel.

- media_session_id is a unique identifier, assigned by the 5GMS AF to a media streaming session. This identifier also includes a unique identifier of the managing Media Session Handler.

The Media Session Handler shall at least subscribe to the “{provisioningSessionId}/#” topic, enabling the reception of all notification messages that are associated with a particular Provisioning Session.

### 11.7.3	Notification message format

The 5GMS AF shall format each notification it publishes to the Media Session Handler as an MQTT Application Message conveyed as the payload of an MQTT PUBLISH message.

- The Topic property of the Variable Header shall be as specified in clause 11.7.2.

-	The Payload Format Indicator property of the Variable Header shall indicate UTF-8 encoding of the Payload field.

The notification message shall be conveyed in the Payload field which shall be a formatted as a JSON [38] object using the UTF-8 character encoding as specified in table 11.7.3-1:

Table 11.7.3-1: NotificationMessage data type

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| type | string enum | 1..1 | The type of notification message. |
| serviceAccessInformation | ServiceAccessInformation | 0..1 | Present if type is SERVICE_ACCESS_INFORMATION. |
| dynamicPolicy | DynamicPolicy | 0..1 | Present if type is DYNAMIC_POLICY. |
| networkAssistanceSession | NetworkAssistanceSession | 0..1 | Present if type is SERVICE_ACCESS_INFORMATION. |