3GPP TSG-SA WG4 #126	S4-231659 r1

Chicago, USA, 13-17 November 2023

Source:	Dolby Laboratories Inc. (Rapporteur)

Title:	[FS_FGS] Some Updates on Film Grain Synthesis Testing

Version:	0.2

Agenda Item:	3.7

Document for:	Discussion

# 2	Proposed Scenario and evaluation framework

In order to evaluate the performance and justify potential benefits a qualitative and quantitative analysis on what would be the costs and benefits would be needed.

| # | Parameter | Definition |
| --- | --- | --- |
| 1 | Scenario name | Film Grain Synthesis Scenario |
| 2 | Motivation for the scenario | Study, test and analyze effect of film grain synthesis applied to varied test scenarios encoded with H.265/HEVC, in order to ascertain whether there is benefit in (1) preservation of artistic intent and/or (2) masking artifacts / saving bitrate while maintaining visual acuity, etc. |
| 3 | Description of the scenario | Film grain can be unavoidably lost by conventional video compression workflow shown in Figure 1. The high frequency grain in the source content (x) is either completely or partially lost due to quantization in adaptive streaming or broadcast applications, the resulting decoded output x is void of the artistic effect of grain intented in source. Even if grain is partially preserved at high bitrates, overall coding efficiency is poor as the grain is not spatially or temporally predictable and significant number of bits are used in coding the high frequency components of the noisy grain. In case of Film Grain Preservation Workflow as per Figure 2, the source video (x) is input to a denoising process that outputs a video sequence from which noise or film grain is attenuated or removed. A film grain parameterization process then compares the source and denoised videos to determine film grain model parameter values, which relate to the variance, spatial frequency characteristics, colour correlation, and other statistical characteristics of the film grain. The process of denoising followed by the film grain model parameter estimation is commonly referred to as the film grain analysis process. After these processes are performed, the denoised video is then encoded and the film grain model parameter values are either signalled in the coded bitstream (ydn) or provided to the decoder by some external means. On the decoder side, the film grain model parameter values are parsed and input to a film grain synthesis process that generates simulated film grain and blends the grain with the decoded video (xdn) to output decoded video with simulated film grain (xg). |
| 4 | Supporting companies and 3GPP members |  |
| 5 | Source format properties - Spatial resolutions - Chroma Format - Chroma Subsampling - Aspect ratios - Frame rates - Colour space formats - Transfer Characteristics - Bit depth - Other signal properties | 1920x1080, 3840,2160 YUV 420P 16: 9 24, 29.97, 30 ,59.94 ,60 ITU-R BT.709 or BT.2020 BT.709, PQ 8, 10 |
| 6 | Encoding and decoding constraints and settings: Typical encoding constraints and settings such as - Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. - Random access frequency - Error resiliency requirements - Bitrates and quality requirements - Bitrate parameters (CBR, VBR, CAE, HRD parameters) - ABR encoding requirements (switching frequency, etc.) - Latency requirements and specific encoding settings - Encoding context: real-time encoding, on device encoding, cloud-based encoding, offline encoding, etc. - Required decoding capabilities | HEVC/H.265 Main Profile & Main10 Profile, Level 5.3 Detailed config parameters embedded in cfg file below |
| 7 | Performance Metrics and Requirements - A clear definition on how the performance needs to be evaluated including metrics, etc addressing the main KPIs of the scenario. - Objective measures such as PSNR, VMAF, etc, may be used. - Subjective evaluation is not excluded and may be done, but needs commitment | Subjective Metrics - MOS (pairwise method with hidden reference) - Relative Grading (A v/s B method) - (to be added) JND (Just Noticeable Difference) subjective tests |
| 8 | Interoperability Considerations for the application - Streaming with DASH/HLS/CMAF - RTP based delivery | The following media formats have been tested and validated for FGS interoperability. - MP4 - MPEG-DASH - HLS |
| 9 | Test Sequences - A set of selected test sequences that are provided by the proponents in order to do the evaluation. They should cover a set of source format properties | Preserving Artistic Intent Scenario FHD - BQTerrace - OldTownCross - InToTree Scenario 4K-TV - Crowdrun - Meridian (from existing 3GPP dataset) |
| 10 | Detailed test conditions: - Provides a proposal for detailed test conditions, for example based on a reference software together with the sequences and configuration parameters. | Following denoiser, HEVC software codecs with film grain analysis and synthesis support are used to assess grain characterization: - Denoiser: ffmpeg ‘nlmeans’ filter is used to denoise source content as pre-processing step before encoding. Sample Commnad ./ffmpeg -f rawvideo -pix_fmt yuv420p -s:v 1920x1080 -I ../InToTree_1080p50.yuv -vf nlmeans=3.5:5:3:9:7 InToTree_nlmeans_3.5_5_3_9_7.yuv; - Film grain analysis algorithm: Frequency filtering analysis algorithm in HM-18.0 reference encoder is used to model the grain parameters to be inserted in FGC SEI - Film grain synthesis algorithm: SMPTE RDD5 based Frequency filtering grain synhthesis algorithm in HM-18.0 reference decoder is used for blending grain with decoded video to output grainynthesizedd video xg - Browser player with FGS support: For Subjective evaluation, Ittiam HEVC FGS web browser player is used as it supports SMPTE RDD5 based frequency film grain synthesis using the FGC SEI messages embedded in HEVC bitstream. https://demo.ittiam.com/demo/i265_fgs/ |
| 11 | External Performance data |  |
| 12 | Additional Information |  |

An independent scenario for FGS may be defined.

# 4	Communication with JVET

It is agreed to send an LS to JVET informing them of our intent to evaluate the impact of FGS on our 3GPP scenarios listed in this study.

This LS is intended to request information on their ongoing activity with AG5 on similar evaluation in order to see if they can meet our timeline.

This clause is a placeholder for information we want to share with JVET and our requests.

- Inform JVET on the study item on Film Grain Synthesis (list the objectives or point to/attach the SID)

- Inform JVET on our timeline justified by the intent to meet release18 for normative work.

- Explain to JVET the type of evaluations that we will conduct in SA4

- Ask JVET if they have similar activities planned/ongoing/done and see if they can share results (or by when).

An LS is expected to be sent before the next JVET meeting (11th October 2023).

Update: The LS has been sent (S4-231587) to MPEG/JVET (m65645) and a response statement is generated from the October MPEG/JVET meeting (S4-231648, 3GPP SA4 #126).

ANNEX: FGS Testing Methodology and Results

FGS workflow for preserving artistic intent.

Film grain can be unavoidably lost by conventional video compression workflows that are shown in Figure 1. The high frequency grain in the source content (x) is either completely or partially lost due to quantization in adaptive streaming or broadcast applications, the resulting decoded output x is void of the artistic effect of grain intented in source. Even if grain is partially preserved at high bitrates, overall coding efficiency is poor as grain is not spatially or temporally predictable and significant number of bits are used in coding the high frequency components of the noisy grain.

Figure 1 HEVC workflow without film grain

HEVC Encoder

Video Stream

Film Grain Analysis Disabled

HEVC Decoder

x

y

x

Following notations are used for the film grain preservation workflow depicted in Figure 2:

- x is source video (uncompressed)

- xdn is denoised source video.

- ydn is compressed bitstream of denoised video.

- xdn is the decoded denoised video.

xg is the grain synthesized video, using the FGC SEI parameters.

Figure 2 HEVC film grain modeling and synthesis workflow

De-Noising

HEVC Encoder

x

-

Film Grain Analysis

VideoStream

Film Grain Analysis Enabled

Difference Noise

Film Grain SEI

HEVC Decoder

+

xdn

ydn

xdn

xg

Conventional codec

Film grain framework modules

Encoder side

Decoder side

Film Grain Synthesis

Quality Evaluation Methods for FGS

The benefits of grain characterization and synthesis to preserve artistic intent are assessed using subjective visual verifications test scores of 11 to 14 viewers.

- x

- xdn

Subjective Performance Evaluation Methods

- MOS Scoring Method

  - ITU-T_REC-P190 based subjective performance evaluation method (pair-wise testing with hidden reference). This method is a category judgement where the test sequences are presented one at a time and are rated independently on a category scale (1 to 10). The present test procedure must include a reference version of each test sequence shown as any other test stimulus.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Samsung 65” S95B, HDMI (3840×2160) |
| Viewing distance | 2 viewers sitting at 1.5H, 1 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 14 (5 female, 9 male) |

- A-B Preference Method

  - The test sequences were evaluated using a 5-score scale, the 5 scores (A>>B, A>B, A==B, A<B, A<<B) were mapped to the values (3, 1, 0, -1, -3) and the random A-B assignment was reverted. In the comparison of the compressed sequences with and without FGS, positive numbers indicate the FGS version to be scored higher.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Samsung 65” S95B, HDMI (3840×2160) |
| Viewing distance | 2 viewers sitting at 1.5H, 1 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 11 (3 female, 8 male) |

xg(x)

|  | x |  |  |  | xg |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Scenario FHD Test Results (Subjective)

Scenario 4K-TV Test Results (Objective)

xg(x)

|  | x |  |  |  | xg |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Scenario 4K-TV Test Results (Subjective)

FGS workflow for masking coding artifacts

The classical use case of film grain technology is to be used in an encoder to remove the original film grain from the source video to reduce bitrate for distribution. The film grain model parameters of the original film grain are coded as parameters in the bitstream. The receiver can use the film grain parameters to re-synthesize the film grain to help preserve the original look of the video. Film grain technology can also be used with video sources that do not have original film grain. In this case, it can be used as a compression aid to improve the subjective quality by subtly masking artifacts such as blockiness, banding and blurred textures at low bitrates.

Figure 3 HEVC workflow for film grain estimation for masking coding artifacts

HEVC Encoder

Film Grain Estimation

VideoStream

Film Grain Estimation Workflow

Film Grain SEI

HEVC Decoder

+

xd

y

x

xg

Conventional codec

Film grain framework modules

Encoder side

Decoder side

Film Grain Synthesis

xd

FGC SEI Inserter

yfg

Following notations are used for the film grain estimation for masking coding artifacts workflow depicted in 3:

- x is source video (uncompressed)

- yfg is compressed bitstream with FGC SEI message inserted.

- xd is the decoded video.

xg is the grain synthesized video, using the FGC SEI parameters.

Subjective Test Results A|B Method with DMOS

Just Noticeable Difference (JND) Subjective Test Method

In experimental psychology, the term Just Noticeable Difference (JND) means the amount of something that must be changed in order for a difference to be noticeable, such that it may be detected at least 50% of the time.  This is applied in various sensation and perception studies.  In the context of the film grain, this JND method may be used for either to test for artistic intent or for masking visual artifacts.

The JND objective test measures these two things depending on the use case via side by side comparison:

- Preservation of artistic intent: measure the difference between the clean reference and the reference video with added FGS,

- Applying film grain to mask coding artifacts: find the highest compression level, adjusted by QP, at which point the coding artifacts become noticeable with and without added FGS.

|  | Without film grain synthesis | With film grain synthesis |
| --- | --- | --- |
| Reference | true lossless quantization | true lossless quantization with film grain synthesis |
| Test | compressed at various QPs in [18, 42] | compressed at various QPs in [18, 42] with film grain synthesis |

  - If the video file size is not limited, just use the original reference video. If the video size needs to be small, use QP=4 for the reference video.

  - Keep the QP the same between the video with / without FGS applied.

The steps of this JND test method is as follows:

- Initial stimulus: set at lowest quality.  Initial ΔQP set at 8.

- When a subject provides a correct response (i.e., there is difference between the two videos or the reference video looks better than the test video), the QP value is decreased by ΔQP.

- When a subject makes an incorrect response (i.e., no difference between the two videos or the test video looks better than the reference video), the QP value is increased by ΔQP.

- How we determine the value of ΔQP:

  - The initial ΔQP is set to 8.

  - Keep the previous ΔQP when the same response follows an earlier response.  That is, an incorrect response after earlier incorrect response, or a correct response after a previous correct response occurs.

  - ΔQP is reduced to half of the previous ΔQP, when an incorrect response occurs after a correct response or vice versa.

  - The chart below illustrates an example of adaptive QP adjustment according to a subject’s response.

  - This process stops when it reaches either of the stopping conditions below:

    - 4 times of upward and downward pattern then arrive at QP=1 step size,

    - When the number of responses reach the maximum trial number of responses. (This maximum number is TBD)

  - The JND threshold value is determined as the last correct response.  For example in the chart above the JND threashold value corresponds to the last point in the chart.

  - Prior to the above test, a training session is needed; the best set of examples showing significant difference and no difference using all test sequences will be presented to the test subject.

    - 2 different types of comparison

      - Both the reference and the test are FG-free.  The other set is for the reference and the text material both to have FGS added.

      - There is no direct side by side comparison between the clean (no FG) and the video with FG.

      - The order of the test videos is randomized.  The playback time for each sample video is 10 seconds.