3GPP TSG-SA4 Meeting #126	S4-231675

# Chicago, USA, 13th - 17th, Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.812 | CR | pseudo | rev | - | Current version: | 0.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_ARMRQoE] pCR on observation points |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_ARMRQoE |  |  |  |  |  | Date: |  |  | 11-02-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition of AR/MR QoE reference model and Metrics Observation Points need to be aligned to TS 26.119 v0.3.0. Some clarifications need to be added on the observation points part. Based on the latest progress of MeCar PD v8.2.1, additional clarifications are needed on the observation points. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the the Figure 6.2-1: AR/MR QoE reference model and Metrics Observation Points, and exchange the definitions of OP3 and OP4. - Adding description on Scene Information and buffers formats for OP-2. - Adding description on MAF-API for OP-3. - Adding description on scene information and buffer formats for OP-4. - Editorial change for the title of section 7.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AR/MR QoE reference model and Metrics Observation Points will not be aligned to MeCar WI. Editor’s note are not addressed in OPs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2, 6.2.2, 6.2.3, 6.2.4, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.2	AR/MR QoE reference model

The AR/MR QoE framework and the observation points defined in clause 5.1 of TS 26.119 [5] can be reused as baseline for the AR QoE reference model, which is illustrated in Figure 6.2-1.

NOTE:	The observation points can also be used to identify the advanced AR/MR QoE metrics.

Figure 6.2-1: AR/MR QoE reference model and Metrics Observation Points

It’s also noted that the above observation points may be further updated based on the agreements of the AR/MR QoE metrics identification and definition.

END of 1st Change

2nd Change

6.2.2	Observation Point 2

Scene Manager is a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the XR Runtime. XR Scene Manager has access to the latest pose and tracking information from the XR Runtime which is then provided. Based on this information, the Scene Manager may for example determine the objects visible to the user at a given point in time or more generally the objects that may be needed to be rendered in the next rendering cycles.

Media Access Function is a set of functions that enables access to media and other XR-related data that is needed in the Scene manager or XR Runtime to provide an XR experience. The media access function accesses the network resources or sends data to the network using the established media pipelines.

Observation point 2 is derived from the API which exchanges scene description information and primitive buffers formats between Scene Manager and Media Access Function. Scene description information may be generated from the application or in a scene description delivery document. The primitive buffers are defined via this API for different media types that can be rendered by the Visual and Audio renderer. The combination of the scene description information and the primitive buffers provide sufficient information to the presentation engine in order create an immersive audio-visual experience. OP2 is defined to monitor:

Editor’s Note: Parameters to be monitored in OP2 are FFS, which can be aligned with MeCar WI TS 26.119.

6.2.4	Observation Point 3

Observation point 3 is derived from the API which exchanges information between XR Source Management and Metrics Access Functions.  Information collected from XR Runtime may be serialized, time stamped, and compressed via this API, including: Viewer pose and projection parameters needed to render using the xrLocateViews function to render each view for use in a composition projection layer, camera and microphone output, etc. For XR metadata to be possibly delivered over a network interface, the raw formats including information on timing needs to be defined in order to permit serialization of the data in metadata delivery. Information includes viewer pose, triggers and actions, etc.

OP3 is defined to monitor:

-	Viewer pose

Editor’s Note: Additional parameters to be monitored in OP3 are FFS, which can be aligned with MeCar WI TS 26.119.

6.2.3	Observation Point 4

Observation point 4 is derived from the API which exchanges information between Media Access Function and 5G System. The information delivered from this API includes: Scene Description, Audio and video formats for parallel decoding of multiple buffers, pose information, metadata, etc. This API is equivalent to the 3GPP VR Operation Point as defined in TS 26.118, which includes spatial and temporal resolutions, and the encoding format, etc.

OP4 is defined to monitor:

-	Spatial and temporal resolutions

Editor’s Note: Additional parameters to be monitored in OP4 are FFS, which can be aligned with MeCar WI TS 26.119.

END of 2nd Change

3rd Change

7.1	NWDAF based metrics configuration and collection

The Network Data Analytics Function (NWDAF) is a core network function performing

-	Inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and analytics results exposure.

-	Training Machine Learning (ML) models and new training services (e.g. providing trained ML model) exposure.

The analytics information is either statistical information of the past events, or predictive information. The NWDAF can collect the service experience, e.g. subscribing to the DC-AF for the QoE metrics. Based on the collected QoE metrics, the NWDAF may provide the estimation of the service experience for a specific application, a specific network slice or a specific UE as defined in clause 6.4 of TS 23.288 [25]. The AR/MR QoE metrics in present document also can be used by NWDAF based metrics configuration and collection.

END of 3rd Change