Source:	Bytedance

Title:	[ISAR] Cross Check report for ISAR MASA rendering

Agenda item:	7.7

Document for:	Discussion and Agreement

1. Introduction

According to the plan of ISAR group, Bytedance would perform MASA cross-validation for split rendering before the 127-bis-e meeting. Among them, the experimental preparation was from March 21st to 25th, and subjective listening was from March 27th to 28th. Then analyzed data and wrote test report from March 29th, and reviewed this report on March 30th.

2. Experiment

2.1 Experimental basis

ITU-R BS.1534-3 Method for the subjective assessment of intermediate quality level of audio systems.

2.2 Experimental environment

2.2.1 Platform

Test platform built based on webMushra that refer from this site：https://github.com/audiolabs/webMUSHRA

Fig.1 Mushra platform

2.2.2 Listening room

Area：19.4㎡(4M*5m)

Noise rating number：25dB

Sound Transmission Class：55dB

Reverberation time：0.3s(+-0.1s)

Fig.2 Listening room

2.2.3 Headphone

Name：Beyerdynamic DT880

Type of headphones: Over-ear

Sensitivity: 96dB

Frequency response range: 5-35000Hz

Fig.3 Listening headphone

2.3 Experimental material

In the ISAR test plan, there are several suggested key elements and 4 experiments are proposed. This report corresponds to Experiment 2: Testing against performance requirement for MASA. Test items are described as follows:

- Hidden reference: Native coding system (IVAS@512kbps rendered to post renderer pose)

- LP7 anchor: Hidden reference, 7Khz LP filtered

- 0-DOF native transcoding reference (IVAS@512kbps binaurally rendered to pre-renderer pose, IVAS stereo coded@256kbps)

- System 1 under test

All audio files are downloaded from the subfolders of …\Box\_EXTERNAL_ISAR_LL\BS1534-4_Bytedance(https://dolby.box.com/s/baw46gcujcku80knlkami15btm1zlp5g).

Table 1. subjective evaluation test sequences

| No. | Name | No. | Name |
| --- | --- | --- | --- |
| 1 | HOA3_3_HOA2traj4.c01-04.wav | 7 | ism4_7_HOA2traj7.c01-04.wav |
| 2 | ism4_1_HOA2traj6.c01-04.wav | 8 | ism4_8_HOA2traj2.c01-04.wav |
| 3 | ism4_2_HOA2traj2.c01-04.wav | 9 | ism4_9_HOA2traj7.c01-04.wav |
| 4 | ism4_4_HOA2traj1.c01-04.wav | 10 | ism4_10_HOA2traj5.c01-04.wav |
| 5 | ism4_5_HOA2traj5.c01-04.wav | 11 | ism4_11_HOA2traj3.c01-04.wav |
| 6 | ism4_6_HOA2traj8.c01-04.wav | 12 | ism4_12_HOA2traj3.c01-04.wav |

2.4 Listeners

Listeners include recording engineers, audio quality evaluators, audio technical staff and other professionals, 14 people in total.

2.5 Evaluation method

In this subjective test, listeners score each item according to the similarity between the measured item and the reference, mainly compare these characteristics such as timbre, sound quality, sense of space, sense of out head, etc.

2.6 Preparation and training

Listeners must read the experimental design doc before formal test, to guarantee everyone knows well the experimental procedure that includes how to use the test platform, how to score test items, what test step is, etc.

Listeners must do a set of practice to familiar test environment and test notices before formal tests. Practice materials are chosen from formal experimental materials.

3. Discussion and Suggestion

3.1 Post screening

The post-screening method excludes those listeners who rate an obviously impaired anchor signal very highly and those who frequently rate hidden references as obviously impaired.

- If an evaluator scores less than 90 on the hidden reference condition for >15% of the test items, they should be excluded from the collected responses

- If an evaluator scores more than 90 on the anchor condition for >15% of the test items, they should be excluded from the collected responses

Post screening results: For the evaluations of the overall binaural effect on MASA, the data of 10 out of 14 evaluators were used for subsequent statistics.

3.2 Results

The mean score of each test sequence and its 95% confidence interval are in Table 2:

Table 2 MASA test details and [95%]

| No. | Item | C01 |  | C02 |  | C03 |  | C04 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mean | 95% Confidence Interval | Mean | 95% Confidence Interval | Mean | 95% Confidence Interval | Mean | 95% Confidence Interval |
| 01 | HOA3_3_HOA2traj4 | 92.8 | [87.7,97.8] | 42.8 | [31.0, 54.5] | 68.3 | [49.2, 87.3] | 89.2 | [81.4, 96.9] |
| 02 | ism4_1_HOA2traj6 | 87.8 | [78.4, 97.1] | 43.5 | [25.9, 61.0] | 70.8 | [56.7, 84.8] | 85.5 | [74.0, 96.9] |
| 03 | ism4_2_HOA2traj2 | 92.6 | [86.6,98.5] | 39.6 | [23.9, 55.2] | 74.2 | [60.4, 87.9] | 90.1 | [83.8, 96.3] |
| 04 | ism4_4_HOA2traj1 | 89.9 | [80.8, 98.9] | 52.6 | [33.1, 72.0] | 62.2 | [43.8, 80.5] | 87.7 | [81.0, 94.3] |
| 05 | ism4_5_HOA2traj5 | 89.0 | [81.7, 96.2] | 36.5 | [22.9, 50.0] | 62.6 | [46.4, 78.7] | 89.4 | [82.1, 96.6] |
| 06 | ism4_6_HOA2traj8 | 91.0 | [85.6, 96.3] | 39.2 | [25.7, 52.6] | 76.2 | [63.2, 89.1] | 88.4 | [81.6, 95.1] |
| 07 | ism4_7_HOA2traj7 | 89.3 | [83.0, 95.5] | 54.7 | [43.2, 66.1] | 83.0 | [71.7, 94.2] | 89.3 | [78.7, 99.8] |
| 08 | ism4_8_HOA2traj2 | 92.1 | [87.1, 97.0] | 39.5 | [24.4, 54.5] | 80.2 | [70.5, 89.8] | 86.8 | [80.1, 93.4] |
| 09 | ism4_9_HOA2traj7 | 85.5 | [75.8, 95.1] | 36.8 | [23.3, 50.2] | 72.6 | 57.9, 87.2] | 90.7 | [84.8, 96.5] |
| 10 | ism4_10_HOA2traj5 | 90.2 | [82.8, 97.5] | 40.1 | [23.0, 57.1] | 80.2 | [69.5, 90.8] | 86.3 | [76.2, 96.3] |
| 11 | ism4_11_HOA2traj3 | 83.9 | [69.9, 97.8] | 36.4 | [20.3, 52.4] | 76.6 | [61.2, 91.9] | 87.7 | [78.0, 97.3] |
| 12 | ism4_12_HOA2traj3 | 85.8 | [71.2, 100.3] | 35.4 | [24.1, 46.6] | 87.5 | [79.7, 95.2] | 87.7 | [79.6, 95.7] |

Visualization of mean score of each test sequence and its 95% confidence interval are in Fig.4

Fig.4 Mushra test details of MASA

Overview of C1-C4 (mean&95) is in Table 3

Table 3 MASA test summarize and [95%]

| C01 |  | C02 |  | C03 |  | C04 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mean | 95% Confidence Interval | Mean | 95% Confidence Interval | Mean | 95% Confidence Interval | Mean | 95% Confidence Interval |
| 89.1 | [86.9,91.3] | 41.4 | [37.6, 45.1] | 74.5 | [70.9, 78.1] | 88.2 | [86.2, 90.2] |

Visualization of overview of C1-C4 in Fig.5

Fig.5 Mushra test overview of MASA

In above tables and figs, C1 represents hidden reference, C2 represents Anchor, C3 represents 0 DoF transcoding reference, C4 represents system 1 under test.

4. Conclusion

In this MASA cross checking, the test results are C1 > C4 > C3 > C2. In addition, a follow-up interview with the tester found that the sound location of C3(0 DoF) materials are not stable enough compared to other products, that may be due to the rendering method.

References

https://www.itu.int/dms_pubrec/itu-r/rec/bs/R-REC-BS.1534-3-201510-I!!PDF-E.pdf

Selection Test and Processing Plan Aspects for IVAS Specific ISAR Solutions