3GPP TSG-SA4 Meeting #127-bis-e	S4-240636

# Online, 8th – 12th Apr 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0027 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment on support of MBS data reception for UEs using power saving functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21. 900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, SA2 5MBS_Ph2 introduced the support of MBS data reception for UEs using power saving functions. The MBS User Service Announcement needs to be enhanced to further include a start time and/or a sequence of scheduled activation times (e.g. a first time and a periodicity) for corresponding MBS distribution session, considering the unreachable times for the UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support of MBS data reception for UEs using power saving functions to align with other WGs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and misaligned designs among WGs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new), 4.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *(all new text)

4.X	Support of MBS data reception for UEs using power saving functions

When power saving functions are enabled in the UE side, e.g. MICO (Mobile Initiated Connection Only) mode with Active Time, or extended DRX (Extended Discontinuous Reception) as defined in clause 5.31.7 of TS 23.501 [2], the UEs are usually unreachable for long periods of time. In order to make sure those UEs can wake up at coordinated times when the MBS data is to be transmitted, a start time and/or a sequence of scheduled activation times (e.g. a first time and a periodicity) may be provided to the UE in the service announcement as described in clause 6.16 of TS 23.247 [5].

The MBS Application Provider may provide the Active periods to the MBSF (directly or via NEF) for corresponding MBS Distribution Session when creating the MBS User Data Ingest Session. The Active periods indicate a start time and/or a sequence of scheduled activation times (e.g. a first time and a periodicity). Then the MBSF compiles the above coordinated times into the Service Announcement which is further delivered to the MBS client.

# * * * * Second change * * * *

4.5.5	MBS User Data Ingest Session parameters

This entity models an MBS User Data Ingest Session, as provisioned by the MBS Application Provider and as managed by the MBSF. The baseline parameters for an MBS User Data Ingest Session are listed in table 4.5.5-1 below.

NOTE:	A linkage from the MBS User Data Ingest Session to its parent MBS User Service is additionally required at stage 3. The User Service identifier defined in table 4.5.3-1 serves this purpose.

The set of active periods may be updated by the MBS Application Provider at any time. The state of constituent MBS Distribution Sessions (and their corresponding MBS Distribution Session Announcements) may need to change as a consequence.

Table 4.5.5-1: Baseline parameters of MBS User Data Ingest Session entity

| Parameter | Cardinality | Assigner | Description |
| --- | --- | --- | --- |
| User Data Ingest Session Identifier | 1..1 | MBSF | An identifier for this MBS User Data Ingest Session that is unique in the scope of the parent MBS User Service (see clause 4.5.3). |
| MBS User Service Announcement | 0..1 |  | The MBS User Service Announcement (see clause 4.5.7) currently associated with this MBS User Data Ingest Session. Present only if all constituent MBS Distribution Sessions are in the ESTABLISHED or ACTIVE state. |
| Active periods | 0..* | MBS Application Provider | Periods of time during which the MBS User Data Ingest Session is active in the MBS System. This may be determined considering the unreachable times for UEs using power saving functions as described in clause 6.16 of TS 23.247 [5]. If omitted, the MBS User Data Ingest session is intended to be active until further notice. |

The MBS User Data Ingest Session is composed of one or more MBS Distribution Sessions (see clause 4.5.6 below) and these shall be provisioned in the same operation as the enclosing MBS User Data Ingest Session. It is not valid for an MBS User Data Ingest Session to have no MBS Distribution Sessions defined.

MBS Distribution Sessions may be added to or removed from an MBS User Data Ingest Session by the MBS Application Provider at any time, subject to the minimum number specified above. The MBS User Service Announcement may need to change as a consequence to refer to a revised set of corresponding MBS Distribution Session Announcements.

# * * * * End of changes * * * *