3GPP SA4#127-bis-e	S4-240673

Online, 08-12 Apr 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | pseudo | rev |  | Current version: | 1.1.4 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improving M6 general parts |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Updating clause 10, interface at reference point M6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 10.2.1: new entry in the table - 10.2.2: updating the methods - 10.2.3: updating the events |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | M6 is underdefined and not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 10	UE media session handling APIs

## 10.1	Introduction

This clause defines the client APIs exposed by the Media Session Handler to the Media-aware Application at reference point M6 and to the Media Access Function at reference point M11.

NOTE:	Client-driven management of edge processing resources via reference point M6 is not specified in this release.

## 10.2	Media Session Handler client API

### 10..2.1	Media Session Handler internal properties

The Media Session Handler maintains internal properties as defined table 10.2.1-1. Note that the parameters are conceptual and internal. They serve only for the purpose of defining the media session handling APIs.

Table 10.2.1-1: Parameters of Media Session Handler

| States and Parameters |  | Definition |
| --- | --- | --- |
| _Configuration[externalServiceId] |  | The Media Session Handler maintains a separate configuration for each set of Service Access Information it has knowledge of, indexed by its external service identifier. |
|  | _mediaAccess | Media access information including the provisioning session type and streaming access information. |
|  | _networkAssistance | Network Assistance configuration. |
|  | _policyTemplate | Policy Template configuration. |
|  | _consumptionReporting | Consumption reporting configuration. |
|  | _metricsReporting | Metrics reporting configuration. |
| _status[mediaDeliverySessionId] |  | The Media Session Handler maintains a separate status record for each currently active media delivery session, indexed by media delivery session identifier. |
|  | _generalStatus | General status information. (See table 10.2.3-1.) |
|  | _dynamicPolicyStatus | Dynamic Policy status information. (See table 10.3.2-1) |
|  | _networkAssistanceStatus | Network Assistance status information. (See table 10.4.2-1) |
|  | _consumptionReportingStatus | Consumption Reporting status information. (See table 10.5.2-1.) |
|  | _metricsReportingStatus | Metrics Reporting status information. (See table 10.6.2-1.) |

The above information is accessible through reference point M6.

### 10.2.2	General Media Session Handler methods

#### 10.2.2.1	Request to update Service Access Information

### This method at reference point M6 is used to request the Media Session Handler to request the latest Service Access Information from the Media AF.

#### 10.2.2.2	Retrieve media access information

This method is used for retrieving the provisioning session type and streaming access information at the reference point M6.

#### 10.2.2.3

#### Start media delivery session

There are different ways to start a Media Delivery Session. One typical one is that the start is bound to the call by the application directly or through Media Player/Streamer with an entry point obtained from Service Access Information. That start method offers a client–server like interface realized by M6. The service is bound such that the Media Session Handler communicates back to the Application or Media Player/Streamer/… with the status of the Media Delivery Session.

The alternative way is to pass the Service URL to the Media Session Handler through reference point M6.

#### 10.2.2.4	Stop media delivery session

Similarly to start, a Media Delivery Session can be stopped through reference point M6.

#### 10.2.2.5	Tear down media delivery session

This method is used to release the allocated resources and the Media Delivery Session identifier through reference point M6.

#### 10.2.2.3	Stop media delivery session

### 10.2.3	General Media Session Handler information

Table 10.2.3-1 specifies the status information that can be obtained from the Media Session Handler through reference point M6.

Table 10.2.3-1: General Media Session Handler Status Information

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| SESSION_HANDLING_STATUS | Enumerated: STARTED STOPPED ERRORED TEARED-DOWN | Media delivery session identifier | The status of media delivery session: STARTED: The Media Delivery Session is assigned, and media is being delivered. STOPPED: The Media Delivery Session is assigned, but the media is not being delivered at this time. ERRORED: There is an error in media session handling. TEARED-DOWN: The Media Delivery Session was teared-down, and resources were released. |

Table 10.2.3-2 provides a list of general notification events exposed at reference point M6.

Table 10.2.3-2: General Media Session Handler Notification Events

| Event | Definition | Payload |
| --- | --- | --- |
| SESSION_HANDLING_STARTED |  | Triggered when media is being delivered. . | Media delivery session identifier, externalServiceId. |
| SESSION_HANDLING_STOPPED |  | Triggered when media session handling stopped for a specific Media Entry Point. | Media delivery session identifier, externalServiceId. |
| SESSION_HANDLING_TEARED-DOWN |  | Triggered when media session handling was activated for a specific Media Entry Point. | Media delivery session identifier, externalServiceId. |

Table 10.3.3-3 provides a list of general error events exposed at reference point M6.

Table 10.2.3-3: General Media Session Handler Error Events

| Status | Definition | Payload |
| --- | --- | --- |
| ERROR_SESSION_HANDLING | Triggered when there is an error in the media session handling. | Media delivery session identifier. |