3GPP TSG-SA4 Meeting #127-bis-e	S4-240700

# Online, , 8th Apr 2024 - 12th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.511 | CR | 0012 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding IVAS codec support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Sweden AB, Ericsson LM, Fraunhofer IIS, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of IVAS to 5GMS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of IVAS Decoding and Generation Capabilities based on the IVAS Operation Point in 26.117 Recommendation for IVAS for - Downlink Streaming Default Profile - Uplink Streaming Default Profile - Downlink 360 VR profile - Uplink VR 360 Profile Optional upport for IVAS for - Television Profile |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for IVAS in 5GMS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.2.3/5.2.4, 5.2.7.3/5.2.7.4, 5.2.8.3, 5.3.3/5.3.4, 5.3.5.3/5.3.5.4, 5.4.2, 5.5.2, 5.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GMS	5G Media Streaming

5GMSA	5G Media Streaming Architecture

AMR	Adaptive Multi Rate

AMR-WB	Adaptive Multi Rate – Wide Band

API	Application Programming Interface

AS	Application Server

AVC	Advanced Video Coding

AVC-HD	Advanced Video Codec – High Definition

CMAF	Common Media Application Format

DASH	Dynamic Adaptive Streaming over HTTP

EVS	Enhanced Voice Services

HD	High Definition

HDR	High Dynamic Range

HD-HDR	High Definition and High Dynamic Range

HEVC	High Efficiency Video Coding

HLG	Hybrid Log-Gamma

HLS	HTTP Live Streaming

IMSC	Internet Media Subtitles and Captions

IVAS	Immersive Voice and Audio Services

MPEG	Moving Picture Experts Group

OMAF	Omnidirectional Media Application Format

PSS	Packet-switched Streaming Service

TTML	Timed Text Markup Language

TV	Television

UE	User Equipment

UHD	Ultra High Definition

VR	Virtual Reality

VCL	Video Coding Layer

* * * Next Change * * * *

### 5.2.3	Audio decoding

If the 5GMSd Client supports the reception of audio:

-	the IVAS decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	xHE-AAC stereo decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

NOTE:	xHE-AAC® is a registered trademark of Fraunhofer in Germany and other countries and is used with Fraunhofer’s permission.

-	eAAC+ decoding capability shall be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	AMR-WB+ decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

### 5.2.4	Speech decoding

If the 5GMSd Client supports the reception of speech:

-	the IVAS decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	the EVS decoding capability shall be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	the AMR-WB decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

-	the AMR decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.2.

* * * Next Change * * * *

#### 5.2.7.3	Speech media profiles

If the 5GMSd Client supports the reception of speech, then the following shall be supported:

-	the EVS playback requirements as defined in 3GPP TS 26.117 [4] clause 7.4.2.4.

If the 5GMSd Client supports the reception of speech, then the following should be supported:

-	the IVAS playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

-	the AMR-WB playback requirements as defined in 3GPP TS 26.117 [4] clause 7.3.2.4.

If the 5GMSd Client supports the reception of speech, then the following may be supported:

-	the AMR playback requirements as defined in 3GPP TS 26.117 [4] clause 7.2.2.4.

#### 5.2.7.4	Audio media profiles

If the 5GMSd Client supports the reception of audio, then the following should be supported:

-	the IVAS playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.8.4.

If the 5GMSd Client supports the reception of audio, then the following shall be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.4.

If the 5GMSd Client supports the reception of audio, then the following may be supported:

-	the AMR-WB+ playback requirements as defined in 3GPP TS 26.117 [4] clause 7.7.2.4.

* * * Next Change * * * *

#### 5.2.8.3	Audio media profiles

For AMR:

-	the <profiles> parameter is defined in TS 26.117, clause 7.2.2.4 as audio/mp4 profiles='camr'

-	the <codecss> parameter is defined in TS 26.117, clause 7.2.2.4 as 'samr'

For AMR-WB:

-	the <profiles> parameter is defined in TS 26.117, clause 7.3.2.4 as audio/mp4 profiles='camw'

-	the <codecs> parameter is defined in TS 26.117, clause 7.3.2.4 as 'samw'

For EVS:

-	the <profiles> parameter is defined in TS 26.117, clause 7.4.2.4 as audio/mp4 profiles='cevs'

-	the <codecs> parameter is defined in TS 26.117, clause 7.4.2.4 as 'sevs'

For IVAS:

-	the <profiles> parameter is defined in TS 26.117, clause 7.5.2.4 as audio/mp4 profiles='civs'

-	the <codecs> parameter is defined in TS 26.117, clause 7.5.2.4 as 'sivs'

* * * Next Change * * * *

### 5.3.3	Audio encoding

If the 5GMS UE supports the transmission of audio:

-	the sender requirements for the eAAC+ stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.2.3 shall be supported.

-	the sender requirements for the IVAS Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.5.3 should be supported;

- 	the sender requirements for the xHE-AAC stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.4.3 should be supported.

### 5.3.4	Speech encoding

If the 5GMS UE supports the transmission of speech:

-	the sender requirements for the IVAS Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.5.3 should be supported;

-	the sender requirements for the EVS Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.4.3 shall be supported;

-	the sender requirements for the AMR-WB Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.3.3 should be supported;

-	the sender requirements for the AMR Operation Point as defined in 3GPP TS 26.117 [4] clause 6.2.2.3 may be supported.

* * * Next Change * * * *

#### 5.3.5.3	Speech media profile

If the 5GMSu Client supports the transmission of speech, then the following shall be supported:

-	the EVS content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.4.2.5.

If the 5GMSu Client supports the transmission of speech, then the following should be supported:

-	the IVAS content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.5.

-	the AMR-WB content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.3.2.5.

If the 5GMSu Client supports the transmission of speech, then the following may be supported:

-	the AMR content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.2.2.5.

#### 5.3.5.4	Audio media profile

If the 5GMSu Client supports the transmission of audio, then the following shall be supported:

-	the eAAC+ stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.5.

If the 5GMSu Client supports the transmission of audio, then the following should be supported:

-	the IVAS content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.5.

-	the xHE-AAC stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.8.5.

* * * Next Change * * * *

### 5.4.2	Audio decoding

If the 5GMSd Client supports the Television (TV) profile, it shall support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.2.

If the 5GMSd Client supports the Television (TV) profile, it should support xHE-AAC stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.4.

If the 5GMSd Client supports the Television (TV) profile, it may support IVAS Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.5.

* * * Next Change * * * *

### 5.5.2	Audio

#### 5.5.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

-	the IVAS Operation Point Receiver requirements as defined in 3GPP TS 26. 117 [4], clause 6.3.5.2

#### 5.5.2.2	DASH encapsulation

If the 5GMS UE supports 3D/VR audio for DASH services, it should include a receiver that complies with:

-	the OMAF 3D Audio Baseline Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.118 [13], clause 6.2.2.3.

-	the CMAF IVAS Media Profile Receiver requirements for file format signalling and encapsulation as specified in TS 26.117 [4], clause 7.5.2.4.

* * * Next Change * * * *

### 5.6.2	Audio

#### 5.6.2.1	Operation Points

If the 5GMS UE supports 3D/VR audio, it should include an encoder that can encode a bitstream that can be decoded by a receiver that complies with:

-	the 3GPP MPEG-H Audio Operation Point Receiver requirements as specified in TS 26.118 [13], clause 6.1.4.

-	the IVAS Operation Point Receiver requirements as defined in 3GPP TS 26. 117 [4], clause 6.3.5.2

#### 5.6.2.2	Encapsulation

Media Encapsulation in 5G Media Streaming for uplink is defined based on the MPEG Common Media Application Format (CMAF) [7]. A sender shall at least support the generation of CMAF tracks.

* * * End of Changes * * * *