3GPP TSG-SA WG4 Meeting #127bis-e	S4-240710

# Online, 8-12 April 2024

Source:	Dolby Sweden AB, Fraunhofer IIS, Ericsson LM, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Title:	Pseudo-CR on Adding ISAR track-a split rendering feature

Spec:	3GPP TS 26.250

Agenda item:	7.7 - ISAR (Immersive Audio for Split Rendering Scenarios)

Document for:	Endorsement

1. Introduction

This Pseudo-CR relates to the selection of an ISAR split rendering solution of the IVAS codec standard targeted in Phase/Track 2/a of the ISAR work.

2. Reason for Change

The split rendering feature enabled with this Pseudo-CR was selected by SA4 according to the ISAR selection procedure.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 26.250.

* * * First Change * * * *

# 5	Transcoding functions

An algorithmic description of the IVAS codec is provided in [3].

As shown in Figures 1 & 2, the audio encoder takes its input, and can produce an output at the decoder/renderer, in various audio output formats. Input and output audio signals consist of one or multiple constituent channels of the respective audio format and in some cases metadata. The constituent channels are in the form of 16-bit uniform Pulse Code Modulated (PCM) signals at sampling frequencies of 8 kHz (only EVS interoperable coding), 16 kHz, 32 kHz or 48 kHz. The audio may typically originate from and terminate within the audio part of the UE or from the network side.

The detailed mapping between blocks of input audio to encoded blocks (in which the number of bits depends on the presently used codec mode) and from these to output blocks of reconstructed audio is described in [3]. The supported bit rates of the EVS interoperable coding are provided in [2]. Stereo and immersive audio coding is offered at the following discrete bit rates [kbps]: 13.2, 16.4, 24.4, 32, 48, 64, 80, 128, 160, 192, 256, 384, and 512, with supported bit rate ranges and supported source-controlled rate operation (DTX) listed in Table 1.

Table 1: Ranges of supported source codec bit rates for stereo and immersive audio coding modes of the IVAS codec

| Input audio format | Range of supported bit rates (kbps) | Source Controlled Rate Operation Available |
| --- | --- | --- |
| Stereo, binaural audio(1 | 13.2 – 256 | Yes, up to 256 kbps |
| Scene-based audio (Ambisonics: FOA, HOA2, HOA3) | 13.2 – 512 | Yes, up to 80 kbps |
| Metadata-assisted spatial audio (MASA) | 13.2 – 512 | Yes, up to 512 kbps |
| Object-based audio (ISM)(2 | 13.2 – 512(3 | Yes, up to 512 kbps |
| Multi-channel audio | 13.2 – 512 | No |

Notes:

(1 A head-trackable binaural audio format at rates ranging from 256 kbps to 768 kbps is additionally supported as intermediate split rendering representation.

(2 Combined input audio format combining scene-based audio with ISM (OSBA) is supported. Metadata-assisted spatial audio with ISM (OMASA) is supported.

(3 13.2 kbps – 128 kbps for 1 ISM, 16.4 kbps – 256 kbps for 2 ISMs, 24.4 kbps – 384 kbps for 3 ISMs, resp. 24.4 kbps – 512 kbps for 4 ISMs.

For stereo input, a downmix tool is provided to generate a mono signal for EVS interoperable stream without additional delay.

There are 3 functionality levels of IVAS codec operations (including rendering) with increasing complexity/memory requirements.

An IVAS codec implementation of functionality level 1 shall support […]

An IVAS codec implementation of functionality level 2 shall support level 1 functionality and in addition […]

An IVAS codec implementation of functionality level 3 shall support the full set of IVAS codec features and functionalities, i.e., level 2 functionality and further […].

An IVAS audio codec capable UE shall support at least functionality level 1.

* * * Next Change * * * *

# 6	Rendering

IVAS rendering is the process to generate the IVAS audio output in the same or a different audio format than the input format, whereby in some cases, such as stereo-to-stereo, there is no particular rendering processing other than the decoding. The IVAS decoder provides integrated binaural rendering functionality for headphone reproduction including head-tracking and scene orientation control and integrated rendering for loudspeaker reproduction. IVAS binaural rendering also supports a reverberation effect. There is also the possibility to feed the IVAS decoder output to a customized external renderer while bypassing the integrated renderer. A special feature of the renderer is that it supports split operation with pre-rendering and transcoding to a head-trackable intermediate representation that can be transmitted to a post-rendering end-device. This enables moving a large part of the processing load and memory requirements for IVAS decoding and rendering to a (more) capable node/UE while offloading the final rendering end-device.

IVAS rendering can also be operated stand-alone, i.e., without prior IVAS encoding/decoding of the input audio signal.

IVAS rendering is described in [3] and [4].

* * * End of Changes * * * *