Source:	Dolby Sweden AB, Fraunhofer IIS, Ericsson LM, Huawei Technologies Co Ltd., Nokia Corporation, NTT, Orange, Panasonic Holdings Corporation, Philips International B.V., Qualcomm Incorporated, VoiceAge Corporation

Title:	Report Covering the Compliance to Design Constraints for Split Rendering Candidate Solution for IVAS Codec

Document for:	Discussion and Agreement

Agenda Item:	7.7

Introduction

This contribution reports on how the split rendering candidate solution for IVAS Codec by the IVAS Codec Public Collaboration complies with the design constraints defined for ISAR phase 1/track a solutions. The design constraints are specified in TR 26.865 clauses 6 (Physical Design Constraints) and 7 (Functional Design Constraints). Reference is made to these.

lists the provided selection deliverables associated with the selection of the split rendering candidate solution for IVAS Codec by the IVAS Codec PC.

CMD lines used for complexity and memory measurements

This section provides the command lines of the IVAS/ISAR candidate solution executables used for the complexity and memory measurements. The command lines also show the used test vectors, defined under the IVAS codec work. The measurements were done by using the ITU-T WMC Tool also used for IVAS codec complexity and memory measurements by the IVAS codec PC.

|  | HOA3 + LCLD | MC714 + LC3plus | ISM + LC3plus | MASA + LCLD |
| --- | --- | --- | --- | --- |
| IVAS ENC | ./IVAS_cod -sba 3 512000 48 ../testv/stv3OA48c.wav enc_out.pkt | ./IVAS_cod -mc 7_1_4 512000 48 ../testv/stv714MC48c.wav enc_out_mc.pkt | ./IVAS_cod -ism 4 .. /testv/stvISM1.csv .. /testv/stvISM2.csv .. /testv/stvISM3.csv .. /testv/stvISM4.csv 512000 48 .. /testv/stv4ISM48s.wav enc_out_ism4.pkt | ./IVAS_cod -masa 2 .. /testv/stv2MASA2TC48c.met 512000 48 .. /testv/stv2MASA2TC48c.wav enc_out_masa.pkt |
| IVAS DEC | ./IVAS_dec -render_config rc.txt -t ../trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out.pkt split_out_sba_lcld_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config rc.txt -t ../trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_mc.pkt split_out_mc_lc3plus_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config ./Rc.txt -t .. /trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_ism4.pkt split_out_ism4_lc3plus_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; | ./IVAS_dec -render_config ./Rc.txt -t .. /trajectories/full_circle_in_15s_delayed.csv BINAURAL_SPLIT_CODED 48 enc_out_masa.pkt split_out_masa_lcld_20ms.pkt Rc.txt: [SPLITREND] BITRATE = 768000; DOF = 3; HQMODE = 1; |
| ISAR POST REND | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_sba_lcld_20ms.pkt -o split_out_sba_lcld_20ms_out.wav -fs 48 -t ../trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_mc_lc3plus_20ms.pkt -o split_out_mc_lc3plus_20ms_out.wav -fs 48 -t ../trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_ism4_lc3plus_20ms.pkt -o split_out_ism4_lc3plus_20ms_out.wav -fs 48 -t .. /trajectories/full_circle_in_15s.csv | ./ISAR_post_rend -if BINAURAL_SPLIT_CODED -i split_out_masa_lcld_20ms.pkt -o split_out_masa_lcld_20ms_out.wav -fs 48 -t .. /trajectories/full_circle_in_15s.csv |

Physical design constraints applicable for IVAS split rendering scenarios

The following physical design constraints apply for split rendering solutions involving the IVAS codec:

| Physical attribute | Constraint | Values based on “isar_selection_branch” |
| --- | --- | --- |
| Complexity of operation in end-rendering lightweight device | The complexity of operation in end-rendering lightweight device shall not exceed 80+(DOF*20) wMOPS. 0DOF: 80 1DOF: 100 2DOF: 120 3DOF: 140 | 3DOF (with LC3plus): max 73.66, avg 70.7 3DOF (with LCLD): max 60.96, avg 51.41 |
| Complexity of operation at capable device/node | The complexity of operation at pre-rendering device/node shall be characterized. | IVAS decoder + 3DOF split pre-rendering with LC3plus (MC714): max 1063.97, avg 1037.15 IVAS decoder + 3DOF split pre-rendering with LC3plus (ISM4): max 864.88, avg 854.2 IVAS decoder + 3DOF split pre-rendering with LCLD (HOA3): max 925.5, avg 837.0 IVAS decoder + 3DOF split pre-rendering with LCLD (MASA2): max 662.12, avg 653.06 |
| Memory footprint of operation in end-rendering lightweight device | The RAM consumption shall not exceed 100+(DOF*50) kWords. 0DOF: 100 kWords 1DOF: 150 kWords 2DOF: 200 kWords 3DOF: 250 kWords The ROM (PROM and table ROM) shall not exceed 150 kWords. | (word = 4 Bytes) RAM (heap + stack): 3DOF (with LC3plus): 67.51 kWords 3DOF (with LCLD): 67.87 kWords PROM (lc3plus + lib_isar + lcld): 20.04 kWords TROM (lc3plus + lib_isar + lcld): 42.14 kWords |
| Memory footprint of operation at pre-rendering device/node | The memory footprint of operation at pre-rendering device/node shall be characterized. | RAM: IVAS decoder + 3DOF split pre-rendering with LC3plus (MC714): 586.751 kWords IVAS decoder + 3DOF split pre-rendering with LC3plus (ISM4): 393.24 kWords IVAS decoder + 3DOF split pre-rendering with LCLD (HOA3): 467.63 kWords IVAS decoder + 3DOF split pre-rendering with LCLD (MASA2): 290.39 kWords PROM (lib_com + lib_dec + lib_rend):172.6 kWords TROM (lib_com + lib_dec + lib_rend):156.4 kWords |
| Algorithmic motion-to-sound latency in head-tracked rendering operation | 0-DOF: no constraint 1-DOF: 30 ms for rotations around corrected axis (in post rendering), for other axes no constraints 2-DOF: 30 ms for rotations around corrected axes (in post rendering), for the remaining axis no constraint 3-DOF: 30 ms for rotations around all axes (in post rendering) | 2.5ms <= Algorithmic motion-to-sound latency <= 22.5 ms |
| Algorithmic audio delay | The total algorithmic end-to-end audio delay including IVAS algorithmic delay shall not exceed 50 ms. | LC3plus; LCLD; DOF. SBA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. MC714@512kbps; 32+5+2.5 = 39.5ms; 32+5 = 37ms; 3. ISM@512kbps; 32+5+2.5 = 39.5ms; 32+5 = 37ms; 3. MASA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. OMASA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. OSBA@512kbps; 38+5+2.5 = 45.5ms; 38ms; 3. LC3plus; LCLD; DOF. SBA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. MC714@512kbps; 32+2.5 = 34.5ms; 32+5 = 37ms; 0. ISM@512kbps; 32+2.5 = 34.5ms; 32+5 = 37ms; 0. MASA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. OMASA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. OSBA@512kbps; 38+2.5 = 40.5ms; 38ms; 0. NOTE: The additional delay for 1-3DOF on top of IVAS is 5ms for pose correction with LCLD coding, 7.5ms (5+2.5ms) for pose correction and LC3plus coding. The additional delay on top of IVAS for 0-DOF operation is 2.5ms for LC3plus coding and 0 or, respectively, 5ms for LCLD coding. In case of pre-rendering in CLDFB domain additional delay by LCLD coding can be circumvented. |
| Bit rate of coded intermediate representation | The Split Rendering solution should offer 3-DOF operation at multiple bit rates with rate switching support and shall at least offer operation at 768 kbps. For 3-DOF operation, 384 kbps and 512 kbps should be offered in addition to the required 768 kbps. For 1-DOF operation, 256 kbps should be offered. For 0-DOF operation, 256 kbps and even lower bit rates should be offered. For cases with high transmission channel capacities, high bit rate operation modes > 768 kbps may be offered if QoE gains can be demonstrated. | DOF; Bitrates supported (kbps). 0; 256, 384, 512. 1; 384, 512, 768. 2; 384, 512, 768. 3; 384, 512, 768. |

Functional design constraints applicable for IVAS split rendering scenarios

The following functional design constraints apply for split rendering solutions involving the IVAS codec:

| Functional attribute | Constraint |  |
| --- | --- | --- |
| Immersive audio formats of native coding format | All required immersive IVAS encoder input formats according to Pdoc IVAS-4 shall be supported. IVAS stereo input format should be supported for 0-DOF operation. | SBA: Done MC: Done ISM: Done MASA: Done OSBA: Done OMASA: Done Stereo: Not done |
| Bit rates of required immersive audio coding modes of native coding format | All required bit rates of IVAS immersive operation modes according to Pdoc IVAS-4 shall be supported. All IVAS stereo bit rates should be supported for 0-DOF operation. | All bitrates supported for immersive formats Stereo: Not done |
| Head-trackability of immersive audio formats | The head-trackability of the immersive audio formats of the native coding format shall be retained. Preservation of the DOF level is the objective, reduced DOF levels may be provided. Note: an audio format supported by the native coding system may be 3-DOF head-trackable, i.e., around 3 axes (yaw, pitch, roll). The Split Rendering system should retain this possibility. Complexity or bit rate reduced variants may though reduce this to lower DOF levels like yaw-only correction (1-DOF). | Done |
| Packet loss concealment (PLC) | A PLC solution shall be provided. | Done |
| Non-diegetic audio support | The solution shall support (1DOF – 3DOF) diegetic audio and (0-DOF) one-channel non-diegetic audio and two-channel (stereo or binaural) non-diegetic audio. It shall be possible to overlay post rendered audio obtained from instances operated with diegetic and non-diegetic audio. | Done |

Conclusion

The comparative evaluation of the properties of the split rendering candidate solution for IVAS Codec by the IVAS Codec Public Collaboration shows that all design constraints defined in TR 26.865 are met.

- References

[1]	3GPP TR 26.865: Immersive Audio for Split Rendering Scenarios; Requirements