3GPP TSG-SA WG4 Meeting #127e	S4-240714

eMeeting,  8th - 12th  April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.439 | pCR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Energy consumption collection at OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaGREEN |  |  |  |  |  | Date: |  |  | 29-03-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | One of the objectives of S4-240468 states the following: “Document existing APIs, metrics and mechanisms inside or outside 3GPP that could be used for energy measurement, reporting and exposure of media services. This includes whether and what information is exposed, how it is exposed, and at what granularity (QoE measurement and reporting, QoS measurement and reporting, audience measurement and reporting, event exposure, CMCD reporting, etc.).” In this context, it is proposed to add the proposed content to the latest draft of TR 26.439 v 1.0.0 so that it is not left incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.439 on “Energy consumption collection at OAM” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

X.X. Energy consumption collection at OAM

# X.1	General

This section provides a brief reference to SA WG5 and ETSI specification related to Energy consumption evaluation and collection at OAM specified in Rel-18.

## X.1.1	Collection of network energy related information

The specification TS 28.554 [1] defines the Energy Consumption KPI of a Physical Node in clause 6.7.3. The network energy related information that can be collected by OAM includes the information in Table X.1.1-1.

Table X.1.1-1: Network energy related information collected by OAM

| KPI category | Description | Reference in TS 28.554 [6] |
| --- | --- | --- |
| Energy consumption | Energy Consumption of a gNB | 6.7.3.4.2 |
| related information | Energy Consumption of the NG-RAN | 6.7.3.4.1 |
| reflected by Energy | Energy Consumption of the 5GC | 6.7.3.2.1 |
| Consumption (EC) | Energy Consumption of a 5G Network Function | 6.7.3.1.1 |
|  | Estimated Energy Consumption of a Virtualized Network Function | 6.7.3.1.2 |
|  | Energy Consumption of a Physical Network Function (PNF) as well as other Power, Energy, Environmental (PEE) measurements | Clause 5.1.1.19.2 of TS 28.552 [5] |
|  | Energy Consumption of a network slice | 6.7.3.3 |
| Energy efficiency | Energy Efficiency of the NG-RAN data | 6.7.1 |
| related information | Energy Efficiency of the 5GC | 6.7.4.1 |
| reflected by Energy Efficiency KPIs | Energy Efficiency of a network slice | 6.7.2 |

In case of Virtualized NF (VNF) hosted on a physical node, the EC of the VNF is estimated as a portion of the EC of the physical node, based on its relative virtual CPU usage, virtual memory usage, virtual disk usage and I/O traffic (all metrics collected from ETSI MANO) as defined in clause 6.3.1.2 of TS 28.554 [1].

## X.1.2	Exposure of network energy related information

Network energy relation information may be collected by OAM and exposed (other mechanisms may exist) as defined by Service-Based Management Architecture (SBMA). TS 28.533 [3] defines the Service Based Management Architecture (SBMA), where any authorized consumer willing to collect such measurements / KPIs must create an instance of a performance metric production job (i.e. an instance of the PerfMetricJob information object class - see clause 4.3.31 of TS 28.622 [4]) by invoking the createMOI operation of the Provisioning Management Service (MnS) (see clause 11.1.1.1 of TS 28.532 [2]). The consumer must specify:

-	Which measurement(s) / KPI(s) are expected to be collected, such as e.g. the Energy Consumption of a 5G NF, etc.;

-	On which network entities (represented by managed objects) such as e.g. a SMF x, UPF y, etc.;

-	Which granularity period is expected to produce measurements / KPIs (in seconds);

-	Which reporting method is expected, mainly:

-	file-based: in this method the performance data is accumulated for certain time before it is reported; the data is delivered as a file; the file content encoding technique can be either XML or ASN.1,

-	stream-based: in this method, the producer, when the performance data are ready, sends the performance data to the consumer. The volume of the performance data reported by streaming is expected to be small, and the granularity period of the performance data stream and is expected to be short. The stream content encoding technique can be either GPB or ASN.1.

Depending on the selected reporting method, the consumer collects the measurements / KPIs as follows:

-	In the case of file-based reporting, the producer sends the notification NotifyFileReady to subscribed consumer(s) when a new file becomes available on the producer for upload by consumer(s);

-	In the case of stream-based reporting, the producer sends units of streaming data to the consumer by using the reportStreamData operation.

## End of change

## 2nd Change

X.X. References

[1]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[2]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[5] 	3GPP TS 28.552: “Management and orchestration; 5G performance measurements”.

## End of change