Agenda item: 	10.8

Source: 	Qualcomm Inc.

Title: 	[FS_5G_RTP_Ph2] A generic solution for Key Issue #1: Inaccuracy of PDU Set Size information

Document for	Discussion and Agreement

# Introduction

In the 5G RTP phase 2 SID [6], Key issue #1 is:

- Inaccuracy of the PDU Set Size (PSSize) information. TS 26.522 defines the PDU Set marking RTP header extension, which allows the traffic characteristics known at the RTP traffic source to be exposed to the network to allow for cross-layer design (e.g., assisting network resource allocation at the RAN). Rel-18 WI identified the issue of the inaccuracy of the PDU Set Size (PSSize) caused by network operations such as NAT46/64, TURN, IP segmentation, and segmented routing. The issue may cause inefficacy or degraded user experience but did not get enough time for a thorough study. The issue needs to be further studied in Rel-19.

[In this paper, we discuss Key issue #1, providing justification and a potential solution.

The PDU Set Size (PSSize) is calculated at the RTP packet source, including the RTP/UDP/IP packet headers, and is indicated in the “PDU Set marking” RTP header extension. The PSSize seen by the UPF may be different from the indicated PSSize value in the RTP header extension due to various reasons, including, but not limited to:

- NAT64 or NAT46, as noted in [1]

- IP fragmentation, where each increment in the number of IP packets adds an additional size worth of an IP packet header to the PSSize

- TURN, where the TRUN server may add a STUN message header, a STUN attribute, and transport address, as noted in [2].

- Segment Routing, where an ingress router adds to the IP packet a list of segment identifiers for the segments in the Segment Routing domain [3]. This has an impact on RAN only if UE is part of the Segment Routing domain.

These reasons may individually or jointly affect the accuracy of the PSSize, for example, both NAT46 and IP fragmentation may happen to the IP packets of a PDU Set.]

# The importance of accuracy in PSSize

Although the impact of those reasons on the size of a single IP packet seems insignificant, a PDU Set may consist of many IP packets and the aggregate impact can still be significant. If the PSSize for which gNB schedules is less than the actual PSSize, when the last packets arrive it may take gNB one or more slots to schedule them, therefore delaying the delivery of the PDU Set, which is detrimental to low-latency applications such as XR applications. The opposite can also happen, which wastes resources.

Therefore, we have the following observation:

Observation 1: it is important to make the PSSize accurate for low-latency applications.

# Proposed solution

There are efforts to correct the impact of NAT46/64 on the PSSize [3][4]. However, these efforts are not able to tackle other causes. Moreover, the list of causes in the introduction clause is not complete – even if we list all possible causes today, novel network protocols that change the PSSize are likely to be deployed in the future. We have the following observation:

Observation 2: A generic solution for correcting the PSSize is preferred.

The UPF is the gateway to the 5G core. A UPF may handle a very large amount of traffic. In fact, some network operators have very few UPFs. Therefore, any solution that requires UPF to take action, such as addition, subtraction and multiplication, is undesirable. We have the following observation:

Observation 3: To reduce the UPF complexity, it is preferred not to require UPF to correct the PSSize.

When we don’t know the causes, how do we correct the error? There is a similar problem in physical-layer communication, where the channel seen by a receiver is the result of reflection and refraction of many unknown objects in the radio propagation environment. The solution there is to measure the channel by the sender sending a pilot signal known to the receiver and the receiver comparing the pilot signal and the received signal. We borrow the measurement idea, and the counterpart of the ‘pilot signal’ is the indicated PSSize value in the RTP header extension and the counterpart of the ‘received signal’ is the observed PSSize. If a PDU Set is delivered successfully, the UE will observe the same PSSize (by summing the sizes of all PDUs of a PDU Set) as the gNB does.

Once the UE figures out the difference between the indicated PSSize and the observed PSSize, it can signal to the sender on how to pre-compensate for the PSSize.

Proposal 1: UE computes the difference between the actual PSSize and the indicated PSSize, and signals the difference to the RTP sender for PSSize pre-compensation.

Specifically, the UE calculates a correction ratio - the actual PSSize to the indicated PSSize ratio – and sends the correction ratio to the RTP sender. The RTP sender pre-compensates the PSSize by multiplying the PSSize and this correction ratio.

To validate the effectiveness of the proposed method, we tested the method on video sequences. The results below are for the Racehorse video sequence, and the video encoder is H.264 with a target NAL Unit size of 1400 bytes. The average video frame size is 22.377 kB. The cumulative distribution of the slice size (with each slice encapsulated into an IP packet) is shown in Figure 1. Note that there are a significant number of sizes uniformly distributed between 0 and around 1300 bytes.

NAT46 occurs in the network.

Figure 1 Slice size distribution

Scenario 1 (MTU=576 bytes):

The MTU size is set to 576 bytes (considered as a ‘safe’ MTU, because it is the IP packet size that allIPV4 nodes need to support [RFC791]). This leads to fragmenting a packet size of 1400 bytes into three IP packets. As a result, the error in the PSSize comes from two sources: the presence of NAT46 and IP fragmentation. For the measurement-based correction method, the correction ratio is initialized to 1.

NOTE: The network configuration, e.g., NAT46 and MTU, typically changes much slower than the time scale of the feedback delay which is on the order of RTT. Thus, the feedback mechanism does not cause instability.

NOTE: As needed feedback - The UE can adapt the rate of feedback based on the observed error in the PSSize and a threshold on the tolerance of error.

The correction ratio (blue line) is shown in Figure 2.

Figure 2 Correction for the per-frame feedback (blue line) and one-time feedback (red line).

The various PSSize’s are are shown in Figure 3. The errors in the PSSize are shown in Figure 4. Without PSSize correction, the mean absolute error of the PSSize is 1616 bytes; with PSSize correction assuming NAT46 only, the mean absolute error is 1283 bytes; with the proposed measurement based PSSize correction, the mean absolute error is 27.5 bytes.

Figure 3 The actual PSSize (cyan line) vs the PSSize without PSSize correction (blue line), the PSSize with measurement-based correction (red) and the PSSize with the ‘NAT46/64 only correction’ (green).

If the UE sends the feedback once during the transmission of the video sequence, without further correction ratio received, the sender will use a correction ratio of 1 for future frames. The correction ratio is shown by the red curve in Figure 2.The mean absolute error is 23.9 bytes, which is lower than the error when per-frame (or per PDU Set) feedback is used. This is because the first frame is an I-frame with a large size and the estimated ratio based on it tends to be more accurate than the estimates obtained in other smaller sized frames which are P frames.

Observation 4: For measurement-based PSSize correction, one-time feedback can be more accurate than more-frequent feedback.

Figure 4 PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback(red) for Scenario 1.

Scenario 2 (MTU=1300 bytes):

This is to show the effect of MTU size. The MTU size is set to 1300 bytes. A typical packet of 1400 bytes is fragmented into 2 packets. The errors in the PSSize are shown in Figure 5. The mean absolute error in the PSSize are 963.3 bytes, 630.0 bytes and 24.7 bytes for the case of no PSSize correction, NAT46 only correction and measurement-based correction, respectively.

With one-time feedback, the error is 20.7 bytes, again lower than the error of per-frame feedback 24.7 bytes.

We see that the respective errors decrease compared to smaller MTU size. However, the errors for case of no PSSize correction and the case of NAT46 only correction are still significant.

Figure 5 PSSize error for the cases of without correction (blue), with correction assuming NAT46 only (green), and with the proposed measurement-based correction with per-frame feedback(red) for Scenario 2.

From the results, we observe that:

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

For the ‘NAT46/64 only correction’ method to work, the sender needs to be aware of the presence of NAT46/64. The awareness may be obtained by feedback from the receive on the type of the IP address type seen by the receiver. The feedback may be one time during a session, and can be done by SDP signaling in a similar way in [5].

TS26.522 [1] provides guidelines for preventing IP fragmentation, either through path MTU discovery or by assuming a conservative MTU size at the sender in generating IP packets. Path MTU discovery needs support from the routers on the end-to-end path and incurs communication overhead, and a conservative MTU size may lead to unnecessarily small IP packet sizes which come with a higher packet header cost (i.e., the ratio of the size of packet headers to the size of the media). We consider the guidelines as a solution for IP fragmentation prevention.

We compare the three solutions. The first criteria is whether the solution is generic, i.e., whether the solution can tackle multiple and even unknown causes to the error in the PSSize. The criteria ‘need support from the network?’ means whether the network needs to be configured (e.g., configured to support path MTU discovery) to enable a solution.

Observation 6: Pros and Cons of the three solutions

| Solution | Generic | Accuracy | Need support from the network? | Communication overhead? | Need spec change? |
| --- | --- | --- | --- | --- | --- |
| NAT46/64 only correction | No | Low | No | Low | Yes |
| IP fragmentation prevention guidelines | No | Low | Yes if use path MTU discovery; no if use conservative MTU size. | Moderate if use path MTU discovery; none if use conservative MTU size. | No |
| Measurement-based correction | Yes | High | No | From low (one-time feedback) to high (per PDU Set feedback) | Yes |

# Proposals

We propose to add clause 1-3 to the 5G RTP phase 2 TR 26.8xx.

Additionally, the observations and proposals in clause 1-3 are summarized below.

Observation 1: it is important to make the PSSize accurate for low-latency applications.

Observation 2: A generic solution for correcting the PSSize is preferred.

Observation 3: To reduce the UPF complexity, it is preferred not to require UPF to correct the PSSize.

Proposal 1: UE computes the difference between the actual PSSize and the indicated PSSize, and signals the difference to the RTP sender for PSSize pre-compensation.

Observation 4: For measurement-based PSSize correction, one-time feedback can be more accurate than more-frequent feedback.

Observation 5: The proposed pre-compensation based PSSize correction method effectively reduces the PSSize error.

Observation 6: Pros and Cons of the three solutions

| Solution | Generic | Accuracy | Need support from the network? | Communication overhead? | Need spec change? |
| --- | --- | --- | --- | --- | --- |
| NAT46/64 only correction | No | Low | No | Low | Yes |
| IP fragmentation prevention guidelines | No | Low | Yes if use path MTU discovery; no if use conservative MTU size. | Moderate if use path MTU discovery; none if use conservative MTU size. | No |
| Measurement-based correction | Yes | High | No | From low (one-time feedback) to high (per PDU Set feedback) | Yes |

# References

[1] 3GPP TS 26.522 V1.0.0, March 2024.

[2] S4aR230110, “PDU Set identification with TURN data delivery”, 3GPP SA4 post #125 RTC Adhoc Call, Oct 25, 2023.

[3] RFC 8402, “Segment Routing Architecture”, July 2018.

[4] S4-231305, “On Origin IP Version and PDU Set Size”, 3GPP SA4 #126, August 21, 2023.

[5] S4-231809, “On the Mismatch of IP Address Types and PDU Set Size Adjustment”, 3GPP SA4 #126, Nov 13, 2023.

[6] SP-240482, New SID on 5G Real-time Transport Protocol Configurations, Phase 2, March 2024.