3GPP SA4#127-bis-e	S4-240723

Online, 08-12 Apr 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | pseudo | rev |  | Current version: | 1.1.4 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding edge resource management to M6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Updating clause 10, interface at reference point M6 to include edge resource management |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 10.2.1: adding two enteries to the table - 10.4a (new) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete spec: while clause 8 talks about edge provisioning, there is no mention of edge resource management by the application in clause 10. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 10	UE media session handling APIs

## 10.1	Introduction

This clause defines the client APIs exposed by the Media Session Handler to the Media-aware Application at reference point M6 and to the Media Access Function at reference point M11.

NOTE:	Client-driven management of edge processing resources via reference point M6 is not specified in this release.

## 10.2	Media Session Handler client API

### 10.2.1	Media Session Handler internal properties

The Media Session Handler maintains internal properties as defined table 10.2.1-1. Note that the parameters are conceptual and internal. They serve only for the purpose of defining the media session handling APIs.

Table 10.2.1-1: Parameters of Media Session Handler

| States and Parameters |  | Definition |
| --- | --- | --- |
| _Configuration[externalServiceId] |  | The Media Session Handler maintains a separate configuration for each set of Service Access Information it has knowledge of, indexed by its external service identifier. |
|  | _networkAssistance | Network Assistance configuration. |
|  | _policyTemplate | Policy Template configuration. |
|  | _edgeResources | Edge Resources configuration. |
|  | _consumptionReporting | Consumption reporting configuration. |
|  | _metricsReporting | Metrics reporting configuration. |
| _status[mediaDeliverySessionId] |  | The Media Session Handler maintains a separate status record for each currently active media delivery session, indexed by media delivery session identifier. |
|  | _generalStatus | General status information. (See table 10.2.3-1.) |
|  | _dynamicPolicyStatus | Dynamic Policy status information. (See table 10.3.2-1) |
|  | _networkAssistanceStatus | Network Assistance status information. (See table 10.4.2-1) |
|  | _edgeResourcesStatus | Edge Resources status information. (See table 10.4a.2-1.) |
|  | _consumptionReportingStatus | Consumption Reporting status information. (See table 10.5.2-1.) |
|  | _metricsReportingStatus | Metrics Reporting status information. (See table 10.6.2-1.) |

2nd Change

### 10.4a	Edge Processing

### 10.4a.1	Edge Resources methods

a Media-aware Application may request the allocation of edge resources by a request through reference point EDGE-5/M6. This method is usually deployed if the EDGE_ELEGIBILITY (clause 4.1.2) is true. Then the application knows that it is eligible to use the resources and then it can request the edge resources to be allocated and instantiated.

Similarly, the Application can request terminating an edge resource by sending a request through reference point EDGE-5/M6 if the UE supports EDGE-5.

### 10.4a.2	Edge Resources information

Table 10.4a2-1 specifies the status information that can be obtained from the Media Session Handler through reference point M6/EDGE-5 if the UE supports EDGE-5.

Table 10.4a.2-1: Status Information relating to Edge Resources

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |
| EDGE_ELEGIBILITY | Boolean |  | Indicates if this application is eligible for using edge resources |

Table 10.4a.2-2 provides a list of general notification events exposed at reference point EDGE-5/M6 if the UE supports EDGE-5 functionality.

Table 10.4a.2-2: Edge Resources Events

| Event | Definition | Payload |
| --- | --- | --- |
| EDGE_ACTIATION | Triggered when an edge resource is activated. | Media delivery session identifier |
| EDGE_DEACTIATION | Triggered when an edge resource is deactivated. | Media delivery session identifier |

Table 10.4a.2-3 provides a list of general error events exposed at reference point EDGE-5/M6 if the UE supports EDGE-5 functionality.

Table 10.4a.2-3: Error Events relating to Edge Processing

| Status | Definition | Payload |
| --- | --- | --- |
| EDGE_ERROR | Triggered when an edge resource is faulted. | Media delivery session identifier |