3GPP TSG-SA WG4 Meeting #129-e	S4-241508

# Online 19th-23th August 2024

Source:	HUAWEI

Title:	[FS_AI4Media] pCR on IMS mapping

Spec:	3GPP TR 26.927 v0.8.0

Agenda item:	9.6

Document for:	Discussion and agreement

1. Introduction

According to last SA4-e (AH) VIDEO SWG post 128 meeting, it was decided to move forward on the study of AI4Media mapping to IMS in SA4 before sending LS to SA2. This contribution proposes to move the PD content that agreed in S4-241284 to TR 26.927.

The content is updated according to the comments received during SA4#129:

- Removing the editor’s note in the beginning since the context is clear and there is no need to rename the AI function names.

- Removing AI Model Repository from the architecture diagram and the flow. Instead, AI models are downloaded from DC AS.

- Changing DCSF to NEF for DC AS accessing IMS AS following SA2 conclusion.

- Adding a basic flow for AI/ML model distribution

2. Reason for Change

To progress the study regarding the AI4Media mapping to IMS in TR 26.927.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 26.927.

* * * First Change * * * *

## 5.4	Architecture and procedures for AI data delivery over IMS

### 5.4.1	Architecture and components

Figure 5.4.1-1 AI data delivery over IMS architecture

Figure 5.4.1-1 shows a mapping of AI media functionalities to the IMS data channel architecture which is defined in clause AC.2 of TS 23.228 v18.5.0. The mapped AI media functionalities over IMS are the following:

UE:

-	AI Inference Engine: It has the capability to perform the inferencing of received (split) AI models.

-	AI Data Access/Delivery: It handles the access and delivery of AI data including

-	Download the AI model data for inference process. This includes instantiating an AI data access client to access and retrieve AI models or AI model subsets over the network (e.g., by streaming or downloading the model from a remote server).

-	Access/deliver intermediate data when an inference is split between the UE and the network.

-	Encode data to deliver with serialization and optionally compression techniques. Or conversely decode the received data with deserialization or optionally decompression techniques.

MF:

-	AI Data Access/Delivery: It handles the access and delivery of AI data as described above.

-	AI Media Inference Engine: It has the capability to perform the inferencing of (split) AI models.

DC AS:

-	It monitors, shares and/or reports network capabilities with/to the UE. This may be used for the selection of the model for a UE inference or for the selection of the UE model subset part for a split inference topology between the UE and the network. It may also provide AI model (including subset) storage and downloading function.

### 5.4.2	Procedures for AI/ML model distribution

Figure 5.4.2-1 Procedures for AI/ML model distribution

0. The audio/video and data channel sessions are established between the UE-A and the UE-B.

1. UE-A runs AI/ML application.

2. The application requires AI/ML model for inference, the UE-A sends a request for downloading AI/ML model to MF through application data channel, the MF forward the request to DC AS.

3. The DC AS responses with AI/ML model data.

4. The UE-A performs AI inferencing.

5. The UE-A may send the inference output result to the UE-B through application data channel or RTP.

### 5.4.3	Procedures for Split AI/ML operation

Figure 5.4.3-1 Procedures for split AI/ML operation

0. The audio/video and data channel sessions are established between the UE-A and the UE-B.

AI Split Inference Negotiation (This step may be performed at the beginning or during the session when the UE or network status has changed):

Alternative Case#1: Network decides the split inference:

1a. The UE-A gets the UE’s capability information, which may include the AI inference processing capabilities, supported AI framework information, connection capabilities, etc. When the UE-A discovers the UE’s local capabilities can’t meet the AI service requirement, it decides to trigger the split inference process.

2a. The UE-A sends an AI split inference request to the DC AS with UE’s capability information and the service requirement information.

3a. The DC AS gets the MF’s capability information, which includes the AI inference processing capabilities, supported AI framework information from the AI Inference Engine, and selects a proper AI model (including the UE AI model subset and the network AI model subset) for split inference from all matched AI models (with different candidate split points) based on the service requirement information, the UE’s capability information and the network’s capability information.

4a. The DC AS sends an AI Inference Resource Allocation request to the NEF, the request includes the selected network AI model subset information (including the split point and the intermediate data information). The request is transferred to the IMS AS.

5a. The IMS AS instructs a media resource allocation request to the MF.

6a. The MF responds with a successful result to the IMS AS, the IMS AS transfers the response to the NEF, and the NEF transfers it to the DC AS.

7a. The DC AS sends the AI Split Inference Response with the selected UE AI model subset information (including the split point and the intermediate data information) to the UE-A.

Alternative Case#2: UE decides the split inference:

1b. The UE-A gets the UE’s capability information, which may include the AI inference processing capabilities, supported AI framework information, connection capabilities, etc. When the UE-A discovers the UE’s local capabilities can’t meet the AI service requirement, it decides to get the AI models from the network for split inference process.

2b. The UE-A sends an AI Model Information Request to the DC AS with the UE’s capability information and the service requirement information.

3b. The DC AS collects all matched AI models with different candidate split points based on the service requirement information, the UE’s capability information and the network’s capability information.

4b. The DC AS sends the AI Model Information Response with all matched UE AI model subset(s) information (including the split point and the intermediate data information) to the UE-A.

5b. The UE-A selects a proper AI model based on the UE’s capability information and the received information in the AI Model Information Response.

6b. The UE-A sends an AI Split Inference Request to the DC AS with the selected network AI model information.

7b. The DC AS sends an AI Inference Resource Allocation request to the NEF, the request includes the selected network AI model subset information (including the split point and the intermediate data information). The request is transferred to the IMS AS.

8b. The IMS AS instructs a media resource allocation request to the MF.

9b. The MF responds with a successful result to the IMS AS, the IMS AS transfers the response to the NEF, and the DCSF transfers it to the DC AS.

10b. The DC AS sends the AI Split Inference Response to the UE-A.

AI Model Subset Delivery:

11. The MF downloads the network AI model subset from the DC AS.

12. The UE-A downloads UE AI model subset from the DC AS over the established data channel.

AI split inference:

Alternative case#1: data source from peer user

13a. The MF receives media data from the peer user.

14a. The MF performs network AI inferencing.

15a. The MF deliveries the intermediate data to the UE-A.

16a. The UE-A performs AI inferencing and output the inference result.

Alternative case#2: data source from local user

13b. The UE-A performs UE AI inferencing based on the media data generated locally.

14b. The UE-A deliveries the intermediate data and passes it to the MF.

15b. The MF performs network AI inferencing.

16b. The MF sends the inference output result to the UE-B.

NOTE 1: AI split inference and the negotiation may also happen between the peer UE and the MF. The flow is similar and not repeated here.

* * * End of Change * * * *