3GPP TSG-S4 Meeting   #131-bis-e	S4-250505

# Electronic Meeting, 11th–17th April 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | 0087 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD_PRO-MED] Stage-3 Aspects of Multi-access Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD_PRO-MED |  |  |  |  |  | Date: |  |  | 2025-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 26804 clause 7.3.3 describes recommendations for normative specification arising from version 19 for stage-3. This contribution implements those recommendations on topic of multi-access media delivery |  |  |  |  |  |  |  |  |
| Summary of change: |  | Below is the summary of changes to extend the media stream handling API between the 5GMS-aware application/Media Session Handler and the Media Stream Handler: - Changes to the Configuration Settings API to enable configuration of multipath delivery configuration - Changes to the Dynamic Status Information API to allow for status information exchange about multi-access connection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-access media delivery feature is incomplete |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.4, 13.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-250505: Version implementing normative recommendations documented in clause 7.3.3 on topic of multi-access media delivery |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 13.2.4	Configurations and settings API

DASH streaming for a particular downlink media delivery session may be configured by the 5GMSd-Aware Application at reference point M7d or by the Media Session Handler at reference point M11d with the parameters provided in table 13.2.4-1. Note that these parameters may be set and they may also be observed.

Table 13.2.4-1: Media Player Configuration API

| Status |  |  | Type | Definition |
| --- | --- | --- | --- | --- |
| sessionId |  |  | string | A media delivery session identifier for the downlink media streaming session that has been initialised using the method specified in clause 13.2.3.2. |
| source |  |  | Object | Provides the MPD and all contained information. |
| consumptionMode |  |  | Enum | Defines two modes: live: in this case the target latency is maintained, if specified in the service description, according to the parameters vod: in this case the latency is set by the application and the latency settings are ignored. |
| maxBufferTime |  |  | Integer | Maximum buffer time in milliseconds for the service. |
| serviceDescriptionId |  |  | id | Selects a service description by selecting an identifier. |
| serviceDescriptions[] |  |  | Service description parameters | Configures a service description as defined in annex K of ISO/IEC 23009-1 [32]. This allows the application to define additional service descriptions beyond those defined in the MPD. |
|  | id |  | id | Sets a service description identifier different from the ones available in the service descriptions in the MPD or modifies existing service descriptions. |
|  | serviceLatency |  | Object | Sets service description parameters for the service latency, as defined in table K.1 of ISO/IEC 23009-1 [32]. |
|  | playBackRate |  | Object | Sets service description parameters for the playback rate, as defined in table K.2 of ISO/IEC 23009-1 [32] when the service is consumed in live mode. |
|  | operatingQuality |  | Object | Sets service description parameters for the operating quality, as defined in table K.3 of ISO/IEC 23009-1 [32]. |
|  | operatingBandwidth |  | Object | Sets service description parameters for the operating bandwidth, as defined in table K.4 of ISO/IEC 23009-1 [32]. |
| mediaSettings[] |  |  | Media type audio, video, subtitle | Sets the selected Adaptation Set based on the available Adaptation Sets for each media type. |
| metricsConfiguration[ ] |  |  | Object | Zero or more sets of settings for collecting metrics in relation to the downlink media streaming session. |
| enableMultipathDelivery |  |  | boolean | Configures whether to use multipath delivery using multiple access networks. True: multipath delivery is enabled using multiple access networks False: multipath delivery is disabled |
| multipathTransportProtocol |  |  | string | Multipath transport protocol (MPQUIC and/or MPTCP) to be used in the multipath delivery connection |
| pathsForMultipathDelivery |  |  |  | Number of paths used by the Media Stream Handler for multipath delivery connection to a target service location/endpoint: - In case of MPQUIC based multipath delivery, this parameter represents the number of MPQUIC paths - In case of MPTCP based multipath delivery, this parameter represents the number of MPTCP subflows If both the minimum and maximum number of paths for the multipath delivery connection below are set to zero, multipath operation is disabled for that service location/endpoint |
|  | min |  | Integer | Minimum number of paths used by the Media Stream Handler for multipath delivery connection when MPTCP transport protocol is used |
|  | max |  | Integer | Maximum number of paths used by the Media Stream Handler for multipath delivery connection when MPTCP transport protocol is used |
| addressesForMultipathConnection |  |  |  | Configures the addresses to be learned over Multipath TCP connection |
|  | min |  | Integer | Minimum number of addresses that are to be learned over the Multipath TCP connection |
|  | max |  | Integer | Maximum number of addresses that are to be learned over the Multipath TCP connection |
| multipathConfigurationOptions |  |  |  | Configuration options for all interfaces in the multipath delivery connection |
|  | interfaceConfigurationOptions |  | Array of Objects | Configuration options for an interface in the multipath delivery connection |
|  |  | interfaceIdentifier | string | Identifier of the interface |
|  |  | pathIdentifier | string | If this is included, the multipath path manager creates a MPTCP subflow or MPQUIC path over this interface |
|  |  | backup | boolean | Indicates whether the interface serves as a backup interface |

second CHANGE

### 13.2.6	Dynamic Status Information

Table 13.2.6-1 provides a list of dynamically changing status information that can be obtained from the Media Player via reference point M7d or M11d. A separate set of Dynamic Status Information is provided for each active downlink media streaming session, indexed by its media delivery session identifier initialised per clause 13.2.3.2.

Table 13.2.6-1: Media Player Dynamic Status information

| Status |  |  |  | Type | Parameter | Definition |
| --- | --- | --- | --- | --- | --- | --- |
| state |  |  |  | Enumeration |  | An enumerated value from table 13.2.2-1 indicating the current state of the Media Player. |
| averageThroughput |  |  |  | float | none | Current average throughput computed in the ABR logic in bit/s. |
| bufferLength |  |  |  | float | MediaType "video", "audio" and "subtitle" | Current length of the buffer for a given media type, in seconds. If no type is passed in, then the minimum of video, audio and subtitle buffer length is returned. NaN is returned if an invalid type is requested, the presentation does not contain that type, or if no arguments are passed and the presentation does not include any adaption sets of valid media type. |
| liveLatency |  |  |  | float | none | Current live stream latency in seconds based on the latency measurement. |
| mediaSetting[ ] |  |  |  | MPDAdaptationSet | MediaType "video", "audio" and "subtitle" | Current media settings for each media type based on the CMAF Header and the MPD information based on the selected Adaptation Set for this media type. |
| mediaTime |  |  |  | float | None | Current media playback time from media playback platform. The media time is in seconds and is relative to the start of the playback and provides the media that is actually rendered. |
| playbackRate |  |  |  | float | None | The current rate of playback. For a video that is playing twice as fast as the default playback, the playbackRate value should be 2.00. |
| availableServiceDescriptions[ ] |  |  |  | Array of service descriptions |  | Provides the list of available selectable service descriptions with an id to select from. Those are either configured ones or the ones in the MPD. |
| availableMediaOptions[ ] |  |  |  | List of Adaptation Set or Preselection ids | MediaType "video", "audio" "subtitle" "all" | Provides the list of available media options that can be selected by the application based on the capability discovery and the subset information. |
| service‌Operation‌Points |  |  |  | array(Service‌Operation‌Point) |  | The set of Service Operation Points declared in the presentation manifest (e.g. DASH MPD) of the current media presentation. |
| operative‌Service‌Operation‌Point |  |  |  | integer |  | A zero-based index into the service‌Operation‌Points array indicating the Service Operation Point currently operative in the playback session. Set to -1 if the array is empty. |
| metrics[ ][ ] |  |  |  | Metrics |  | A data blob of metrics for each configured metrics collecting scheme. |
| multipathConnectionStatus |  |  |  | Object |  | Status information of multipath delivery connection |
|  | interfaceStatus |  |  | array(Object) |  | Status information of interface within the multipath delivery connection |
|  |  | interfaceIdentifier |  | Integer |  | Identifier of the interface |
|  |  | numberofPaths |  | Integer |  | Number of MPTCP subflows or MPQUIC paths on the interface represented by interfaceIdentifier |
|  |  | pathStatus |  | array(Object) |  | Statys information of all MPTCP subflows or MPQUIC paths over the interface |
|  |  |  | pathIdentifier | Integer |  | Identifier of MPTCP subflow or MPQUIC path |
|  |  |  | status | string |  | Status information of MPTCP subflow or MPQUIC path |

Table 13.2.6-2 provides a list of configured operation point information that can be obtained from the client. Any change to a parameter below shall be announced with a notification OPERATION_POINT_CHANGED as specified in table 13.2.5-1.

Table 13.2.6-2: Media Player Service Operation Point Information

| Parameter |  |  | Type | Definition |
| --- | --- | --- | --- | --- |
| ServiceOperationPoint |  |  | Object | The currently configured Service Operation Point parameters according to which the DASH client is operating. |
|  | externalIdentifier |  | String | The external identifier uniquely identifying this Service Operation Point in the presentation manifest (e.g. DASH MPD). |
|  | mode |  | Enum | The following operation modes are defined: live: The DASH client operates to maintain configured target latencies using playback rate adjustments and possibly resync. vod: The DASH client operates without latency requirements and rebuffering may result in additional latencies |
|  | maxBufferTime |  | Integer | maximum buffer time in milliseconds for the service. |
|  | switchBufferTime |  | Integer | buffer time threshold below which the DASH clients attempt to switch Representations. |
|  | latency |  | Object | Defines the latency parameters used by the DASH client when operating in live mode. |
|  |  | target | Integer | The target latency for the service in milliseconds. |
|  |  | max | Integer | The maximum latency for the service in milliseconds. |
|  |  | min | Integer | The maximum latency for the service in milliseconds. |
|  | playbackRate |  | MediaType audio, video, all | Defines the playback rate parameters used by the DASH client for catchup mode and deceleration to avoid buffer underruns and maintaining target latencies. |
|  |  | max | Real | The maximum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  |  | min | Real | The minimum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  | bitRate |  |  | Defines the operating bit rate parameters used by the DASH client used for a specific media type or aggregated. The values are on IP level. |
|  |  | target | Integer | The target bit rate for the service in bit/s that the client is configured to consume. |
|  |  | max | Integer | The maximum bit rate for the service in bit/s that the client is configured to consume. |
|  |  | min | Integer | The minimum bit rate for the service in bit/s that the client is configured to consume. |
|  | playerSpecificParameters |  |  | Player-specific parameters may be provided, for example about the used algorithm, etc. |

End of changes