3GPP TSG-SA WG4 #131-bis-e	S4-250612

e-Meeting, 11-17 April 2025

Source:	Qualcomm, Incorporated

Title:	Using a Periphonic Speaker Array for Improved Soundfield Reconstruction

Agenda item:	14.2

Document for:	Agreement

# Introduction

This document assesses the feasibility of using the ETSI TS 103 224 [1] method in combination with the periphonic array in TS 26.260 Clause 4.1.1.3 [2] in non-free-field environments and concludes that such an approach is viable.

To reliably characterize and test the performance of audio algorithms on user equipment (UE), it is necessary to capture and reconstruct an acoustic field in a robust and repeatable manner. An exemplary recording and field reconstruction technique has been described and standardized in ETSI TS 103 224 [1] and is already used in 3GPP TS 26.132 [3] for background noise tests. While this method has been shown to reconstruct a recorded field with a sufficient degree of spectral similarity and coherence, its efficacy in reconstructing more complex properties of acoustic fields for spatial audio capture scenarios has not been verified. In addition, applicability of the method to the periphonic array of TS 26.260 is also an area for investigation.

The work item ATIAS_Ph2 Tdoc SP-241314 [4] has, as one of its objectives, to “define new test methods and performance requirements/objectives, for the assessment of capture and playback of complex sound scenes, i.e., sound scenes with more than one source and from more than one defined direction.” For capturing of complex sound scenes, a loudspeaker array capable of accurately reproducing sound fields is needed as part of a test methodology.

3GPP TS 26.260 [2] defines, in Clause 4.1.1.3, a periphonic loudspeaker array capable of reproducing diffuse sound fields and HOA signals [2]. The test method requires a well-defined free-field environment to ensure reproducibility and repeatability of the sound field. At 3GPP SA4 #130 meeting, some companies expressed an interest, during discussions, to extend the test method to support test rooms beyond anechoic chambers.

In this contribution, the source:

- Reviews the method and reconstruction quality using the layout recommended in TS 103 224,

- Discuss its applicability to the periphonic loudspeaker layout defined in TS 26.260 (as a more geometrically complete and backwards compatible setup), and

- Proposes an extended reconstruction system metric to assess the reconstruction accuracy of microphone and loudspeaker layouts and the acoustic environments for the reconstruction.

The ETSI TS 103 224 test method in combination with the periphonic array from TS 26.260, applies to cases where the acoustic field must be reconstructed with improved field properties for immersive audio capture. An example of such application is testing UEs in the sending direction (SND) (in scope of ATIAS_Ph2 and possibly DaCAS).

# Technical Background

## Standard Overview

The multipoint reconstruction method described in [1] is based on the principle of regularized inversion in the frequency domain: Given a volumetric soundfield captured by a microphone array, the soundfield can be reconstructed by a surrounding loudspeaker array via inverse filters from each microphone element to each speaker element.

For frequency k, the transform from a vector of loudspeaker signals Xk to a vector of microphone signals Yk is characterized by the equation

Yk=HkXk

where the (i,j)th entry of matrix Hk represents the complex transfer function between the ith microphone element loudspeaker and jth loudspeaker element. A prototype reconstruction signal Xk is then derived by filtering the microphone signal Yk with a compensation filter Ck such that

Xk=CkYk

where Ck serves as a transform between speaker and microphone subspaces. The principle of operation draws on the existence of the Tikhonov-regularized pseudoinverse as an optimal minimum-power solution, and its efficacy can be validated using microphone elements located proximally to the original microphone elements. The algorithmic flow described in Section 6.2 of [1] is as follows:

- Level adjustment: Adjust speaker gains such that broadband pressure is produced with sufficient headroom.

- System identification: Obtain room impulse responses from each speaker to microphone element Hk.

- Preprocessing impulse responses: Window impulse responses using a time-dependent LPF.

- Calculate inverse filters: Obtain inverse filters Ck using an iterative binary search constrained by the maximum pressure received at the microphone array.

- Final equalization: Process reference recordings with the filters and record the resulting field using the same microphone layout, plus validation microphones. This yields an in-situ error function Ek that is inverted to yield a final compensation filter Dk=Ek-1.

With the inclusion of the final equalization filter, the final reconstruction speaker signal Xk and the corresponding microphone signal Yk are expressed as

Xk=DkXk=DkCkYk

Yk=HkXk=HkDkCkYk

According to TS 103 224, the level difference and complex coherence between reconstructed microphone signal Yk and the original microphone signal Yk are used as final metrics of reconstruction quality. In addition to the variables originally described, the source further defines Sk as the Reconstruction System Matrix (RSM). The RSM is used to inspect the reconstruction quality of a given combination of loudspeaker layout, microphone layout, and playback environment

Sk≜HkDkCk

In the original version of ETSI TS 103 224, the microphone arrangement was fixed while the speaker arrangement remained flexible with certain conditions on microphone-to-speaker distance and radial symmetry based on ETSI ES 202 396-1 [5]. In addition, constraints on reverberant properties of the reconstruction room were given. The cumulative reconstruction recommendations were described in Section 6 of [1].

In recent amendments of [1], the method has been generalized to include completely flexible microphone and loudspeaker layouts; this allows for the method to be used in acoustic environments where it is not feasible to follow the original recommendations on transducer geometry or test room. It additionally follows from the flexible layouts that the pre-optimization reverb tail windowing (Clause 6.2.3), selection of microphone array subset, and extended-frequency optimization (Clause 6.2.4) are made optional under the flexible method (Clauses 7.3, 7.4). Examples of the recommended loudspeaker arrangement and of a flexible loudspeaker arrangement are given in Figure 1 below.

Figure 1: Sample classic layout (left) and sample flexible layout (right).

## Algorithmic Implementation

An implementation of the algorithm presently discussed exists and is commercially available. However, it is the understanding of the Source that the hardware used in the commercial implementation is currently restricted to a maximum number of ten (10) speaker output channels. To accommodate speaker layouts exceeding this limit, the source uses a separate implementation of the method. The efficacy of this implementation is validated in Section 3, and results for the proposed speaker layout using this implementation are shown in Section 4 of the present contribution.

The final in-situ recording has not been incorporated into the present algorithm implementation, so a final equalization filter takes the form of a broadband unity gain, or Dk=I. Actual calculation of the final filter remains a point of work for a future contribution.

# Validation Experiment

## Objective

The objective of this experiment is to examine the reconstruction quality of a representative layout as recommended in original versions of the technical specification [1].

## Measurement Setup

Eight (8) microphones are arranged to favor field reproduction for binaural applications according to Clause 5.4 of [1], namely with four microphone elements concentrated at each ear of a head and torso simulator (HATS). The microphone elements are individually calibrated and compliant with Clause 5.1.2.

Eight (8) loudspeakers are arranged in the corners and side midpoints of a square with alternating loudspeakers at +15 cm and -15 cm heights with respect to the HATS ear reference point (ERP), according to the recommendation in [5]. Each loudspeaker consists of a two-way rear-ported system with a 5” woofer and 0.75” tweeter, resulting in a nominal frequency response of 54 Hz – 20 kHz. The reconstruction setup is situated in an anechoic chamber with 2.5×2.5×2.5 m of usable space and an ambient noise level of 17.1 dB(A). The reconstruction environment is shown in Figure 2 below.

Figure 2: Measurement environment for validation loudspeaker condition.

## Reconstruction System Matrix

To inspect the reconstruction quality of a given loudspeaker reconstruction array, the RSM Sk is collected for all frequencies and shown in Figure 3 below. All plots shown in the present contribution are smoothed over one-third (1/3) octave intervals.

Figure 3: RSM for validation loudspeaker layout.

In the above figure, the (i,j)th indexed plot represents the contribution of signal at the jth microphone element of the original recording to the signal at the ith microphone element of the reconstructed soundfield. By extension, the ith row of plots represents the decomposition of the ith microphone signal in the reconstructed field into the signal vector of the entire array in the original soundfield.

In each plot, the grey curve depicts the maximum magnitude across a row of Sk with respect to frequency, which corresponds with the maximum energy contributed from each original microphone signal and is denoted as the maximum magnitude function hereafter. At frequencies above 1 kHz, it is apparent that the maximum magnitude function is most like the transfer function at diagonal entries. This similarity suggests that the content captured at any given mic is received by the same mic upon reconstruction.

At frequencies below 1 kHz, the maximum magnitude function is like the magnitudes at all microphones in the row of transfer functions, suggesting that the original content captured at a given mic is distributed evenly to other mics upon reconstruction. This is corroborated by the physical relationship between wavelength and microphone spacing, namely that the spatial resolution of an acoustic field to be captured and reconstructed is proportionally related to frequency and inversely related to wavelength. This is described in Clause 5.1.1 of [1].

Figure 4a depicts the magnitude means of diagonal and off-diagonal entries of the RSM and Figure 4b depicts the difference between the two means. The highly positive difference between the diagonal and off-diagonal means, shown in Figure 4b, indicates that the distinct microphone signals at the original acoustic field are captured by the same microphones in the reconstructed acoustic field. This suggests a one-to-one reconstruction of the soundfield.

Figure 4: Diagonal and off-diagonal transfer functions of the RSM (a, left) and magnitude difference between magnitude means of all diagonal and all off-diagonal transfer functions separately (b, right), for the validation loudspeaker layout.

# Proposed Layout Experiment

## Objective

The objective of this experiment is to use a loudspeaker array with increased periphony as compared to the original recommendations in Clause 6.0 of [1]. The layout tested consists of approximately equidistant points on a spherical surface such that spherical integrations exhibit minimal discretization error [6].

## Measurement Setup

Eight (8) microphones are arranged around a HATS in the same layout as in the previous condition and used for recording and field reconstruction.

Twenty-five (25) speakers are arranged in a spherically equidistant array as described for N=4 in Annex A of [2] with an array radius of Rspk=1.1 m. Each loudspeaker element consists of a single 3” driver with a sealed back-volume, resulting in a nominal frequency range of 120 Hz – 18 kHz with approximately monopole directivity. The loudspeaker array is situated in a reverberant room with an open floor plan and nonstationary background noise. The ambient noise level, 60 dB reverberation time, and 80 ms clarity index are derived from the impulse response measured with the first loudspeaker and the first microphone and are listed in Table 1 below.

Table 1: Environmental acoustic properties of the proposed loudspeaker layout

| Ambient Noise Level | RT60 | C80 |
| --- | --- | --- |
| 41.0 dB(A) | 0.306 s | 28.8 dB |

Figure 5: Measurement environment for periphonic loudspeaker layout.

## Reconstruction System Matrix

Figure 6 shows the RSM of the method using the periphonic array in the experimental condition.

Figure 6: RSM for periphonic loudspeaker layout.

Similarly to the ETSI TS 103 224 implementation validation condition, the diagonal entries of the RSM coincide with the maximum magnitude function for frequencies above 1 kHz. For frequencies below 1 kHz, diagonal and off-diagonal entries exhibit similar magnitudes.

In contrast to the validation condition, the level difference between diagonal and off-diagonal transfer functions is higher than that of the validation condition. This difference can be interpreted as a higher correlation between microphone signals in the original and reconstructed soundfield and is most apparent by the increased difference between diagonal and off-diagonal averages in Figure 7b below.

Figure 7: Diagonal and off-diagonal transfer functions of the RSM (a, left) and magnitude difference between magnitude means of all diagonal and all off-diagonal transfer functions separately (b, right), for periphonic loudspeaker layout.

# Conclusion and Proposal

Based on the discussion above:

- The reconstruction method in ETSI TS 103 224 can be applied to the periphonic speaker array of order N=4 from 3GPP TS 26.260.

- The use of a periphonic speaker array of order N=4 indicates an increase in the soundfield reconstruction accuracy (compared to the 8-speaker array used, e.g. for the current background noise reproduction tests in 3GPP TS 26.132).

- A Reconstruction System Matrix (RSM) metric is proposed to assist in the assessment of soundfield reconstruction accuracy given a combination of microphone layout, loudspeaker layout, and acoustic environment.

NOTE: An augmentation of the RSM metric to measure the (undesirable) presence of microphone overfitting is for further study. Alternative microphone arrays, with potentially augmented soundfield capture properties are also for further study.

It is proposed to adopt the ETSI TS 103 224 method in combination with the N=4 periphonic speaker array from 3GPP TS 26.260 as a working assumption to generate diffuse sound-fields in non-free-field test rooms.

# References

| [1] | ETSI TS 103 224, “A sound field reproduction method for terminal testing including a background noise database,” 2022. |
| --- | --- |
| [2] | 3GPP TS 26.260, “Objective test methodologies for the evaluation of immersive audio systems,” 2024. |
| [3] | 3GPP 26.132, “Speech and video telephony terminal acoustic test,” 2022. |
| [4] | SP-241314, ATIAS_Ph2 Tdoc, “WID on Terminal Audio quality performance and Test methods for Immersive Audio Services, Phase 2”. |
| [5] | ETSI ES 202 396-1, “Background noise simulation technique and background noise database,” 2022. |
| [6] | U. M. Jörg Fliege, “A Two-stage Approach for Computing Cubature Formulae for the Sphere,” 1996. |

Image reproduced from

https://cdn.head-acoustics.com/fileadmin/data/global/Datasheets/Playback_Simulation/3PASS-lab-Noise-Simulation-6990-Data-Sheet.pdf

Image reproduced from

https://cdn.head-acoustics.com/fileadmin/data/global/Datasheets/Playback_Simulation/3PASS-flex-Noise-Simulation-6995-Data-Sheet.pdf

It is worth noting that other quadrature formulae for integration over a spherical grid are acceptable, such as the Lebedev and Gauss-Legendre grids. Equiangular grids are also acceptable, though its efficacy may diminish with dense angular spacings.