3GPP TSG-SA WG4 Meeting #131-bis-e			S4-250643

Online, 11 – 17 April 2025

Source:	Xiaomi EV Technology

Title:	Adding recording scenarios and database

Agenda item:	7.6

Document for:	DISCUSSION&AGREEMENT

# Introduction

This proposal describes  a recording setups and scenarios conducted in a hall. Corresponding database was captured using Target Device 1[1], and the associated Excel files (Recordings Database.xlsx and Recording Setups and Scenarios.xlsx ) are attached .

# Content

A hall with long reverb time was used to recorded speech signal from loudspeaker.

## Recording setups and scenarios

Sound source:

loudspeaker

Source signal:

speech

Signal characteristics

Language: Chinese

Length: 32s

Level: -24 dB RMS

Sample rate: 48 kHz

Bit depth: 24bit

Sound source calibration

Level: Loudspeaker calibrated with -12 dBFS pink noise to 70 dBSPL(A) at the UE position.

Frequency response: Not applicable.

Acoustic environment

Music/ Lecture Hall

Reverb time:

| frequency(Hz) | RT60(Second) |
| --- | --- |
| 100 | 2.27 |
| 125 | 2.11 |
| 160 | 2.04 |
| 200 | 1.75 |
| 250 | 1.5 |
| 315 | 1.58 |
| 400 | 1.52 |
| 500 | 1.58 |
| 630 | 1.59 |
| 800 | 1.68 |
| 1000 | 1.73 |
| 1250 | 1.72 |
| 1600 | 1.7 |
| 2000 | 1.68 |
| 2500 | 1.67 |
| 3150 | 1.7 |
| 4000 | 1.69 |
| 5000 | 1.49 |

Detailed Positioning

Source distance: d_UE = 3m

Source direction:  Main diaphragm towards the UE

Source angles (azi, ele):  azi = 0°, ±30°, ±60°, ±90°, ±120°, ±150°, 180° (12 angles in total), ele = 0°

The space between the walls and sound source/UE should be at least 0.5 m

Description/additional info:

The UE position is set in the Center stage.

Loudspeaker: Genelec 8030A

Soundcard: RME Fireface UCX"

[Note: For the high-reverberation scenarios (RT60 ≥ [TBD]) , it’s recommended to employ professional microphones (like ambisonic microphone) to capture reference data.]

## Recordings database

Target device No:
1

Recording scenario No:
1

File name:
e.g. Hall_Original-Horizontal_0deg_M-2.wav

Language: 
Chinese

UE orientation / placement
Horizontal: The UE is on the same plane as all speakers, with the UE's long edge is towards the ceiling and floor, and the screen is towards the Azi 180° speaker.

Vertical: The UE is on the same plane as all speakers, with the UE's wide edge is towards the ceiling and floor, and the screen is towards the Azi 180° speaker.

Sample rate
48000

Bit depth
16bit

Audio format
wav

Rec Duration
Overall length 32s, first and last 1s is silence for each file

Number of channels and microphone mapping: 
4 channels,  ch1=mic1, ch2=mic2, ch3=mic3, ch4=mic4

Description/additional info
Fixed single source

Source signal contains 2 speeches (male(M) and female (W)), 12 source angles, and two recording placement (Horizontal and Vertical), 48 files in total.

License

(C) Copyright Beijing Xiaomi Mobile Software Co., Ltd (Xiaomi) 2025

All Rights Reserved.

This database called "Common database for target device A" is protected by copyright law and international treaties. The database shall be used solely for the purpose of the 3GPP Standardization of the Diverse audio Capturing System for UEs (DaCAS)resulting therefrom. The use of this database for any other purpose is not permitted.

Xiaomi retains the full right to revise this database and/or documentation, to make changes from time to time in the content hereof without obligation of Xiaomi to notify any person of such revision or change. Further, Xiaomi reserves the right to assign, license or donate this database to a third party.

The database is provided “AS IS”, without any express or implied warranty of any kind and nature, including, but not limited to, any warranty of non-infringement, merchantability and/or fitness for a particular purpose.

This copyright notice must be included in each reproduction of any part of the database materials.

## Database Management

Given the expectation of a larger database in the future, the database for this document has been uploaded to the following path:

Link:

https://kpan.mioffice.cn/webfolder/ext/zh%24DE%24XejVH%24uVm31GQvyw%40%40?n=0.1314974258223881

password:

0x7o

The final solution for uploading the database will depend on discussions during the meeting.

# Conclusion

## This document details a long-reverberation scenario and includes a fixed-sound-source recording database. Corresponding Excels for scenario and database are also attached. Further databases expansions  (like moving source, multi-source ) will be implemented following the discussion in the meeting.

## References

## [1]S4aA250006: Proposal of a target device.