3GPP TSG-SA WG4 Meeting #131-bis-e	S4-250661

Online, 11 – 17 April 2025

Agenda item: 	8.5

Source: 	Dolby Laboratories, Inc.

Title: 	[AMD_PRO-MED] Analysis of required changes to 5GMS stage-3 specifications for implementing Work Topic 2: Media delivery from multiple service locations

Document for	Discussion

# Introduction

Media delivery from multiple service endpoints/locations as introduced in clause 5.19 and based on the conclusions in clause 6.19 of TR 26.804 [1] recommended the following stage-3 updates:

i.	Document the generic MIME content types and references to valid profiles or relevant external specifications for Content Preparation Templates used for the purposes of multi-source/service location content preparation (item 2 of clause 5.19.7 of TR 26.804).

ii.	Extend the ContentHostingConfiguration resource to allow Content Distributions to be declared in hierarchical or peer-to-peer configurations (item 4 of clause 5.19.7 of TR 26.804).

iii.	Extend the ContentHostingConfiguration resource to allow the 5GMSd Application Provider the capability to influence the configuration and deployment of Content Distributions with the 5GMSd AS at the time of provisioning (item 5 of clause 5.19.7 of TR 26.804).

iv.	Clarify the use of the Media Entry Point for the purposes of communicating service location and multi-source/service location configuration information to 5GMSd Clients (item 6 of clause 5.19.7 of TR 26.804).

v.	Clarify the expectation that the Media Player natively supports the multi-source/service location approach in use (item 8 of clause 5.19.7 of TR 26.804)

vi.	Introduce CMMF [2] in TS 26.511 [3] as a format for delivering media from multiple service locations including possible definition of CMMF profiles for use in 5GMS.

vii.	Introduce Content Steering [4] as an M4 API in TS 26.512 [5] and for use with 3GP-DASH (TS 26.247 [6]).

viii.	Support other relevant aspects resulting from stage-2.

This document provides an analysis of the changes and updates required to implement the above in TS 26.510 [7], TS 26.511 [3], TS 26.512 [5], and TS 26.247 [6].

# Analysis of updates needed to 5GMS stage-3 specifications

## General extensions

The below analysis identifies general extensions to existing stage-3 specifications and highlights specific items that require further discussion.

-	Reference point M1 changes

-	Update Content Hosting/Publishing provisioning APIs (TS 26.510) for both M1d and M1u to support multiple service location deployments.

-	Update Content Hosting provisioning APIs (TS 26.510 and TS 26.512) to bring M10d into scope and allow a Media Application Provider to chain Content Hosting resources together via reference point M10. Discussion required:

-	Should service chaining using M10u (which is currently not defined in TS 26.501) be allowed?

-	Content Preparation Templates (TS 26.512 and potentially TS 26.510) for enabling delivery from multiple service location (e.g., Media Entry Point (e.g., MPD) manipulation, CMMF encoding/decoding, etc.). Discussion required:

-	Currently, Content Preparation Templates are ill-defined. Essentially, TS 26.510 and TS 26.512 say an Application Provider provisions the 5GMS System using a HTTP POST of a Content Preparation Template resource that is simply identified by its MIME type. Are there requirements that these Content Preparation Template resources must conform to? Can these Content Preparation Templates contain executable code (e.g., a script, Docker image, VM, etc.)? Do these Content Preparation Template resources contain only some configuration information that is used by a process that is known to the 5GMS AS? If so, where are these processes defined?

-	Should a general Content Preparation Template that automatically creates a Media Entry Point containing service location information based on how the 5GMS System is configured by the 5GMS Application Provider be defined (see the discussion below about Media Entry Points)?

-	Where should Content Preparation Templates be defined? Content Preparation Templates are somewhat unique to 5GMS and should be defined somewhere within the 5GMS specifications. From experience trying to implement something like a Content Preparation Template on commercial platforms (e.g., Fastly, Limelight, etc.), the implementation has always resulted in a bespoke solution.

-	Reference point M2 changes

-	No updates expected.

-	Reference point M3 changes

-	No updates expected.

-	Reference point M4 changes

-	Expand the scope of a Media Entry Point beyond just a MPD, HLS Master Playlist, or URL to a video file (TS 26.510 and TS 26.512) to allow for a streaming session configuration (e.g., containing information such as service locations, multiple service location configuration information, etc.) to be provided as well. There are a couple of different facets to this item:

-	A method to communicate multiple service location information within a Media Entry Point is needed. For some existing Media Entry Points (e.g., MPDs), this is not necessarily a problem since this additional information can be embedded into them. However, there are other types of Media Entry Points where this is not possible (e.g., a URL to a video file, HLS playlist, etc.) which requires a different approach. The approach proposed is to define a higher-level document that acts as the Media Entry Point. This document provides the necessary multiple service location information (or a pointer to, yet another, document that contains this information) along with the original Media Entry Point or a pointer to the original Media Entry Point (e.g., URL to a MPD, video file, etc.). Of course, this could be extended to provide additional information beyond just service location information.

-	Provide the necessary information to the 5GMS client to access media from multiple service locations when media is selected from a catalogue by the 5GMS-Aware Application (i.e., a URL to an MPD is provided via M8/M6/M7). In this case, it could be assumed the MPD/HLS playlist/etc. has not been updated to reflect how the 5GMS System has been configured. The concept above works for this case if the higher-level Media Entry Point document is allowed to have a pointer that contains a wildcard indicating that the URL to the MPD/video file/etc. should be obtained from the 5GMS-Aware Application (or somewhere else).

-	Define/describe the use of M4 for the purposes of multiple service location delivery (TS 26.510 and TS 26.512). Since different approaches/technologies use M4 in different ways, the specifics are captured below.

-	Updates to the canonical 5GMS AS authority (TS 26.512). Currently a canonical domain name is created by the Media AF that only references the Provisioning session even when multiple distribution/contribution configurations are defined in the Content Hosting/Publishing Configuration. A method to differentiate between service locations associated with different distribution/contribution configurations is needed.

-	Reference point M5 changes

-	Clarify the scope and use of Media Entry Points in the Service Access Information APIs (TS 26.510) to support the expanded scope of the Media Entry Point described above.

-	Reference point M6 changes

-	APIs may need to be defined specific to this topic that allow the 5GMS-Aware Application to configure/monitor the use of multiple service locations. APIs have not yet been defined noting that defining a general API may be difficult since different multiple service location technologies may require different APIs. (TS 26.512)

-	Reference point M7 changes

-	APIs may need to be defined specific to this topic that allow the 5GMS-Aware Application to configure/monitor the use of multiple service locations. APIs have not yet been defined noting that defining a general API may be difficult since different multiple service location technologies may require different APIs. (TS 26.512). Clause 13.2 in TS 26.512 currently has an API that allows you to pass an MPD to the Media Player. Perhaps an API that also allows a Media Player to be configured using an API that communicates a streaming configuration (see discussion above on the Media Entry Point) is useful.

-	Reference point M8 changes

-	No updates expected.

-	Reference point M9 changes

-	No updates expected.

-	Reference point M10 changes

-	Bring M10 into scope for the purposes of service chaining (TS 26.510 and TS 26.512). This also requires defining media ingest and publish protocols for M10 (TS 26.512). Discussion required:

-	Is push-based ingest on M10 allowed (noting that M10 looks like M2 from the perspective of a downstream 5GMS AS instance, and M10 looks like M4 from the perspective of an upstream 5GMS AS instance)? If it is, there are potential impacts to M1 (a modification to the Content Hosting/Contribution APIs would be required to allow for push on M4/M10 to be configured) and M4 (push on M4/M10 would be required). If push is not allowed, there could be impacts to latency when service chaining is in use.

-	Reference point M11 changes

-	APIs may need to be defined specific to this topic that allow the Media Session Handler to configure/monitor the use of multiple service locations. Defining a general API may be difficult since different multiple service location technologies may require different APIs (TS 26.512). Clause 13.2 in TS 26.512 currently has an API that allows you to pass an MPD to the player. Perhaps an API that also allows a Media Player to be configured using an API that communicates a streaming configuration (see discussion above on the Media Entry Point) is useful.

-	Reference point M12 changes

-	No updates expected.

-	Reference point M13 changes

-	Define procedures of the M13d and M13u interfaces (TS 26.512).

## Extensions specific to Content Steering (ETSI TS 103 998)

The below analysis identifies extensions specific to Content Steering [3] to existing stage-3 specifications and highlights specific items that require further discussion.

-	Reference point M1 changes

-	Content Preparation Template to support Media Entry Point creation/modification for Content Steering (e.g., perform MPD manipulation to add Content Steering information to existing MPDs). (TS 26.512). Reference the above for more details about what needs to be discussed further.

-	Reference point M2 changes

-	No updates expected.

-	Reference point M3 changes

-	No updates expected.

-	Reference point M4 changes

-	Content Steering manifest retrieval (TS 26.512). Content Steering (as defined in ETSI TS 103 998) defines a process for obtaining Content Steering information from a Content Steering Server. Discussion required:

-	Where does this belong in TS 26.512?

-	Description of what the Access Client does with the Content Steering information obtained on reference point M4 (TS 26.512). Discussion required:

-	Where does this belong in TS 26.512?

-	Media Entry Point example (TS 26.247 or TS 26.512).

-	Reference point M5 changes

-	No updates expected.

-	Reference point M6 changes

-	It may be useful to define an API specific to Content Steering (TS 26.512). Should APIs be defined and where should they be included in TS 26.512?

-	Reference point M7 changes

-	It may be useful to define an API specific to Content Steering (TS 26.512). Should APIs be defined and where should they be included in TS 26.512?

-	Reference point M8 changes

-	No updates expected.

-	Reference point M9 changes

-	No updates expected.

-	Reference point M10 changes

-	No updates expected.

-	Reference point M11 changes

-	It may be useful to define an API specific to Content Steering (TS 26.512). Should APIs be defined and where should they be included in TS 26.512?

-	Reference point M12 changes

-	No updates expected.

-	Reference point M13 changes

-	No updates expected.

## Extensions specific to CMMF (ETSI TS 103 973)

The below analysis identifies extensions specific to CMMF [3] to existing stage-3 specifications and highlights specific items that require further discussion.

-	Reference point M1 changes

-	Content Preparation Template for CMMF Encoding/Decoding to be performed by the 5GMS AS (TS 26.512). Reference the above for more details about what needs to be discussed further.

-	Reference point M2 changes

-	No updates expected.

-	Reference point M3 changes

-	No updates expected.

-	Reference point M4 changes

-	Media Entry Point formats and examples that contain CMMF configuration information (TS 26.512). Different versions, based on how the mapping between media objects and CMMF objects are performed (see below), should be created.

-	Procedure for accessing media communicated from one or more service locations using CMMF (e.g., parallel/simultaneous download of CMMF objects/stripes from multiple service locations for each individual media object) (TS 26.512).

-	Where does this belong in TS 26.512?

-	Reference point M5 changes

-	No updates expected.

-	Reference point M6 changes

-	It may be useful to define an API specific to CMMF (TS 26.512). Should APIs be defined and where should this be included in TS 26.512?

-	Reference point M7 changes

-	It may be useful to define an API specific to CMMF (TS 26.512). Should APIs be defined and where should this be included in TS 26.512?

-	Reference point M8 changes

-	No updates expected.

-	Reference point M9 changes

-	No updates expected.

-	Reference point M10 changes

-	No updates expected.

-	Reference point M11 changes

-	It may be useful to define an API specific to CMMF (TS 26.512). Should APIs be defined and where should this be included in TS 26.512?

-	Reference point M12 changes

-	No updates expected.

-	Reference point M13 changes

-	No updates expected.

-	Other updates not specific to defined reference points

-	5GMS supported CMMF modes/capabilities (TS 26.511 or TS 26.512). CMMF (as defined in ETSI TS 103 973) contains a lot of options on how to encode and package media within a CMMF object. This update defines a subset of modes and capabilities that should be supported for CMMF’s use within 5GMS. For example, this update could specify which code types are supported, which sub-atoms are supported, etc.

-	Mappings between CMAF objects and CMMF objects (TS 26.511 or TS 26.512). Specify different methods to map between CMAF objects and CMMF objects. The following mappings will need to be defined:

-	A general mapping between segmented media and CMMF objects (e.g., each individual segment listed within an MPD maps into one or more CMMF objects/stripes). Note, the contents of the MPD do not need to be known to the 5GMS AS since each (addressable) object ingested at reference point M2 can be mapped to one or more CMMF objects. However, this does need to be described within the 5GMS specifications.

-	A reference point M4 request-based mapping for non-segmented media (e.g., CMAF chunks). CMMF objects are created by the 5GMS AS based on the requests received from Access Clients on reference point M4. For example, this will describe how a byte range request by the Access Client is translated into addressable CMMF objects transmitted at reference point M4.

-	An explicit mapping of an MPD to CMMF objects. Essentially, the mapping is embedded within an MPD for every segment listed. An example could be something like the EFDT described in ETSI TS 103 973.

-	Define CMMF profiles for 5GMS downlink/uplink (TS 26.511 or TS 26.512).

# Proposal

It is proposed to implement the above identified updates (after resolution of the items marked for discussion) within 5GMS stage-3 specifications.

# References

[1] 	3GPP TS 26.804: "Study on 5G media streaming extensions".

[2] 	ETSI TS 103 973: “Coded Multisource Media Format (CMMF) for Content Distribution and Delivery”, October 2024.

[3] 	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[4] 	ETSI TS 103 998: “Publicly Available Specification (PAS); DASH-IF: Content Steering for DASH”, January 2024.

[5] 	3GPP TS 26.512: "5G Media Streaming (5GMS) Protocols".

[6] 	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[7] 	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".