3GPP TSG-SA WG4 Meeting #131e	S4-250690

Online,  April 11th – 17th, 2025

Title:	Reply on Inclusion of NTN intended service area in the Service Announcement

Response to:	-

Release:	Rel-19

Work Item:	NR_NTN_Ph3-Core

Source:	SA4

To:	RAN2

Cc: 	SA2

# Contact Person:

# Name:	Thorsten Lohmar

E-mail Address:

# Attachments:	S2-2501328

# 1. Overall Description:

# First, SA2 has inserted the “MBS broadcast support for NTN” some time ago. It seems transparent to SA4, but there are some issues.

- SA2’s stage 2 is only on MB-SMF and MB-UPF. Stage 2 of MBSF and MBSTF is in SA4

  - Nmbsmf (Nmb13 and N29mb) are extended with NR NTN support. Nmbsf would require some extensions, independently from this “ISA in Service Announcement” issue.

  - CT3 has already completed stage 3 for Nmbsmf. Interestingly, some data types (specifically the TS 29.571 MbsServiceArea data types) are also used at Nmbsf and also in Service Announcement

    - Here, CT3 has added the feature of “civic addresses”. Since CT3 uses some common data types, the Nmbsf API also supports Civic addresses. I think, we should request CT3 to add a Note, that the civic address is only allowed for Nmbsmf.

    - The MbsServiceArea data type is also used as common data type in SA4’s Service Announcement. I think, we should start creating a separate Data Type for Service Announcement, so that the ISA is properly JSON encoded.

# Then, on the RAN2 request for inserting the ISA into Service Announcement, I am still struggling to understand the motivation

- SA2 has already extended the MB-SMF API with support for NR NTN (See S2-2501328, which is attached in the LS).

  - Here, the AF provides the ISA in form of a geoshape (together with the TAI list and the TMGIs) with the MBS Session Creation to the MB-SMF. The ISA and the TMGI are then trickling down to RAN.

  - In RAN, the TMGI is announced via MCCH and the ISA is announced via a new SIB. UEs are supposed to read these SIBs (WIUC).

  - When a UE enters a cell, the UE reads the SIB and the ISAs.

  - à It is unclear for me, why the ISA should also be added into Service Announcement. The UE gets the ISA from SIB, which is typically read by the UE.

  - à I start getting the impression, that “adding ISA into Service Announcement” is an optimization to avoid reading the MCCH to find the info on the ISA-ID.

  - When this assessment is correct,

    - it might be better to use the ISA-ID in the Service Announcement, since the ISA is already known from reading the SIB. Possibly, it can be either ISA or ISA-ID in Service Announcement.

    - There should be also the option to NOT include the ISA (or ISA-ID) into Service Announcement, since this looks like an optimization.

# 2. Actions:

# To SA WG 4:

# RAN2 kindly request SA4 to confirm whether the inclusion of the ISA in the Service Announcement can be supported within the Rel-19 timeframe.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #129bis		7th April – 11th April. 2025				China

TSG RAN WG2 Meeting #130		19th May – 23rd May 2025				Malta