3GPP TSG-SA SA4#131-bis-e	S4-250730

# Online, 11 – 17 April 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.956 | CR |  | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: |  | pCR on inclusion of dynamic mesh and V-DMC in scenario 2 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  | InterDigital, Nokia, Samsung, Sony Group Corporation,China Mobile |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Beyond2D |  |  |  | Date: |  |  | 2025-04-04 |
| Category: | B |  |  |  | Release: |  |  | Rel-19 |
|  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |  |
| Reason for change: |  |  |  | Dynamic mesh have been included in the TR as representation format. This contribution includes dynamic mesh and V-DMC in scenario 2 |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  | Includes dynamic mesh and V-DMC in scenario 2 |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  | Scenario for dynamic mesh would be unclear |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  | Changes are tracked starting from TR V0.3.0 |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  | S4-250447 |  |  |  |  |  |  |  |  |  |

== CHANGE 3 ===

### 7.3.5	Encoding and decoding constraints and settings

The following table provides an overview of encoding and decoding constraints for V-PCC with H.265/HEVC as codec for the Volumetric Video with single asset streaming scenario. Contribution aspects are not considered in this table.

Table 7.3.5  Encoding and decoding constraints

| Encoding and Decoding Constraints | V-PCC with H.265/HEVC |
| --- | --- |
| Relevant Codec and Codec Profile/Levels | H.265/HEVC Main 10 Profile Level 4.1, 5.1 Metadata stream parsing |
| Random access frequency | 1 seconds |
| Bit rates and quality configuration | Fixed QP Geometry: [32;28;24;20;16] Fixed QP Texture: [42;37;32;27;22] bitrates [1;50 Mbps] |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | Covering a range of relevant bitrates and qualities |
| Latency requirements and specific encoding settings | No specific latency requirement |
| Encoding complexity context | Cloud-based encoding, offline encoding |
| Required decoding capabilities | 3 decoder instantiations of H.265/HEVC Main 10 Profile Level 4.1, 5.1 for (occupancy, geometry and color) One synchronized metadata bitstream (Atlas) |

Table 7.3.6  Encoding and decoding constraints

| Encoding and Decoding Constraints | V-DMC with H.265/HEVC |
| --- | --- |
| Relevant Codec and Codec Profile/Levels | Texture: H.265/HEVC Main 10 Profile Level 4.1, 5.1 Geometry: H.265/HEVC Main 10 Profile Level 3.0 (e.g. 256x768 at 30 fps) Metadata stream parsing |
| Random access frequency | 1 seconds |
| Bit rates and quality configuration | TBD |
| Bit rate parameters (CBR, VBR, CAE, HRD parameters) | TBD |
| Latency requirements and specific encoding settings | No specific latency requirement |
| Encoding complexity context | Cloud-based encoding, offline encoding |
| Required decoding capabilities | - Two decoder instantiations of H.265/HEVC Main 10 Profile (Level 5.1 for texture, Level 3.0 for displacement) - For packed video, one H.265/HEVC Main 10 Profile, Level 5.1 - Decoding of the base mesh (not video decoding) TBD |

Editor’s note: Information TBD to be provided

== CHANGE 6 ===

### 7.3.8	Test Sequences

Editor’s Note: The content of this scenario relates to moving persons.

#### 7.3.8.1	Candidate source sequences

Collected candidate raw dense point cloud sequences and dynamic mesh sequences that are available for testing are presented in Annex C.2

#### 7.3.8.2	Selected source sequences for testing

This section lists 5 sequences that have been selected for objective and subjective testing.

Test sequences have been selected based on visual quality and that these have not been used during codec development in MPEG. The following table lists the selected test sequences:

Table 7.3.8.2 Selected source dense dynamic point cloud sequences

Table 7.3.8.3 Selected source dynamic mesh sequences

Editor’s note: To be done for dynamic mesh

The following thumbnails illustrate the selected test sequences:

Figure 7.3.8.2-1 Aliya, Henry, Nathalie, Mitch and Joggle Soccer (content courtesy by Renderpeople, Volucap and XD Productions)

The selected licensed source sequences can be grouped as follows:

-	Group 1 - Freely available to 3GPP members: Nathalie, Mitch and Joggle Soccer

-	Group 2 - Publicly purchasable: Henry

-	Group 3 - Publicly free available: Aliyah

Editor’s note: Test sequences for dynamic mesh to be added

Sequences of group 1 have already been converted to pointclouds of around 2 million points per frame and maximum 10s length and are provided on the server. Sequences of group 2 and group 3 need to be downloaded and converted by those wanting to reproduce completely the test.

Editor’s Note: Provide instructions and scripts on how to convert sequences of group 2 and group 3

#### 7.3.8.3	Metadata for source dense dynamic point cloud sequences

##### 7.3.8.3.1	Overview

For a raw dense point cloud sequence used in the context of this Technical Report, the following metadata is proposed:

-	Name of the sequence

-	Scenario

-	Sequence key as provided in TR 26.956

-	URI to zip file containing PLY files (URL to weblink where the original sequence is available)

-	Name Format of files

-	Frame count

-	Start frame number

-	Frame rate

-	Geometry precision

-	Color format

-	Peak Value (bounding box resolution e.g. 1023, 2047)

-	Copyright statement

-	Contact Person

Optionally, the following metadata can be added:

-	Background

-	TR 26.956 reference

-	Thumbnail preview

-	Preview MP4

-	Average point number per frame

-	Duration

-	MD5 of the zip file containing all point cloud frames

Size in bytes of the zip file containing all point cloud frames.

A JSON scheme is defined in clause 2.2 for this matter. An example is provided in clause 2.3

##### 7.3.8.3.2	JSON Scheme

JSON schema for the raw format can be found in Annex B.2.3.

##### 7.3.8.3.3	Example

An example is attached to the zip file:

{

"Sequence": {

"Name": "Exemple",

"Background": "This is a B2DV format example",

"Scenario": "Streaming of Beyond 2D Produced VoD Content",

"Key": "S01",

"TR26.956": "Annex X.Y.Z"

},

"Properties": {

"URI": "https://dash-large-files.akamaized.net/WAVE/3GPP/Beyond2D/ReferenceSequences/file.zip",

"thumbnail": "https://dash-large-files.akamaized.net/WAVE/3GPP/Beyond2D/ReferenceSequences/file.png",

"preview": "https://dash-large-files.akamaized.net/WAVE/3GPP/Beyond2D/ReferenceSequences/file.mp4",

"NameFormat": "exemple%04d.ply",

"frameCount": 320,

"startFrame": 0,

"frameRate": 30,

"pointsCountMean": 1000000,

"geometryPrecision": 10,

"colorformat": "rgb",

"peak": 2047,

"duration": 10.0,

"md5": "d055a94f35f7594776186fc5d09a9fa4",

"size": 11510343938

},

"CopyRight": "Conditions that are suitable for this study",

"Contact": {

"Name": "Celine Guede",

"Company": "InterDigital",

"e-mail": "celine.guede@interdigital.com",

"generation": "provided by contact"

}

}

#### 7.3.8.3	Metadata for source dynamic mesh sequences

Editor’s note: To be done for dynamic mesh

== CHANGE 7 ===

### 7.3.9	Detailed test conditions

#### 7.3.9.1	V-PCC

Editor’s note: Contribution S4-250446 on update V-PCC testing will replace that section if agreed

##### 7.3.9.1.1	V-PCC test model and configuration files

The public version of the MPEG V-PCC test model named tmc2 in master branch is used to encode and decode dense dynamic point clouds [Vol-26].

Editor’s note:	Configuration files to be validated

For using tmc2 in Random Access (RA) mode, MPEG provides a configuration file [Vol-27].

For each selected test sequence, a configuration file containing information needed for tmc2 configuration will be provided.

##### 7.3.9.1.2	Rate points and test conditions

In line with the V-PCC verification test [Vol-28], 5 rate points R1 to R5 for Random Access (RA) will be used for each test sequence. Rates are obtained with the following V-PCC codec parameters:

|  | R1 | R2 | R3 | R4 | R5 |
| --- | --- | --- | --- | --- | --- |
| QP geometry | 32 | 28 | 24 | 20 | 16 |
| QP Texture | 42 | 37 | 32 | 27 | 22 |
| Occupancy Precision | 4 | 4 | 4 | 4 | 2 |

Editor’s note:	Configuration files to be validated

For using this rate configuration, MPEG provides configuration files for each rate [Vol-29].

##### 7.3.9.1.3	Profiles

In line with the V-PCC verification test [Vol-28], the following profiles are evaluated:

-	Basic which corresponds to V-PCC profile HEVC Main10 V-PCC Basic Rec0. This profile uses the basic V-PCC toolset and a simple reconstruction of points in 3D space.

-	Enhanced which corresponds to V-PCC profile HEVC Main10 V-PCC Basic Rec2. This profile uses the basic V-PCC toolset and a more sophisticated reconstruction of points in 3D space.

-	High which corresponds to V-PCC profile HEVC Main10 V-PCC Extended Rec2. This profile uses the extended V-PCC toolset and a more sophisticated reconstruction of points in 3D space.

##### 7.3.9.1.4	Bitstream Generation, output

The MPEG V-PCC test model is used to encode and decode test sequences as described previously [Vol-26].

To compute metrics, the tool mpeg-pcc-mmetric [Vol-18] is used.

Python scripts will be provided to be able to:

-	Encode each sequence for each condition, rate and profile

-	Decode the corresponding sequence

-	Compute metrics

-	Generate tables and graphs

Below are examples of command lines for the basic profile for a vox11 sequence:

-	to encode a test sequence:

mpeg-pcc-tmc2/bin/PccAppEncoder \

--config=mpeg-pcc-tmc2/cfg/common/ctc-common.cfg \

--config=mpeg-pcc-tmc2/cfg/condition/ctc-random-access.cfg \

--config=mpeg-pcc-tmc2/cfg/sequence/${test_sequence}.cfg \

--config=mpeg-pcc-tmc2/cfg/rate/ctc-r1.cfg \

--configurationFolder=mpeg-pcc-tmc2/cfg/ \

--uncompressedDataFolder=${source_sequence}/ \

--compressedStreamPath=${test_sequence}.bin \

--normalDataPath=${source_sequence}/${source_sequence}_%04d.ply \

--frameCount=32 \

--resolution=2047 \

--mapCountMinus1=0 \

--profileToolsetIdc=0 \

--profileReconstructionIdc=0

-	to decode a test sequence:

mpeg-pcc-tmc2/bin/PccAppDecoder \

--startFrameNumber=0 \

--compressedStreamPath=${test_sequence}.bin \

--reconstructedDataPath=${test_sequence}_dec_%04d.ply \

--inverseColorSpaceConversionConfig=mpeg-pcc-tmc2/cfg/hdrconvert/yuv420toyuv444_16bit.cfg

-	to compute metrics of a test sequence:

mpeg-pcc-mmetric/build/Release/bin/mm sequence \

--firstFrame 0 \

--lastFrame 31 END \

compare --mode pcc \

--inputModelA source_sequence_%04d.ply \

--inputModelB test_sequence_dec_%04d.ply END \

compare --mode pcqm \

--inputModelA source_sequence_%04d.ply \

--inputModelB test_sequence_dec_%04d.ply

For each test, outputs are:

-	Bitstream file

-	Log files containing metrics information

- Encoder log output

-	 Decoder log output

- Metric log output

A CSV file containing concatenated metrics information for each condition and one profile is generated for all sequences and rates.

The following information will be stored:

-	SeqId: identifier of the sequence

-	CondId: tested condition (RA)

-	RateId: tested rate number [R1..R5]

-	nbFrame: number of tested frames

-	NbInputPoints: number of points in the source sequence

-	NbOutputPoints: number of points in the candidate test sequence

-	MeanOutputPoints: mean number of points in the candidate test sequence

-	MeanDuplicatePoints: mean number of duplicated points (with same geometry) in the candidate test sequence

-	TotalBitstreamBits: size of the bistream in bits

-	geometryBits: size of the geometry stream in bits

-	metadataBits: size of the metadata stream in bits

-	attributeBits: size of the attribute stream in bits

-	D1Mean: mseF,PSNR (p2point)

-	D2Mean: mseF,PSNR (p2plane)

-	LumaMean: c[0],PSNRF

-	CbMean: c[1],PSNRF

-	CrMean: c[2],PSNRF

-	PCQM: PCQM PSNR

-	SelfEncoderRuntime: encoder time for current process

-	ChildEncoderRuntime: encoder time for child processes

-	SelfDecoderRuntime: decoder time for current process

-	ChildDecoderRuntime: decoder time for child processes

From this CSV file, an excel spreadsheet is generated to get tables and graphs for interpretation of the results.

##### 7.3.9.1.5	Videos Generation for subjective tests

The representative renderer [Vol-19] is used to generate the videos for the subjective test.

To avoid interference between the background and the test material, a neutral background with a color will be selected. A floor makes the rendered scene more realistic by preventing interference with the test material.

A script will be provided to generate videos with chosen camera path.

For each sequence, a video is generated. A camera path and blend parameter options adapted to each sequence is provided [Vol-30]. The output of this video generation with a camera path is stored as high-quality video sequence of a length as close as possible to 10s.

The generation is done into two steps: first the generation of the RBG raw file with the camera path and then the conversion into YUV or MP4 files.

Here is an example of the two command lines:

1-To generate a RGB raw file with a specific camera path

./PccAppRenderer.exe -d ${test_sequence}\

-i 1 \

-n 32 \

--play=1 \

--playBackward=1 \

--height=1080 --width=1920 \

--type=3 --alphaFalloff=6 --size=3.2 --blendMode=0 \

--floor=1 --overlay=0 \

--camera=${camera_path}.txt

--RgbFile=${test_sequence_rgb}

2-To convert the RGB file to x265 lossless mp4 file

./scripts/convert_video.sh \

-o ${test_sequence}_x265lossless.mp4  \

--videoType=2

--ffmpeg=ffmpeg.exe

-i ${test_sequence_rgb}.rgb

The video sequences are generated with the following video parameters:

-	Video resolution: progressive uncompressed full-range HD format (1920x1080). Note that upsampling by the TV set should be avoided

-	Frame rate: The frame rate will be aligned with the frame rate in the test data set

-	Color space: ITU-R BT.709

-	Sub-sampling: 4:2:0 YUV 10 bits or x265 fast preset lossless

#### 7.3.9.2	V-DMC

Editor’s note: Testing of V-DMC will depend on availability of stable V-DMC specification (including profile to be tested), stable public test model and volunteers to contribute

##### 7.3.9.2.1	V-DMC test model and configuration files

##### 7.3.9.2.2	Rate points and test conditions

##### 7.3.9.2.3	Profiles

##### 7.3.9.2.4	Bitstream Generation, output

##### 7.3.9.2.5	Videos Generation for subjective tests

#### 7.3.9.3	Verification / crosschecks

All produced bitstreams, metric results and produced videos will be crosschecked by at least one other SA4 member to ensure that results are correct.

== CHANGE 8 ===

### 7.3.10	External Performance data

The subjective verification test report for V-PCC can be downloaded from the public MPEG website [Vol-20]

The Brazilian SBTVD Forum performed objective tests with V-PCC. Full results are available in chapter 6.10 (Candidate Technology I), 6.10.3.2 and 6.10.4 of the following document [Vol-31]

== CHANGE 9 ===

### 7.3.11	Additional information

Sequences can be decoded and visualized in real time using a 3D background or in Augmented Reality on a smartphone, tablet, head-mounted display using DASH streaming mode or local file system.

Nokia’s real-time V-PCC decoder implementation that was released as open source: https://github.com/nokiatech/vpcc

A simple scene description could be added to enable the placement of the asset in the scene (position, orientation, scale…) but is outside the scope of this document, which is focused on the format and codec evaluation.

Editor’s note: Add information for V-PCC on 5G-MAG