3GPP TSG-SA4 Meeting #131	S4-

# Geneva, Switzerland, 17th Feb – 21st Feb 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR |  | rev |  | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Aspects to look into during future study on topic of multi-access media delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co. Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21. 900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Rel-19 study on FS_AMD is coming to an end. Clause 5.18 specifies study objectives, key issues and candidate solutions on multi-access media delivery. Some study aspects weren’t addressed sufficiently during this study. This contribution proposes FFS text so any future study in this area can look into these aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add couple of gaps related to impact of multi-access media delivery on dynamic policy, network assistance, and network slicing procedures, and add some recommendation text for future work. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature for mullti-access media delivery will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5, 5.18.7, 6.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.18.5	Gap analysis and requirements

#### 5.18.5.1	Multi-access downlink media streaming using CMMF

#### 5.18.5.2	Multi-Access media delivery using ATSSS

##### 5.18.5.2.1	Gaps in multi-access delivery path management

The following potential open issues are identified in the specific case where multipath delivery is not transparent to the 5GMS-Aware Application:

- There is no specification related to informing the 5GMS-Aware Application of multiple paths when the UE is using ATSSS-based multipath media delivery.

##### 5.18.5.2.2	Gaps in Dynamic Policy procedures

Existing procedures for Dynamic Policies do not allow the 5GMS Client to request policy treatment over a specifc access network if the M4 flows are exchanged using multiple access networks. This is due to the fact that the currently specified data model parameters for dynamic policy procedures (as described in clause 5.18.1.4 of the present document) do not allow for identification of specific paths if multiple paths are possible between the UE and the 5GMS AS. Specifically, the sourceAddress and destinationAddress parameters of IPPacketFilterSet used in the Application‌Flow‌Description type (as described in clause 5.18.1.4 of this present document) both use the IP addresses of the Multi-Access PDU Session (as described in clause 5.18.1.3.1 of present document) assigned to the UE and UPF Steering Functionalities.

For identification of specific path, it is required that either the MPTCP link-specific multipath addresses or the MPQUIC link-specific multipath addresses be used, but these addresses are not routable via N6 (see clause 5.18.1.3.1 of present document), so these IP addresses are not candidates for use during the Dynamic Policy instantiation request that is to be sent to the 5GMS AF over N6.

This is an issue if one of the paths is a problematic path in a multipath environment as described in clause 5.18.1.3.2 of the present document. However, according to clause 5.32.4 of TS 23.501 [23], traffic splitting for GBR QoS Flows is not supported. So, if M4 media flows are transported as GBR QoS Flows, then traffic splitting of M4 media flows using ATSSS is not supported in this release, and the study needs to be revisited in a future release.

NOTE:	The Rel-18 ATSSS specification does not support traffic splitting for GBR QoS Flows between 3GPP and Non-3GPP accesses. The ATSSS specification work in future releases is to be monitored, and referenced here when there is an update.

##### 5.18.5.2.3	Gaps in Network Assistance procedures

Clause 4.0.5 of TS 26.501 [15] describes high level procedure for Network assistance. As specified in this clause network assistance enables the 5GMS Client in the UE to interrogate or manipulate the network Quality of Service for an ongoing media streaming session. Clause 11.4 of TS 26.510 [108] specifies the Network Assistance client API and describes the two network assistance methods – bit rate recommendation request and delivery boost request. For both these requests, it is unclear if 5GMS Client is able to request, or receive, network assistance over a specific access when the application flows at reference point M4 are using the MA PDU Session using multiple access networks as described in clause 5.18.1.3 of the present document.

##### 5.18.5.2.4	Gaps with network slicing

As described in clause 5.18.1.3 of the present document, if the UE requests network slice instance with the MA PDU Session, the same S-NSSAI is allowed to span both access networks. Clause 5.15.19 of TS 23.501 [23] specifies support for Network Slice Replacement, which is a feature that is used to temporarily replace an S-NSSAI with an Alternative S-NSSAI when the S-NSSAI becomes unavailable or congested in the 5G Core network. There is no specification text in clause 5.15.19 of TS 23.501 [23] on the impact of Network Slice replacement for a MA PDU Session.

# * * * * Second change * * * *

### 5.18.7	Summary and conclusions

Multi-access media delivery enables media streaming applications to efficiently access content over multiple access networks. This Key Issue has examined existing specification relating to the ATSSS (Access Trafffic Steering Switching and Splitting) architecture in TS 23.501 [23] and TS 23.502 [24] to identify its impact on 5GMS. Topics relating to application awareness and influence on multi-access delivery, potential enhancements to dynamic policy feature of 5GMS to support multiple access paths, and network assistance with multi-access delivery have been studied. The Key Issue has documented collaboration scenarios and the mapping of the ATSSS architecture into the 5GMS architecture.

The MPTCP and MPQUIC link-specific multipath IP addresses are not routable via N6 as of current release, and therefore identification of specific paths for any 5G Media Streaming procedures is not supported. Further, traffic splitting for GBR QoS Flows is not supported. If M4 media flows are transported as GBR QoS Flows, then traffic splitting of M4 media flows using ATSSS is not supported in this release, and the study is to be revisited in a future release.

It is recommended that:

1.	An informative annex is added to TS 26.501 [23] documenting:

a.	A brief description of multi-access media delivery, based on clause 5.18.1 of the present document.

b.	The mapping of the ATSSS architecture into the 5GMS architecture, as described in clause 5.18.3.2 of the present document.

2.	Changes to the Configuration Settings API and to the Dynamic Status Information API as described in clause 5.18.6.2 of the present document are implemented in TS 26.510 [108] to allow for application configuration and status information exchange for multi-access media delivery.

3.	A future study on the topic of multi-access media delivery looks into the following aspects:

a.	Whether future specification work on ATSSS supports traffic splitting for GBR QoS Flows between 3GPP and non-3GPP accesses. If it is supported, study the impact on splitting M4 media flows if they are transported as GBR QoS Flows  as described in clause 5.18.5.2.2 of the present document.

b.	Impact of multi-access media delivery using ATSSS architecture on UE multi-path management, dynamic policy, network assistance, and network slicing procedures as described in clause 5.18.5.2 of the present document.

c.	Whether and how a closer alignment with the study on media delivery from multiple service endpoints/locations described in clause 5.19 of the present document is required

# * * * * Third change * * * *

## 6.18	Multi-access media delivery

Multi-access media delivery enables media streaming applications to efficiently access content over multiple access networks. This Key Issue has examined existing specification relating to the ATSSS (Access Traffic Steering, Switching and Splitting) architecture in TS 23.501 [23] and TS 23.502 [24] to identify its impact on 5GMS. Topics relating to application awareness and influence on multi-access delivery, potential enhancements to dynamic policy feature of 5GMS to support multiple access paths, and network assistance with multi-access delivery have been studied. The Key Issue has documented collaboration scenarios and the mapping of the ATSSS architecture into the 5GMS architecture.

Based on the conclusions, following are recommended for stage-2:

-	An informative annex is added to TS 26.501 [15] documenting:

a.	A brief description of multi-access media delivery, based on clause 5.18.1 of the present document.

b.	The mapping of the ATSSS architecture into the 5GMS architecture, as described in clause 5.18.3.2 of the present document.

Based on the conclusions, following are recommended for stage-3:

-	Changes to the Configuration Settings API and to the Dynamic Status Information API as described in clause 5.18.6.2 of the present document are implemented in TS 26.510 [108] to allow for application configuration and status information exchange for multi-access media delivery.

It is recommended that future specification work on ATSSS in TS 23.501 [23] and TS 23.502 [24] is to be monitored to further study traffic splitting of M4 flows across multiple access networks.

It is recommended that future study on this topic look into the impact of media delivery using ATSSS architecture on UE multi-access path management, dynamic policy, network assistance, and network slicing procedures as described in clause 5.18.7 of the present document.

# * * * * End changes * * * *