3GPP TSG-SA4 Meeting # 131	S4-250288

# Geneva, CH,  17 - 21 February 2025                                              (revision of S4-250094)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.113 | CR |  | rev | 1 | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on Joint and Blend shape Animation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AVATAR |  |  |  |  |  | Date: |  |  | 11-Feb-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Documentation of animation sample formats that can be used for animation of avatars are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.4.2 	Blend shape and Joint Animation

#### 6.4.2.1 	Overview

Avatar animation in communication services faces a significant challenge to reproduce a realistic representation of the user. At its core, the challenge lies in replicating the intricate movements of human expression and body mechanics in a way that feels natural and engaging. In this section, we refer to the ISO/IEC 23090-39:2025 specification as an example, which standardizes the format for these animation streams.

To understand why different animation techniques are necessary, consider how humans move and express themselves in the real world. Our faces create expressions through the intricate interplay of numerous muscles, each capable of subtle movements that combine to convey complex emotions. In contrast, our bodies move primarily through the coordination of skeletal joints and the surrounding tissues. These fundamental differences in movement mechanics necessitate two distinct approaches to animation: blend shape animation for facial expressions and linear blend skinning for body movement.

In the ARF specification, the animation formats that enable real-time transmission and animation of avatars is specified. The animation data is typically captured through frameworks like OpenXR.

Note: the ARF specification is still work in progress and the formats reported here may still be subject to change.

#### 6.4.2.2 	Blend Shape Animation for Facial Expressions

Blend shape animation works through creating intermediate forms by mixing different blend shapes. This process is governed by a mathematical formula that determines how each point on the face moves:

vout=v0+i=1nwi.vi-v0

This equation shows how to calculate the final position of each vertex on the face (vout). The animation operation starts with the neutral expression (v0) and add weighted contributions from each predefined expression (vi). The weights (wi) determine how much each expression contributes to the final result, much like adjusting the amount of each color when mixing paint.

In the ARF specification, the facial animation sample structured as follows:

| facial_animation_sample() { | Descriptor |
| --- | --- |
| timestamp | int(64) |
| blendshape_set_id | int(16) |
| confidence_present | int(1) |
| reserved | int(7) |
| num_blendshapes | int(16) |
| for(i=0;i<num_blendshapes;i++) { |  |
| blendshape_id | int(16) |
| weight | float(32) |
| if (confidence_present) { |  |
| confidence | float(32) |
| } |  |
| } |  |
| } |  |

Each animation sample represents a time instant, describing which expressions are active and how strongly they influence the final face shape. The confidence value provides a measure of certainty in the expression detection, allowing systems to handle uncertain inputs gracefully. This is particularly important in real-time applications where facial expressions might be captured from video input.

#### 6.4.2.3 	Linear Blend Skinning for Body Animation

Linear Blend Skinning (LBS) takes a different approach, more akin to a puppet's armature system. Just as a puppeteer controls a marionette through a system of joints and strings, LBS uses a skeletal structure to drive the movement of the avatar's outer surface. This approach naturally mirrors how our own bodies move, with skin and muscles following the underlying bone structure.

The mathematical foundation of LBS is expressed through the following equation:

vinew=i=1nwi,j.Mjglobal.vi

This formula describes how each point on the avatar's surface (vi) moves in response to the skeleton's movement. The transformation matrices (Mjglobal) represent how each joint has moved from its starting position, while the weights (wi,j) determine how much influence each joint has over each point on the surface. It's similar to how your skin moves differently depending on its proximity to different joints in your body.

In the ARF specification the joint animation sample is structured as follows:

| joint_animation_sample() { | Descriptor |
| --- | --- |
| timestamp | int(64) |
| joint_set_id | int(16) |
| velocity_present | int(1) |
| reserved | int(7) |
| num_joints | int(16) |
| for(i=0;i<num_joints;i++) { |  |
| location_matrix[16] | float(32) |
| if (velocity_present) { |  |
| velocity_matrix[16] | float(32) |
| } |  |
| } |  |
| } |  |

Each sample captures the position and orientation of every joint in the skeleton at a specific moment in time. The velocity information helps create smooth transitions between poses and can predict future positions, reducing the impact of network latency in real-time applications. This is particularly important for natural-looking movement in hands and eyes, which require especially precise control.

#### 6.4.2.4 	Carriage of animation samples

A potential approach for transmission of the animation samples to the other participants of an AR call is to pack the animation samples in an envelope message that can be carried over different communication channels, such as the IMS Data Channel. For this purpose, the data channel message envelope as defined in TS 26.565 is reused:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | An unique identifier of the message in the scope of the data channel session. |
| Type | string | 1..1 | A urn that identifies the message type. |
| Message | object | 1..1 | The message content depends on the message type. |
| sendingAtTime (ref. T1’) | number | 0..1 | The time when the split rendering metadata message is transmitted from the split rendering client to the split rendering server. |

The following message types may be used:

- urn:3gpp:avatar:v1:blend-shape for the blend shape animation sample

- urn:3gpp:avatar:v1:joint for the joint animation sample

For other animation samples, a dedicated message type needs to be defined.