3GPP TSG-SA WG4 Meeting #131	S4-250317

# CH, Geneva, 17 – 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0007 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5G_RTP_Ph2] PSI Guidelines for HEVC tiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_5G_RTP_Ph2 KI#13, the following aspect was concluded for the normative work: - - Consider extending the PSI guidelines in TS 26.522 [2] for the case when a PDU Set is defined as a tile (as opposed to a video frame or slice). - - Coordinate with SA2 and RAN2 on potential benefits of signaling PDU Set type to the 5G network. Based on this, an objective was added to the 5G_RTP_Ph2 WID: 9. Conduct normative work on guidelines for marking PDU Sets that are not defined as video frames or slices and potentially signalling PDU Set type to the 5G network. NOTE: This objective requires coordination with SA2. TS 26.522 generally assumes that video PDU Sets (H.264/AVC, H.265/HEVC) either carry frames or slices. However, the general definition of a PDU Set is not limited to these constructs, and it may be beneficial for the application to define PDU Sets differently. In addition to frames and slices, video PDU Sets can be defined as other sub-picture constructs such as H.265/HEVC tiles. Such PDU Sets are particularly relevant for immersive media use cases, where it could be beneficial to assign different importance to different regions of a picture, for example depending on content saliency and/or user’s viewing direction, as in the examples of 360-degree and volumetric video streaming. This requires a dynamic spatial adaptation of the PDU Set Importance (PSI) based on user’s viewport and/or other content-related metadata. Depending on the selected criteria (e.g., the user’s actual viewport), the sender can dynamically modify the PSI for each tile. Alternatively, the sender may consider a group of tiles as a PDU Set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide guidelines on setting PSI for HEVC tiles |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No PSI guidelines for HEVC tiles |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev3 [S4-250052]: Addressed the comments received at the RTC SWG Telco on Feb 5 regarding the provided example and the offline comments received after the telco. Rev2 [S4aR250045]: Provided an example and different adaptation criteria. Rev1 [S4aR250031]: Removed the signaling of PDU Set type to the 5G network. |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-T Rec H.264 (08/2021): "Advanced video coding for generic audiovisual services" | ISO/IEC 14496-10:2022: "Information technology – Coding of audio-visual objects – Part 10: Advanced Video Coding".

[3]	ITU-T Rec H.265 (08/2021): "High efficiency video coding" | ISO/IEC 23008-2:2023: "High Efficiency Coding and Media Delivery in Heterogeneous Environments – Part 2: High Efficiency Video Coding".

[4]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[5]	IETF RFC 6184 (2011): "RTP Payload Format for H.264 Video", Y.-K. Wang, R. Even, T. Kristensen, R. Jesup.

[6]	IETF RFC 7798 (2016): "RTP Payload Format for High Efficiency Video Coding (HEVC)", Y.-K. Wang, Y. Sanchez, T. Schierl, S. Wenger, M. M. Hannuksela.

[7]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[8]	3GPP TR 26.998: "Support of 5G glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[9]	IETF RFC 768 (1980): "User Datagram Protocol", J. Postel.

[10]	IETF RFC 5761 (2010): "Multiplexing RTP Data and Control Packets on a Single Port", C. Perkins, M. Westerlund.

[11]	IETF RFC 8285 (2017): "A General Mechanism for RTP Header Extensions", D. Singer, H. Desineni, R. Even.

[12]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[13]	IETF RFC 5905 (2010): "Network Time Protocol Version 4: Protocol and Algorithms Specification”, D. Mills, J. Martin, J. Burbank, W. Kasch.

[14]	IEEE 1588-2019 – IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems, June 2020.

[15]	IETF RFC 4574 (2006): "The Session Description Protocol (SDP) Label Attribute", O. Levin, G. Camarillo.

[16]	IETF RFC 3611 (2003): "RTP Control Protocol Extended Reports (RTCP XR)", T. Friedman, R. Caceres, A. Clark.

[17]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[18]	IETF RFC 7656 (2015): "A Taxonomy of Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources ", J. Lennox, K. Gross, S. Nandakumar, G. Salgueiro, B. Burman.

[19]	IETF RFC 5888 “The Session Description Protocol (SDP) Grouping Framework”, G. Camarillo et al.

[20]	ISO/IEC 60559:2020: “Floating-point arithmetic”.

[21]	3GPP TR 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

# * * * * Second change * * * *

### 4.2.6	Guidelines for PDU Set Marking

#### 4.2.6.2	PDU Set Importance Field

##### 4.2.6.2.6	PSI Guidelines for H.265/HEVC tiles

When using tiled H.265/HEVC encoding, an RTP sender may define a PDU Set as a collection of one or more HEVC tiles, where each PDU Set comprises all PDUs carrying the data for these tiles. This definition is particularly relevant for immersive media use cases, as it enables assignment of different PSI values to distinct regions of a picture. Moreover, PSI values could be dynamically adjusted based on factors such as content saliency and the user’s viewing direction, which is especially beneficial for applications like 360-degree video and volumetric video.

When HEVC tiles are used, a PDU Set should be defined as a Motion Constrained Tile Set (MCTS) to enable independent decoding of an arbitrarily selected set of tiles at the receiver.

NOTE:	In HEVC, intra prediction process for each tile is independen from other tiles within the same picture. However, in terms of inter-picture prediction, motion vectors for each tile can point beyond tile boundaries in the reference pictures. The MCTS technique in HEVC constrains the inter-picture prediction process within a specified set of tiles to reference only regions within the same set of tiles in previous pictures in decoding process. When MCTS is used, the tiling grid of the picture is required to be constant within a coded video sequence. An MCTS can consist of one or more HEVC tiles.

An RTP sender may dynamically modify the PSI for each MCTS over time based on specific criteria. For example, it could assign lower PSI to PDU Sets within the the receiver’s (actual and/or predicted) viewport or pose (e.g. received via RTCP viewport feedback as defined in TS 26.114 [21], Annex Y), thereby prioritizing the regions currently viewed by the user and those likely to be based on the viewport prediction. The PSI adaptation criteria may also include eye gaze or other content/scene-related metadata such as content saliency.

For example, a 360-degree video sender could determine which spatial regions are likely to fall into the receiver’s viewport based on the receiver’s actual and/or predicted pose and assign lower PSI values to PDU Sets covering regions that are more likely to fall into the receiver’s viewport.

An example scenario demonstrating dynamic adaptation of PSI is shown in Figure 4.2.6.2.6-1. In the example, it is assumed that an MCTS comprises a single HEVC tile. However, the operation can be generalized to apply to other cases.

At time t=t1, the sender assigns PSI=3 to the HEVC tiles that are predicted to be fully within the receiver’s viewport at t=t2, and PSI=5 to the HEVC tiles that are predicted to be partially within the receiver’s viewport at t=t2. Background tiles (i.e., tiles that are predicted to be outside the viewport at t=t2) are assigned PSI=10.

Note that the PSI values in the example are chosen arbitrarily and the actual configuration depends on the application requirements. The sender could also assign PSI in an even more fine-granular way by considering the percentage of a tile covered by the receiver’s predicted viewport.

Figure 4.2.6.2.6-1: Example scenario for dynamic adaptation of PSI