3GPP TSG-SA4 Meeting #131	S4-250395

# Geneva, Swizerland , 17th Feb 2025 - 21st Feb 2025				 In revision of S4-250350

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.522 | CR | 0008 | rev | 6 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Guidelines for PDU Set Marking of Multiplexed Streams |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_RTP_Ph2 |  |  |  |  |  | Date: |  |  | 20-02-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 26.822 conclusions in 7.10 states: Furthermore, it is recommended to add guidelines to TS 26.522 [2] for RTP senders that use multiplexing. This recommendation is also included in the 5g RTP Phase 2 work item objectives. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a guideline on how to use RTP HE in case of multiplexed RTP streams |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RTP HE not that useful for majority of cases with multiplexed RTP Streams, work item objectives are not met |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.6 (new sub clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 addressing comments from qualcomm and interdigital, clarifying the case of RTP HE in RTCP and the RTCP PT versus RTP PT Add MPEG-2 TS as example but not the full reference Rev 2Addressed comments during february 5 call Rev 3: editorial improvements Rev 4: remove the protocol description example as it may not be needed as the UPF will not apply specific operations Rev 5: change to all changemarks improve the header of the CR Rev 6: some edits and remove mentions of term lone |  |  |  |  |  |  |  |  |

*** CHANGE (new clause all new text)***

#### 4.2.6.X Guidelines for multiplexed content

An RTP sender could also include RTP HE for PDU Set marking in case of multiplexed streams.

One possibility is RTP multiplexing when different RTP Streams exist (e.g. audio + video).

Another possibility is RTP multiplexing when different RTP Streams and RTCP packets are present.

Another possibility is a multiplex in which RTP packets may contain different media types and in addition RTCP packets can be present (e.g. MPEG-2 TS over RTP and using RTCP).

Also cases may exist with multiple video streams.

To illustrate this, Table 4.2.6.X-1 provides some examples on different multiplexing scenarios and the corresponding guidelines for setting RTP HE are further given in Table 4.2.6.X-2.

The description of each scenario is given and the implication for RTP HE marking in the Tables.

Table 4.2.6.X-1: Example of Multiplexing scenarios

| Scenario | Multiplex Type | Description | Implications for RTP HE for PDU Set Marking for sender |
| --- | --- | --- | --- |
| sc1 | audio + video RTP multiplex | Native audio and video streams are carried in separate RTP streams with different SSRC, and different PT Packets contain either audio or video. | Typically, RTP HE is used for the video stream, audio packets can be unmarked. If both audio and video RTP packets are marked, the RTP HE for PDU Set marking is usually applied to video and audio RTP streams separately. RTP video packets and audio packets are usually marked as separate PDU Sets, not as part of the same PDU Set. |
| sc2 | audio + video , RTCP | Same as sc1, but in this case also RTCP packets exist. Packets contain audio, video or RTCP. | Same as sc1 for audio and video with the following addition. RTP HE cannot be used for RTCP packets, and these are handled as unmarked PDUs. (End of Data Burst signal cannot be used in case RTCP packet is the last one in a data burst). |
| sc3 | audio, video native multiplex | Stream packets can contain both audio and video. In addition, packets can also contain other metadata related to the streams. | In this case, PDU Sets can contain different media types (e.g. MPEG-2 TS over RTP); additional guidance is provided to handle this case in Table 4.2.6.X-2. (In MPEG-2 TS over RTP packets usually 6-7 188 byte transport stream packets are carried in an RTP packet that can contain different media in each TS packet to be identified based on the packet identifier) |
| sc4 | audio, video native multiplex + RTCP | same as sc3 adding RTCP | Same as sc3 including RTCP packets [4] that cannot carry RTP HE and are therefore left unmarked. |
| sc5 | video + video or audio + audio | Similar to sc1, but multiple native audio or multiple native video streams are carried in separate RTP streams with different SSRC, either with different PT field or sharing same PT field. Packets contain content from a single SSRC. | Packets from different RTP streams are marked as separate PDU Sets, ensuring that each PDU Set contains packets only from one RTP stream (single SSRC). |
| sc6 | video + video or audio + audio + RTCP | Same as sc5 adding RTCP | Same as sc5 including RTCP packets [4] that cannot carry RTP Header Extension |

Table 4.2.6.X-2: Guidelines for applying RTP HE in different example multiplexing scenarios

| Scenario | Guideline | Additional Comments |  |
| --- | --- | --- | --- |
| sc1 | Video PDU Sets should be assigned e.g. for video frames/slices and PSI may be set using the guidelines from clause 4.6.2. Audio packets may be unmarked, or in case audio frames consist of multiple packets they may also be marked using RTP HE. PSI of the unmarked packet is determined by the 5G System, based on a configuration, and this can also be based on the payload type. |  | Typically, RTP HE is used for the video stream, audio packets can be unmarked as frames are often a single packet and the marking is not beneficial. In such case only overhead is introduced (see the unmarked PDU case), or the RTP HE can also be used for the audio stream. |  |
| sc2 | Same as sc1. RTCP packets can not be marked using RTP HE (there is no RTP HE for RTCP) and are treated as unmarked packet in the 5G System. PSI should be determined by the 5G System. |  | Same as sc1 for audio and video. End of Data burst signal may not be valid if RTCP is the last packet in a burst as no RTP HE can be added to an RTCP packet. |  |
| sc3 | PDU Sets may be identified by the RTP sender based on the presentation time and the RTP HE can be used to support the PDU Set based QoS handling. PSI may be set to a preconfigured value or the value corresponding to the importance of the most important part of the multiplexed stream using the guidelines from clause 4.6.2. |  | In this case, the grouping of PDU sets will contain different media types, and therefore the guidance cannot only be based on one specific media type. Therefore, PDU Sets could be identified and marked by the RTP sender based on other aspects such as the presentation time. The PSI can be set based on a configuration. |  |  |
| sc4 | Same as sc3 RTCP packets can not be marked and are treated as unmarked packet in the 5G System. | Same as sc3 including RTCP packets that cannot carry the RTP Header Extension. Data burst signal cannot be used if RTCP is the last packet in a burst. |  |  |  |
| sc5 | Video PDU Sets may be assigned e.g. for video frames or slices and PSI may be set using the guidelines from clause 4.6.2 for video, separating RTP streams into separate PDU Sets. Audio Packets can be unmarked or in case audio frames consist of multiple packets they may be marked using RTP HE, separating RTP streams into separate PDU Sets. | Multiple PDU Sets can be "open" at the same time, i.e., some PDUs are received from multiple different SSRC (i.e. different RTP streams), which requires the marking to keep track of multiple simultaneous PDU Set contexts. |  |  |  |
| sc6 | Same as sc5. RTCP are treated as unmarked packet in the 5G System. PDU Set importance can be determined by the 5G system. | Same as sc5 including RTCP packets [4] that cannot carry the RTP Header Extension and need not be marked. |  |  |  |

To support multiplexed content in combination with PDU Set QoS based handling in the 5G System, groups of packets of different media types (audio, video) but same payload type (native multiplex) can also be grouped as a PDU Set (sc3, sc4). This enables groups of packets to benefit from transfer using PDU Set QoS parameters in NG-RAN (PSDB, PSER, PSIHI). In this case, each of the RTP packets can set the RTP HE for PDU Set Marking to enable 5G System to identify corresponding PDU Sets.

To summarize, different options exist when applying RTP HE for PDU Set marking for multiplexed content, for which some guidelines are defined as follows:

-	When RTP multiplexing (sc1, sc2, sc5 and sc6) is used, it is possible to separately mark the PDU Sets in different streams.

-	When packets combine different media types in a payload type such as in sc3 and sc4, PDU sets can be created around a common media presentation time grouping packets based on timestamps. In this case the PDU set importance can be set to a derived or default value or a value configured.

-	In case only packets of single stream are marked (e.g. the video stream), and the packets of other streams are unmarked. In this case the 5G System may still identify PDU set information as detailed in Annex A based on the payload information for example or the payload type.

-	In case packets cannot carry an RTP HE (e.g. RTCP packet), packets can be handled as unmarked PDU and PDU Set information may still be derived in the 5G system in some cases.