3GPP TSG-SA4 Meeting #132	S4-250778

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0013 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RTP Header Extension for Processing Information for IVAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It may be beneficial to use RTP header extension to convey Processing Information (PI) for IVAS. There has been a proposal S4-250652 that proposes an RTP header extension for each of the 20 PI frame types. This causes the one-byte RTP header extension not usable because the RTP header extension ID field can support only 14 RTP header extension elements. Additionally, this approach leads to excessive IANA registration and SDP signaling overhead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes a single RTP header extension for PI for IVAS, and the RTP header extension has a hierachical structure to accommodate the large number of PI frame types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no implementation friendly RTP header extension for PI frames for IVAS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE

# A.5	RTP Header Extension

## A.5.1	General

Besides transmitting Processing Information (PI) frames as part of the IVAS RTP payload, PI frames may be transmitted in the RTP header extension. The RTP header extension posesses a hiearchical architecture to accommodate a large number of PI frame types.

## A.5.2	Hierarchical RTP Header Extension for PI

The payload of the the RTP header extension element for PI has two portions: a summary portion that specifies which PI frame types are present, and a portion for the PI frame types whose presence is specified by the summary portion.

The summary portion consists of 32 bits: the first 31 bits indicate the presence of the PI frames of the 5-bit type bits in A.3.5.5.

## A.5.3	SDP Signaling

The ABNF syntax for the extmap attribute for the signaling of RTP HE for PI is defined as follows, extending the ABNF syntax in RFC 8285:

extensionname = "urn: urn:3gpp:ivas:pi"

extensionattributes = *1(format)

format = "short" / "long"

The extension attribute indicates whether the RTP header extension follows the one-byte format (“short”) or the two-byte format (“long”). When the extension attribute is absent, the format takes the default value “short”.

## A.5.3	Example

0                  1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       0xBE    |    0xDE       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         ID    |     L=4+8+8   |1|0|1|0|0|0|0|0|0|0|0|0|0|0|0|0|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|0|0|0|0|0|0|0|0|0|0|0|0|0|0|0|0|  0th PI frame type (8 bytes) …

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

…

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

….             |  2nd PI frame type (8 bytes)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

…

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

….             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure A.5.3-1 shows an example of the RTP header extension in the two-byte format. The 32-bit summary field indicates the presence of the zeroth PI frame type (with type bits 00000) and the second PI frame type (with type bits 00010), and the two PI frame types follow the summary field.

Figure A.5.3-1: an exmaple RTP header extension for PI for IVAS

Below is an SDP signaling example:

a=extmap:5 urn: urn:3gpp:ivas:pi long