Source:	Bytedance

Title:	Recordings of smartphone-like target device

Document for:	Discussion & Agreement

Agenda Item:	7.6 – DaCAS

# 1. Introduction

For obtaining characterization and development recordings for the DaCAS target devices, necessary recording scenarios are needed. In this contribution we provide the recordings two recording scenarios documented in S4-250654 [1], using the target device definition documented in S4-250590 [2].

# 2. Recordings

Following recording of scenarios 1 and 2 described in [1] are proposed to be included to the common database of DaCAS standardization, which can be used for developing and testing example solution for the target device documented in S4-250590 [2].

## 2.1 Recording scenario 1

Following recording scenario 1 definition in S4-250654 [1], we recorded signal with following properties.

| Sound source: | High quality loudspeaker |
| --- | --- |
| Source signal: | 1 kHz sine |
| Signal characteristics: | Frequency: 1 kHz Length: 10 s, with 1s of silence before and after the sine tone, results in total 12s Level: -0.1 dBFS (as long as calibrated SPL level is met) Sample Rate: 48 kHz Bit depth: 16-bit |
| Sound source calibration: | Source signal calibrated to 94 dB SPL at the measured microphone inlet position |
| Acoustic environment: | BEM simulation in free-field condition, with noise floor equals 10dBA (complying with NR15). |
| Detailed Positioning: | Distance: >0.5 m Source direction: Main diaphragm towards the measured microphone inlet Source angles (azi, ele): (0°,0°) relative to the measured microphone inlet |
| Reference recording: | Record 30 seconds of silence with the device under test to obtain the noise floor of the integrated microphones |
| Description/additional info: | SNR= 71.99 dB Microphone sensitivity S=-45.13 dBFS @ 94 dBA SPL |

Table 1. Recording for scenario 1

We stored the recorded signal to scenario_n1/scenario_1.wav, and the reference signal to scenario_n1/scenario_1_ref.wav.

## 2.2 Recording scenario 2

Following recording scenario 2 definition in S4-250654 [1], we recorded signal with following properties.

| Sound source: | High quality loudspeaker |
| --- | --- |
| Source signal: | Exponential sweep |
| Signal characteristics: | Range: 20 Hz – 20 kHz Length: 5 s Level: -3 dBFS Sample Rate: 48 kHz Bit depth: 16-bit |
| Sound source calibration: | Loudspeaker is calibrated with -3 dBFS pink noise to produce 80 dBA at the UE position. Same playback settings are then used with the defined test signal |
| Acoustic environment: | BEM simulation in free-field condition, with noise floor equals 10dBA (complying with NR15). |
| Detailed Positioning: | Distance: dUE=1.5m Source direction: Main diaphragm towards the UE Source angles (azi, ele): Required sound source directions: - Every azimuth f with maximum step of 10°: f = [0°, 10°, …,350°] - All azimuth directions at the elevation = 0° shall be always recorded - For every 3rd azimuth step (f = [0°, 30°, …, 330°]), elevations q with maximum step of 30°: q = [-60°, -30°, 0°, 30°, 60°] - Poles: = [-90°, 90°] - In practice, actual elevation direction may differ slightly (e.g., due to recording facility limitations) - Applied source directions shall be clearly documented Space between the walls and sound source/UE should be at least 0.5 m |
| Reference recording: | Free-field measurement microphone towards the loudspeaker at the UE position |
| Description/additional info: | Silence before and after the measured direction shall be at least 1 s |

Table 2. Recording for scenario 2

We stored the recorded signal to scenario_n2/ scenario_2_azi=xxx_elev=yyy.wav (xxx and yyy are defined source angles in scenario 2), and the reference signal to scenario_n2/scenario_2_ref.wav.

# 3. BEM and simulation of mic non-ideality

As previously agreed in S4-250654 [1], FEM/BEM is allowed to produce free-field recordings as long as the non-ideality of microphone is taken into account. Given the tight time frame of DaCAS work, we decide to produce recording of S4-250590 [2] using BEM with simulation of mic non-ideality based on measurement of the original target device.

3.1 BEM simulation

Based on the phone and camera dimensions defined in S4-250590 [2], we created the triangle mesh of it. We then treat this triangle mesh as boundary, then simulated all IRs from required directions. Those IRs are then used to produce “clean” recording of scenario 1 and 2 by convolving the synthesized source signal of them.

3.2 Simulation of mic non-ideality

After getting the “clean” recording from BEM simulation, we then post process the clean signal through a pipeline that simulates non-idealities caused by microphones installed onto the smart phone target device. Our post-processing pipeline looks like this:

Figure 1. Post-processing pipeline to simulate non-idealities of installed microphones.

in which:

- Room noise floor level: Apply attenuation to pink noise according to target environment SNR.

- Mic sensitivity: Apply attenuation to noise and simulated audio signal according to target microphone sensitivity.

- Radio noise simulation filter: A FIR to the white noise, with its gain around 10~15 dB, and frequency range within 213~219 Hz

- Mic capsule electric SNR: Apply attenuation to filtered white noise according to target microphone electric SNR.

- Mic port EQ: The frequency-dependent gain/attenuation that mimics the acoustic effect of microphone port and inner cavity. We simulate this EQ curve as a 2nd-order peak EQ, with its resonance frequency calculated by Helmholtz Resonator [3].

- Mic capsule EQ: The frequency-dependent gain/attenuation that mimics the frequency response of a bare mic capsule. We simulated this EQ curve considering the f0 and fh of mic capsules, implemented as cascaded highpass and lowpass filters. Q of the f0 highpass is set to be 0.707 to create a flat passband, and Q of the fh lowpass filter is > 1 to create a roughly +10dB bump at fh.

# 4. Proposal

Based on the presented discussion, we propose to include recordings of scenarios 1 and recording 2 to the common database of DaCAS standardization, for the target device definition documented in S4-250590 [2].

# References

[1]	S4-250654: “On DaCAS recording scenarios”, Nokia.

[2]	S4-250590: “[DaCAS] Propose a target device”, Bytedance.

[3]	Helmholtz resonance: https://en.wikipedia.org/wiki/Helmholtz_resonance