Source:	Xiaomi Technology Spain S.L

Title:	Exponential sweep database for Four-Microphone Smartphone device 1

## Document for:	Discussion & Agreement

## Agenda Item:	7.6

# Introduction

New recording scenarios have been added based on the contributions and agreements from the SA4#131-bis-e meeting[1]. This contribution includes updates for the sweep signal recording scenarios. The corresponding recording signal for target device in [2] is attached in the file 'sweep.zip'.

# Changes to recording scenario 2

| No | 2 |
| --- | --- |
| Sound source | High quality loudspeaker |
| Source signal | Exponential sweep |
| Signal characteristics | Range: 20 Hz – 20 kHz Length: ≥5 s Level: -18 dBFS RMS |
| Sound source calibration | Loudspeaker is calibrated with -18 dBFS RMS pink noise to produce 70 dBSPL (A) at the UE position. Same playback settings are then used with the defined test signal |
| Acoustic environment | Anechoic chamber, free-field properties expected down to 200 Hz, Noise floor should comply with NR15. |
| Detailed Positioning | Distance: dUE ≥0.5m |

## Reason

Length: ≥5 s

- Slow frequency sweeps maintain steady-state resolution curves

Level: -18 dBFS RMS

- Avoid clip since -3dBFS RMS pink noise is larger than 0dBFS. Refer to the the setup in TS 26.260

- In response to the comment from the last post-meeting, the sample rate and bit depth also be included in the database template. The detailed setup for playback will be described in the "Description/Additional Info" section of the database template, taking into account target device.

Distance: dUE ≥0.5m

- For larger speaker with better high frequency performance.

# Proposed database

The exponential sweep recording scenario demands a significant amount of time. Initially, we capture signals across the horizontal plane (azimuth 0° to 350° in 10° steps at 0° elevation) on horizontal orientation. The remaining data will be addressed in subsequent meetings.

## Database information

| File name | e.g. SWEEP_Horizontal_0deg-01.wav |
| --- | --- |
| UE orientation / placement | The UE is on the same plane as speaker, with the UE's long edge is towards the ceiling and floor, and the screen is towards straight ahead at azi 0°. |
| Sample rate | 48kHz |
| Bit depth | 16bit |
| Audio format | .wav |
| Rec Duration | Overall length 12s, first and last 1s is silence |
| Number of channels and microphone mapping: | 4 channels, ch1=mic1, ch2=mic2, ch3=mic3, ch4=mic4 |
| Description/additional info | Source distance: d_UE = 3m The recordings cover 36 angelse (ele 0°, azi 0°~360°, step 10°) in total. Source signal:Exponential sweep signal Frequency range: 20Hz-20kHz Length: 10s Level: -18 dBFS Sample rate: 96 kHz Bit depth: 24bit Fade-in and fade-out be implemented. |

# Conclusion

This document proposes changes to Recording Scenario 2 for approval. And outlines the horizontal plane sweep signals for Target Device 1. We are seeking feedback on this configuration. The remaining angles will be revised based on the final agreement and submitted at the next meeting.

## References

- S4-250654 On DaCAS recording scenarios_r2

- S4aA250006: “Proposal of a target device”, Beijing Xiaomi Mobile Software