| 3GPP TS 26.533 V0.0.1 (2025-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Diverse audio Capturing System for UEs(DaCAS) (Release 19 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	UE concepts and device types	7

4.1	UE concepts	7

4.1.1	General	7

4.1.2	UE Device	7

4.1.3	Compensation on microphone signals	7

4.2.1	Device type 1: Four-Microphone Prototype device	7

4.2.1.1	General	7

4.2.1.2	Detailed parameters	7

4.2.1.3	Device electro-acoustical characteristics	9

4.2.2	Device type 2: Three-Microphone Smartphone device	9

4.2.2.1	General	9

4.2.2.2	Detailed parameters	9

4.2.2.3	Device electro-acoustical characteristics	11

4.2.3.1	General	11

4.2.3.2	Detailed parameters	11

4.2.3.3	Device electro-acoustical characteristics	12

4.2.4.1	General	12

4.2.4.2	Detailed parameters	12

4.2.4.3	The differences between the Four-Microphone Smartphone device 1 and Four-Microphone Smartphone device 2	13

4.2.4.4	Device electro-acoustical characteristics	13

4.2.5.1	General	13

4.2.5.2	Detailed parameters	13

4.2.5.3	Device electro-acoustical characteristics	15

5	High-level technical description.	15

5.1	Input interfaces	15

5.2	Processing procedure.	15

6	Detailed algorithm description	15

6.1	General	15

6.2	Detailed algorithm of example solution 1	15

6.3	Detailed algorithm of example solution 2	15

7	Test reports of example solutions	16

7.1	Test setup	16

7.2	Report of example solution 1	16

7.2	Report of example solution 2	16

8	The adaptability of example solutions	16

8.1	General	16

8.2	 Adaptability of example solution 1	16

8.2	 Adaptability of example solution 1	16

Annex <A> (informative): Change history	17

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This clause shall start on a new page.

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	UE concepts and device types

## 4.1	UE concepts

## 4.1.1	General

UEs are the target devices for immersive audio capture, proposals highlight the need for devices that enable spatial audio processing while addressing legacy hardware constraints. Key goals include standardizing documentation (e.g., coordinate systems for microphone placement) and accommodating diverse form factors. Two primary device categories emerge: analytical prototypes for free-field modeling and practical designs mimicking mainstream smartphones.

## 4.1.2	UE Device

TBD

### 4.1.3	Compensation on microphone signals

#### 4.1.3.1	General

TBD

#### 4.1.3.2	Compensation processing

TBD

## 4.2	Device Types

### 4.2.1	Device type 1: Four-Microphone Prototype device

#### 4.2.1.1	General

This target device type includes two hardware configurations, the first hardware is a four-microphone prototype devices A which is an idealized free-field approximation device, and the second hardware is a four-microphone prototype devices B which is a practical smartphone design device.

#### 4.2.1.2	Detailed parameters

The parameters of the four-microphone prototype device A are as follows:

- Cylinder shape

  - Length 162 mm

  - Width 12 mm (cylinder diameter)

- With a protruding connector bump (on side corresponding to “Bottom view” in Figure 1)

  - Width 20 mm

  - Height 6 mm

  - Protrudes by 3.95 mm

  - HDMI-D Connector

- 4 microphone ports designated

  - Mic Front Left and Mic Front Right with a 140-mm spacing on the front

  - Mic Rear Left and Mic Rear Right with a 140-mm spacing on the rear

The brief schematic diagram of the device is in figure 4.2.1.2-1

Figure 4.2.1.2-1. – Four-microphone prototype device A

The parameters of the four-microphone prototype device B are as follows:

- Monoblock smartphone shape

  - Length 160.6 mm

  - Width 74.4 mm

  - Depth 8.6mm

- With a protruding camera bump on the back

  - Length 35 mm

  - Width 35 mm

  - Depth 4 mm

- 4 microphone ports, used for device B configuration, designated

  - Mic #1

  - Mic #2

  - Mic #3

  - Mic #4

- 2 additional microphone ports, for total of 6 microphone ports, designated

  - Mic #5

  - Mic #6

- HDMI-D Connector on one side (“left face”)

The brief schematic diagram of the device is in figure 4.2.1.2-2

Figure 4.2.1.2-2. – Four-microphone prototype device B

#### 4.2.1.3	Device electro-acoustical characteristics

TBD

### 4.2.2	Device type 2: Three-Microphone Smartphone device

#### 4.2.2.1	General

This three-microphone smartphone is modelled based on a commercially available device that exemplifies current trends in contemporary smartphone design with three built-in microphones

#### 4.2.2.2	Detailed parameters

The parameters of the devices are in the following tables of table 4.2.2.2-1, table 4.2.2.2-2 and table 4.2.2.2-3.

Table 4.2.2.2-1: Device dimensions (mm)

| Height | 163.6 |
| --- | --- |
| Width | 74 |
| Thickness | 8.26 |

Table 4.2.2.2-2: Camera bump size and location (mm)

| Width | 27 |
| --- | --- |
| Height | 33 |
| Thickness | 1.56 |
| xLocation | 6.5 |
| yLocation | 124.1 |

Table 4.2.2.2-3: Microphone port positions on device surface (mm)

| Ch. | Microphone Name | Defining Face | X-axis | Y-axis |
| --- | --- | --- | --- | --- |
| 1 | Bottom Mic | Bottom Face | 25 | 4.16 |
| 2 | Top Mic | Top Face | 62.2 | 3.6 |
| 3 | Camera Mic | Camera Face | 26.5 | 139.9 |

The brief schematic diagram of the device is in figure 4.2.2.2-1.

163.6 mm

74 mm

8.26 mm

M1

M2

M3

M1

Figure 4.2.2.2-1: schematic diagram of the device

#### 4.2.2.3	Device electro-acoustical characteristics

TBD

### 4.2.3	Device type 3: Four-Microphone Smartphone device 1

#### 4.2.3.1	General

This four-microphone smartphone is modelled based on a commercially available device that exemplifies current trends in contemporary smartphone design with four built-in microphones

#### 4.2.3.2	Detailed parameters

The parameters of the devices refer to the following table 4.2.3.2-1.

Table 4.2.3.2-1: the parameters of the device

| No | Device type 3 |
| --- | --- |
| Type | Four-Microphone Smartphone device |
| Size (mm) | 161.4*75.3*9.2 |
| Shape | Rectangular |
| Microphone type | MEMS |
| Number of microphones | 4 |
| Placement of microphones(mm) | Mic1: (-5.2, 80.7, -10.1) Mic2: (-4.9,80.7,9.2) Mic3: (-1.6, -80.2, 7.4) Mic4: (-15.7, -19.2, 0) |

The brief schematic diagram of the device is in figure 4.2.3.2-1.

Figure 4.2.3.2-1: schematic diagram of the device

#### 4.2.3.3	Device electro-acoustical characteristics

TBD

### 4.2.4	Device type 4: Four-Microphone Smartphone device 2

#### 4.2.4.1	General

This four-microphone smartphone is similar to the device 1 in clause 4.2.3, however, there are some differences between the two devices. The parameters and differences are shown in the following clauses.

#### 4.2.4.2	Detailed parameters

The brief schematic diagram of the device is in figure 4.2.4.2-1.

Mic 3

71.5 mm

7.85 mm

38.21 mm

36.73 mm

Mic 4

160.0 mm

Mic 1

Mic 2

Display

Back plate

Figure 4.2.4.2-1: schematic diagram of the device

#### 4.2.4.3	The differences between the Four-Microphone Smartphone device 1 and Four-Microphone Smartphone device 2

TBD

#### 4.2.4.4	Device electro-acoustical characteristics

TBD

### 4.2.5	Device type 5: Three-Microphone XR HMD device

#### 4.2.5.1	General

This XR HMD device is modelled based on a commercially available device that exemplifies current trends in contemporary XR HMD design with three built-in microphones

#### 4.2.5.2	Detailed parameters

The basic information and the microphone positions in table 4.2.5.2-1 are as follows:

- The shape of the proposed XR HMD target device wear on human head

- The microphone placement color-coded as

- Orange: mic 0

- Green: mic 1

- Red: mic 2

Table 4.2.5.2-1: the microphone positions of the device

| Mic # | X | Y | Z |
| --- | --- | --- | --- |
| 0 | 78.55 mm | 46.124 mm | -166.818 mm |
| 1 | 24.027 mm | 11.712 mm | -163.178 mm |
| 2 | -78.457 mm | 46.133 mm | -166.796 mm |

The illustration of the device is in Figure 4.2.5.2-1:

Figure 4.2.5.2-1. schematic diagram of target device

The right-handed coordinate system is defined for the device, and it is shown in Figure 4.2.5.2-2

x

z

y

Figure 2. Right-handed coordinate system for device shape definition and mic placement

#### 4.2.5.3	Device electro-acoustical characteristics

TBD

# 5	High-level technical description.

## 5.1	Input interfaces

TBD

## 5.2	Processing procedure.

TBD

# 6	Detailed algorithm description

## 6.1	General

TBD

## 6.2	Detailed algorithm of example solution 1

TBD

## 6.3	Detailed algorithm of example solution 2

TBD

# 7	Test reports of example solutions

## 7.1	Test setup

TBD

## 7.2	Report of example solution 1

TBD

## 7.2	Report of example solution 2

TBD

# 8	The adaptability of example solutions

## 8.1	General

TBD

## 8.2		Adaptability of example solution 1

TBD

## 8.2		Adaptability of example solution 1

TBD

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-05 | SA4#132 | S4-250853 |  |  |  | Initial skeleton | 0.0.1 |

Change history of this template:

| 2001-07 | Copyright date changed to 2001; space character added before TTC in copyright notification; space character before first reference deleted. | 1.3.3 |
| --- | --- | --- |
| 2002-01 | Copyright date changed to 2002. | 1.3.4 |
| 2002-07 | Extra Releases added to title area. | 1.3.5 |
| 2002-12 | "TM" added to 3GPP logo. | 1.3.6 |
| 2003-02 | Copyright date changed to 2003. | 1.3.7 |
| 2003-12 | Copyright date changed to 2004. Chinese OP changed from CWTS to CCSA | 14.0 |
| 2004-04 | North American OP changed from T1 to ATIS | 1.5.0 |
| 2005-11 | Stock text of clause 3 includes reference to 21.905. | 1.6.0 |
| 2005-11 | Caters for new TSG structure. Minor corrections. | 1.6.1 |
| 2006-01 | Revision marks removed. | 1.6.2 |
| 2008-11 | LTE logo line added, © date changed to 2008, guidance on keywords modified; acknowledgement of trade marks; sundry editorial corrections and cosmetic improvements | 1.7.0 |
| 2010-02 | 3GPP logo changed for cleaner version, with tag line; LTE-Advanced logo line added; © date changed to 2010; editorial change to cover page footnote text; trade marks acknowledgement text modified; additional Releases added on cover page; proforma copyright release text block modified | 1.8.0 |
| 2010-02 | Smaller 3GPP logo file used. | 1.8.1 |
| 2010-07 | Guidance note concerning use of LTE-Advanced logo added. | 1.8.2 |
| 2011-04-01 | Guidance of use of logos on cover page modified; copyright year modified. | 1.8.3 |
| 2013-05-15 | Changed File Properties to MCC macro default. Removed R99, added Rel-12/13. Modified Copyright year. Guidance on annex X Change history. | 1.8.4 |
| 2014-10-27 | Updated Release selection on cover. In clause 3, added "3GPP" to TR 21.905. | 1.8.5 |
| 2015-01-06 | New Organizational Partner TSDSI added to copyright block. Old Releases removed. | 1.9.0 |
| 2015-12-03 | Provision for LTE Advanced Pro logo Update copyright year to 2016 | 1.10.0 |
| 2016-03-08 | Standarization of the layout of the Change History table in the last annex.(Unreleased) | 1.11.0 |
| 2016-06-15 | Minor adjustment to Change History table heading | 1.11.1 |
| 2017-03-13 | Adds option for 5G logo on cover | 1.12.0 |
| 2017-05-03 | Smaller 5G logo to reduce file size | 1.12.1 |
| 2019-02-25 | Replacement of frames on cover pages by in-line text. Clarification of help text on when to use 5G logo. Removal of defunct keywords frame on page 2. Add Rel-16, Rel-17 options, eliminated earlier, frozen, Releases (cover page, below title) Corrections to some guidance text, addition of guidance text concerning automatic page headers under Word 2016 ff. Use of modal auxiliary verbs added to Foreword. More explicit guidance on Bibliography and Index annexes. Converted to .docx format. | 1.13.0 |
| 2019-09-12 | Cover page table outline shown dotted for ease of logo selection. (Author to hide outline after logo selection.) User now needs to delete whole table rows instead of individual cells, which proved to be tricky. Change of style for "notes" in the Foreword to normal paragraphs. Insertion of new bookmarks, correction of location of existing bookmarks. (To improve navigation.) Improvements to guidance text. | 1.13.1 |
| 2021-06-18 | Provision for 5G Advanced logo Update copyright year to 2021 Additional guidance on the use of Heading 8/9 in annexes C, D and X. | 1.14.0 |
| 2022-04-01 | Correction of table formatting Update copyright year to 2022 | 1.15.0 |
| 2023-03-14 | Updated copyright year to 2023 Updated URLs from HTTP to HTTPS Updated FTP link to HTTP counterpart Fixed numbering of annexes | 1.16.0 |
| 2024-03-19 | Updated copyright year to 2024 | 1.17.0 |
| 2024-09-06 | Adds option for 6G logo on the cover page | 1.18.0 |
| 2024-10-15 | Correction of table formatting to ease outline hiding after logo selection. | 1.18.1 |
| 2024-10-18 | Addition/removal of spaces in Release indications and update of Release in the example within the body. Unlock "update field" for ToC. | 1.19.0 |
| 2025-03-17 | Update copyright year to 2025. Emphasize guidelines to hide table borders on coversheet. Adds guidance on editor's notes. | 1.20.0 |