Source:	Xiaomi Technology Spain S.L

Title:	Update DaCAS templates to v2.1

Agenda item:	7.6

Document for:	Discussion & Agreement

# Introduction

Based on contributions and agreements from the past meeting, this document updates DaCAS templates from version 2.0 in [1] to version 2.1 by integrating the update template for target device type in [2] and new target device in [3].

# Recommendations

## 2.1 Recording scenario types

There are three recording scenario types proposed as follows:

Device characterization

- Recordings targeted to identify electro-acoustical characteristics of proposed target devices. For example, to identify device frequency response, etc.

Development recordings

- Recordings targeted to facilitate spatial capture algorithm development and potential tuning.

Evaluation recordings

- Recordings targeted to evaluate example solutions, including subjective and objective evaluation. For objective evaluation, alignment with TS 26.260 is expected.

Above categories should not be understood as strictly orthogonal scenarios, as it is expected that certain scenarios/recordings may be useful for multiple purposes. Furthermore, while other recording scenario types may be anticipated than what is presented here, it is seen useful to consider whether and how the proposed recording scenarios would fit into the above recording scenario types.

## 2.2 Recommendations on defining recording scenarios

Along the above recording scenario types, different recording scenarios for different purposes are anticipated. To ensure definition of meaningful and useful recording scenarios, at least following aspects should be considered when defining new recording scenarios:

- Varying recording scenarios should be represented, from simple single source scenarios to complex scenarios, and from free-field conditions to reverberant conditions.

- For the sake of reproducibility, high quality loudspeakers with well-defined source signals should be favored when possible.

- For device characterization, algorithm development, and objective evaluation purposes, controlled rooms in terms of reverberation and room frequency response should be favored. Referencing existing specifications is recommended, if applicable.

- For subjective evaluation purposes, vast number of varying environments should be represented. Number of proposed recording scenarios in indoor and outdoor environments should be in balance.

- Additional care should be taken when describing and defining the positioning of sound sources and UE.

## 2.3 The relationships of the three templates

The relationships of the three templates are illustrated in Figure 1 below. Furthermore, cross-referencing between the databases should be favored in general, in order to reduce redundancy.

Target device database

Target device No

Type

Size

Shape

…

Recordings database

Recording item No

Target device No

Rec scenario No

UE orientation

…

Recording scenarios database

Recording scenario No

Sound source

Signal characteristics

Playback level calibration

…

Figure 1 – Templates/databases relationship

- Conclusion

It is recommended to agree the following four templates as the formal version 2.1:

- Template for potential target devices-V2.1.xlsx

- DaCAS_Device_Definition_supplement_template2.1.xlsx

- Template for recording setups and scenarios-V2.1.xlsx

- Template for recordings database-V2.1.xlsx

As already agreed, the contents in clause 2 should be considered when designing the databases.

# References

[1] S4aA250015:Update DaCAS templates to v2.0

[2] S4aA250016:[DaCAS] update of Device Definition template

[3] S4aA250019:bytedance_3-mic_XR_HMD_target_device