Source:	Nokia

Title:	Very low bitrate listening test results

Document for:	Discussion

Agenda Item:	7.9 – FS_ULBC

# 1. Introduction

There is an ongoing study “Study on Ultra Low Bitrate Speech Codec”, where two of the objectives are defined as [1]:

- Provide some evidence that the design criteria can be met, for example existing reference codecs

- Identify relevant reference codecs for comparison and evaluation purposes.

One of the assumptions of the study is total transmission data rate of [1-3] kbit/s. There are several traditional DSP based voice codecs and ML/AI based codecs available as described in “Analysis of existing technologies for ULBC” [2].

In this light Nokia has performed two listening tests. The first listening test is based on extACR5 MOS (extended Absolute Category Rating Mean Opinion Score) test methodology and second one is using DCR (Degradation Category Rating) test methodology for a subset of the conditions of the first test. Listening test material was Finnish clean speech.

# 2. Overview

There are several traditional (DSP based) speech codecs available for very low bit operation as described in [2]. They are commonly considered as vocoder-style codecs. They provide understandable voice quality with somewhat limited perceptual quality, often sounding a bit “synthetic”. Their bandwidth is usually limited to narrowband (4 kHz).

On the other hand there are several AI/ML based codecs coming from various universities, researchers, and companies working in the ML area. Some of these codecs target very low bitrates, e.g., 0.5 to 1 kbit/s and some aim at a bit higher operation points, e.g., 1-6 kbit/s. Some are optimized for voice, some are even trained only with certain language. Some codecs are optimized for general audio.

Different ML codecs use varying sampling rates and some of those are different to the common 3GPP sampling rates. Thus we included different bandwidth limits to the listening test. Bandwidths included are:

- Narrowband (NB) 4 kHz

- Mediumband (MB) 6 kHz

- Wideband (WB) 8 kHz

- 10 kHz

- Semi-Super Wideband (SSWB) 12 kHz

- Super Wideband (SWB) 16 kHz

- Fullband (FB) 20 kHz

For the listening test three DSP based vocoder style codecs were chosen:

- Codec2 0.7, 1.3, 2.4, and 3.2 kbit/s, tested using ffmpeg [3], [4]

- MELP 2.4 kbit/s [5]

- MPEG4 HVXC 2.0 and 4.0 kbit/s [6]

3GPP solutions at slightly higher bitrates are represented by:

- AMR 4.75 and 7.95 kbit/s

- AMR-WB 6.6, 8.85, and 12.65 kbit/s

- EVS-NB 5.9 and 7.2 kbit/s

- EVS-WB 5.9, 7.2, and 9.6 kbit/s

- EVS-SWB 9.6 and 13.2 kbit/s

Finally tested ML/AI based codecs include:

- DAC (Descript Audio Codec) 44k 0.9, 1.7, 2.6, 3.4, and 6.9 kbit/s [7]

- TSAC (Modified version of DAC) 44k 0.6, 1.2, 2.5, 3.2, and 5.9 kbit/s [8]

Bolded bitrates and bandwidths were tested in both ACR and DCR listening tests.

# 3. Discussion

New ML based codecs or enhanced variants of earlier codecs are being published basically every month. Nokia has tested several other ML codecs in addition to the tested codecs presented in this input, and the overall conclusion on the current state is that making them work stable enough for practical use is quite difficult. They often rely on certain Python version, several Python libraries and possibly certain version of CUDA acceleration and corresponding hardware. Making them run requires often many hours or even days of work and may finally fail to process the listening test material with some error. For example, some ML codecs consume huge amounts of memory depending on how long the sample is. Even reasonable length audio sample (1 minute) causes crashing. There exists almost infinite amount of combinations of different versions of Python, Python libraries and CUDA derived libraries and the versions and dependencies are generally not well documented.

The sizes of ML codec models vary wildly. Some of them are reasonably sized (e.g., few tens of megabytes), but sometimes they are several gigabytes in size.

For codecs in current tests:

- Descript-audio-codec uses “weight.pth”-file of size 292MB.

- TSAC codec apparently has for mono operation two files “dac_mono_q8.bin” and “tsac_mono_q8.bin” for a total of 128MB.

Computation time is quite hard to measure. It often depends hugely on the platform the software is being run on. If run on a single thread CPU or on a CUDA accelerated platform the run time may vary hundred-fold. Both tested ML-based codecs processed the test material in reasonable time even without GPU acceleration.

4. Listening test Results

Nokia conducted two listening tests using clean speech material. Clean speech consisted of Finnish language sentence pairs spoken by three males and three females, four sample pairs from each. The listening was conducted diotically using Sennheiser HD650 headphones in quiet listening booths. All listeners were experienced listeners, most of them were experts in the field of speech and audio processing. The first listening test used extended ACR5 test methodology, which is straightforward extension of standard P.800 ACR test [9]. The voting scale is extended as shown in Table 1 and Figure 1.

Table 1 Extended ACR5 voting scale

| Grade | Quality |
| --- | --- |
| 5.5 |  |
| 5 | Excellent |
| 4.5 |  |
| 4 | Good |
| 3.5 |  |
| 3 | Fair |
| 2.5 |  |
| 2 | Poor |
| 1.5 |  |
| 1 | Bad |
| 0.5 |  |

Figure 1 Example of extended ACR 5 voting scale in the listening test software.

## 4.1 Extended ACR5 listening test results

ACR listening results in Figure2 and Figure3 show that increased signal bandwidth increases perceptual quality up to a certain point. The high-frequency perception is very personal and also depends on hearing. Overall it seems that 12 kHz signal bandwidth is in the saturation region in this test. On the other hand it can be seen from the bandwidth experiment that 4 kHz signal bandwidth significantly limits the perceived quality of speech.

Regarding DSP based vocoders, MELP 2.4k and MPEG4 HVXC perform better than Codec2. From quality point of view, they could be considered as reasonable references representing DSP based vocoders.

3GPP standard codecs AMR, AMR-WB and EVS at their lowest supported bitrates perform as expected and provide good targets for performance.

Finally ML-based codecs DAC and TSAC have very good performance in clean speech, when considering their low bitrates. TSAC has somewhat better quality than DAC, which is in line with the web-page description, where they state that TSAC is an improved version of DAC. Both tested codecs have poor quality at their lowest tested bitrates below 1 kbit/s. However TSAC starting at 1.2 kbit/s and DAC starting at 1.7 kbit/s could be considered to represent ML-based codec references. Both of them were relatively easy to make run without issues.

Figure 2 Extended ACR5 listening results of low bit rate voice codecs in clean speech.

Figure 3 Extended ACR clean speech listening results with 14 listeners in curve format and logarithmic bitrate.

## 4.2 DCR listening test results

DCR listening test was constructed from a subset of conditions of the ACR listening test. The direct reference was the full-band signal. As can be seen listeners are more likely to notice degradations, when there is reference available. Overall the results are in line with the ACR test, with the exception that MELP is preferred over HVXC 2.0 in DCR test, when it is the other way around in the ACR test. The likely reason is that MELP has full 4 kHz bandwidth, but HVXC 2.0 has  somewhat limited signal spectrum to approximately 3.7 kHz, and it therefore sounds a bit more muffled.

Figure 4 DCR listening test results of low bit rate voice codecs with clean speech

# 5. Summary

We have presented FS_ULBC relevant listening test results both with extended ACR5 and DCR test methodologies. Some of the tested codecs could be considered to be used as reference codecs for fortcoming FS_ULBC work.

The source considers that the results can be documented in FS_ULBC in a suitable document, e.g., planned Pdoc.

# References

[1]	Tdoc SP-250378, “Study on Ultra Low Bitrate Speech Codec”

[2]	Tdoc S4-250565, “Analysis of existing technologies for ULBC”

[3] 	https://github.com/drowe67/codec2

[4]	https://ffmpeg.org/

[5]	https://melp.org/

[6]	https://en.wikipedia.org/wiki/Harmonic_Vector_Excitation_Coding

[7]	https://github.com/descriptinc/descript-audio-codec

[8]	https://bellard.org/tsac/

[9]	ITU-T P.800 : Methods for subjective determination of transmission quality (08/1996)