Agenda item: 	9.8

Source: 	Qualcomm Inc.

Title: 	[FS_ARSpatial] Evaluation of 3D Reconstruction

Document for	Discussion and Agreement

# Introduction

In this contribution, we describe a plan to evaluate the 3D reconstruction function and compare on-device to remote 3D reconstruction.

# 3D Scene Reconstruction

## 2.1	Overview

Augmented Reality (AR) and Extended Reality (XR) headsets and glasses rely on 3D scene reconstruction to understand their surroundings. This function continuously creates a digital 3D representation (a spatial map) of the user’s environment so virtual content can interact realistically with the real world. Modern AR devices perform simultaneous localization and mapping (SLAM) to track their own position while mapping surfaces, obstacles, and features around the user. The reconstruction typically produces a spatial mesh or point cloud of real-world surfaces in real time.

3D Scene reconstruction is considered a very intensive computer vision task and is a baseline for several of the other AR spatial functions described in this TR.

## 2.2 Evaluation

To understand the trade-offs of running this function on device versus delegating it to a companion device or an edge server, we outline a plan for evaluating the complexity, latency, and bandwidth requirements for such a function.

For comparing the performance, we propose the following:

- On a PC that serves as example for an edge server, we run a 3D scene reconstruction script using the Open3D library and a web or stereo camera,

- On an Oculus ® device, we run a script that uses OpenXR API and the XR_FB_scene extension to capture and store a mesh of the environment.

Both scripts should perform the following:

- measure the time taken to reconstruct or update the 3D scene,

- store the reconstructed mesh for evaluation and comparison,

- adjust the update rate of the environment mesh to perform stress tests where the maximum update rate is to be determined.

The evaluation operation should compare the quality of the reconstructed scenes by comparing aspects like:

- number of vertices,

- number of faces,

- mesh smoothness,

- labelling accuracy,

- etc.

# Proposal

We propose to document the evaluation procedure in a new clause of TR 26.819 and to perform the evaluation by the coming meeting.

# References

[1]	 	3GPP TR 26.819, Study on Spatial Computing for Augmented Reality (AR)