Source:	Ericsson LM

Title:	Proposed corrections to the IVAS BASOP Codec

Document for:	Discussion

Agenda Item:	7.5

# Introduction

The verification of the IVAS BASOP Encoder has revealed issues that do not comply with the requirements as specified in clause 3 of [1], which are also attached to the contract between Ittiam and ETSI. This document lists the proposed corrections to meet the requirements of the encoder. Further, there are still open issues on the IVAS BASOP decoder and renderer, summarized in Annex A of [1]. A summary of the open issues in the issue tracker is available in clause 4 of this document.

# Proposed corrections to the BASOP Encoder

| Issue category | Description | Correction |
| --- | --- | --- |
| Complexity and memory | The IVAS BASOP encoder is, on average, 1.45 times more complex than the IVAS FLOAT encoder, whereas the target ratio, based on the EVS encoder, is 0.80. In the worst-case scenario, the IVAS BASOP encoder is 2.48 times more complex than the IVAS FLOAT codec. The corresponding number for EVS BASOP vs EVS FLOAT is 0.90. The average encoder HEAP memory consumption of the IVAS BASOP codec is 0.89 times higher compared to the IVAS FLOAT codec, while the target value derived from the EVS codec is 0.53. Similarly, the maximum encoder STACK memory consumption of the IVAS BASOP codec is 1.14 times higher than that of the IVAS FLOAT codec, with the target value from the EVS codec being 0.75. |  |
| SBA and OSBA low bitrates (1TC) | Elevated MLD scores and several open issues related to audible quality degradations. |  |
| DTX | The DTX activity pattern differs between BASOP in relation to float. In some cases CNG is triggered in the wrong segments, leading to degradations. |  |
| Energy calculation | There seems to be precision issues in general for calculating signal energies. In MDCT stereo the power spectra and energy is calculated in quite a few places that are essential for the whole processing chain, that is for SNS, band-wise M/S decision (see above) and IGF to name the most prominent ones. It seems that one single common exponent/q is chosen for both MDCT and MDST spectra despite that they may have large differences in magnitude. While calculating energies these dynamic differences double, leading to very low precision values and not representable values. One potential solution would be to not use a common exponent/q. Also for the representation of Mid/Side spectra it also might make sense to use separate exponent/q values because the magnitude of Mid and Side data is inherently very different. |  |
| Stereo MS decision | There seem to be an issue with the bandwise M/S decision that is the core of the MDCT stereo coding. This is reflected in both issues #1464 and #1308. As described in issue #1464 this comes from different estimations of the quantization gains and the cause of that is precision issues on the energy calculation. Latest observation with checking SHA version 1ccc3c58 shows an improvement for LTV but still there are some frames with high deviation |  |
| Bandwidth extension | Various open issues pointing towards problems with bandwidth extension, for example, as reported in issues #1091, #1475, #1493, #1510, or #1530. |  |
| S/M classifier | Precision issues in the speech/music classifier impacts the core coder decision which was observed to degrade several signals. |  |
| Various discontinuities in signal, especially but not limited to 16 kHz sampling rate | Various discontinuities are present in the encoded signal, especially but not limited to 16 kHz sampling rate of the input signal. These issues have for example, been reported in issues #1388, #1392, and #1457. |  |

# Proposed corrections to the BASOP Decoder and Renderer

There are still open issues to be resolved from the Decoder and Renderer verification Report in Annex A of [1] and in the issue tracker at 3GPP Forge [3]. A summary table from [1] is found below.

| Issue category | Description | Correction |
| --- | --- | --- |
| Complexity and memory | The IVAS BASOP codec is, on average, 1.85 times more complex than the IVAS FLOAT codec, whereas the target ratio, based on the EVS codec, is 0.76. In the worst-case scenario, the IVAS BASOP codec is 4.88 times more complex than the IVAS FLOAT codec. The corresponding number for the EVS BASOP vs EVS FLOAT is 0.90. The average decoder HEAP memory consumption of the IVAS BASOP codec is 1.09 times higher compared to the IVAS FLOAT codec, while the target value derived from the EVS codec is 0.58. Similarly, the maximum decoder STACK memory consumption of the IVAS BASOP codec is 1.86 times higher than that of the IVAS FLOAT codec, with the target value from the EVS codec being 0.73. |  |
| Packet loss concealment | Significant quality issues for packet loss concealment have been observed (clicks, discontinuities), e.g. #949, #954, #977, #978, #1000, #1008, #1014, #1021, #1022, #1028, #1031, #1039 |  |
| High noise floor compared to float | Issue present even at 512 kbps. See #951, #1015. |  |
| Saturation | Several operating modes show audible distortion with high-level input signals. See e.g. Figure 17 and the daily reports in [6]. |  |
| Crashes | Increased number of crashes were found during verification, especially for high-level input. The codec should be robust also for these inputs. Related to high-level input. |  |
| Regressions | For a few cases regressions were noticed during the verification process, e.g. for packet loss concealment, where new artifacts were appearing as fixes were brought in. |  |
| Test coverage | For non-mainstream operation, more issues have been found, e.g. rate-switching, bandwidth switching, other input bandwidths (not 48 kHz). |  |
| High MLD issues | - Some test cases were observed to give high MLDs, indicating audible artifacts which were later confirmed by manual inspection of the signals. - OSBA at low bitrates should decode near-identically to SBA if the content is equivalent. However, higher MLD in OSBA decoder is observed. |  |
| Degradations found in listening tests | - Stereo at 48kbps with FER and DTX, significant degradation for one item and degradation for overall score (see 6.1.3.2) - OSBA (HOA3 with ISM4) at 320kbps with FER, statistical significant degradation for one item and overall score (see 6.10.2.1.2) |  |
| Low SSNR issues | Very low SSNR cases observed for the following operating points: - MC 7.1+4 @128kbps (ParamMC) - range 26-30dB (see 6.6.2.1) - ISM 3,4 @32,48 kbps (ParamISM) – range 23 – 30dB (see 6.7.2.1) - OSBA (HOA3, HOA2, FOA) – range 12-30dB (see 6.10.2.1.1, 6.10.2.2.1, 6.10.2.3.1) |  |

# Overview of open issues at 3GPP Forge

The issue tracker at 3GPP Forge [3] is continuously being updated, with problems being reported and resolved. This is a snapshot reflecting the current status on May 13, 2025. A few of the oldest issues were filed 7-11 months ago.

Table 1: Open issues per module

| Category |  | Number of issues |  |
| --- | --- | --- | --- |
| Assigned to Ittiam | 89 | Encoder | 56 |
|  |  | Decoder/Renderer | 30 |
|  |  | Other | 3 |
| Assigned to PC, or unassigned | 70 | Encoder | 25 |
|  |  | Decoder/Renderer | 24 |
|  |  | Other | 21 |

Table 2: Open issues with prioritization labels

| Category |  | Priority label |  |
| --- | --- | --- | --- |
| Assigned to Ittiam | 89 | Blocker | - |
|  |  | Critical | 9 |
|  |  | Major | 39 |
|  |  | Medium | 30 |
|  |  | Minor | 4 |
|  |  | No label | 7 |
| Assigned to PC, or unassigned | 70 | Blocker | - |
|  |  | Critical | 10 |
|  |  | Major | 22 |
|  |  | Medium | 14 |
|  |  | Minor | 7 |
|  |  | No label | 17 |

# References

- IVAS BASOP Verification (IVAS-10), https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_132_Fukuoka/Docs/S4-250915.zip

- Verification of IVAS BASOP Encoder, https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_132_Fukuoka/Docs/S4-250914.zip

- IVAS BASOP repository on 3GPP Forge, https://forge.3gpp.org/rep/sa4/audio/ivas-basop