Agenda item: 	10.7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] Discovery and selection of Avatar

Document for	Discussion and Agreement

# Introduction

In this contribution, we propose changes to specify the discovery and selection of the base avatar and the assets.

# Proposed Changes

### 8.4.1 Avatar and Asset Selection

To facilitate the selection and personalization of the avatar of a user during an AR call, the UE starts by downloading a uniquely identified Avatar configuration application. This application is delivered via the bootstrap data channel established between the UE and the IMS network. Once the UE initiates this data channel, the Media Function (MF) routes the request towards the Data Channel Signaling Function (DCSF), which hosts the pre-configured configuration app for that specific subscriber. Through this bootstrap process, the user obtains the Avatar configuration app, enabling the selection and customization of their avatar.

Utilizing this Avatar configuration application, the user is able to browse through a variety of predefined avatars and customize their appearance by selecting clothes, accessories, and other visual attributes from the ARF (Avatar Representation Format) container associated with each avatar. The application allows intuitive experimentation with different visual elements until the user's avatar precisely reflects their identity or intended persona.

After customization, the UE consolidates the user's choices into an Avatar Selection Document. This document is a structured JSON message identified by the type "urn:3gpp:ar:avatar:v1:selection", containing two essential components: the Avatar ID, which uniquely identifies a base avatar stored in the BAR (Base Avatar Repository), and a comprehensive list of asset names representing all selected visual elements from the corresponding ARF container for the chosen avatar. Upon creation, this Avatar Selection Document is transmitted to the Media Function (MF) over the application data channel. The MF retrieves the base avatar from the BAR based on the Avatar ID and user-selected assets listed in the document. The finalized avatar is then made available for download over the application data channel to other call participants.

JSON Schema for Avatar Selection Document (type: "urn:3gpp:ar:avatar:v1:selection"):

| Property Name | Type | Usage | Description |
| --- | --- | --- | --- |
| avatarId | string | M | Uniquely identifies the base avatar stored in the BAR (Base Avatar Repository). |
| assets | array(string) | M | Lists all selected asset names from the ARF container associated with the chosen Avatar ID. Each asset name corresponds to a visual element such as clothing, accessories, or appearance customizations. |

An example of such a message is as follows:

{ "avatarId": "avatar12345", "assets": [ “full_body”, "blue_shirt", "black_pants", "sneakers_white", "glasses_round", "watch_silver" ] }

# Proposal

We propose to agree the content of section 2 and add it to the base CR.