Source:	Nokia

Title:	Proposal on ambience transmission testing

Agenda item:	7.7 (ATIAS_Ph2)

Document for:	DISCUSSION & AGREEMENT

# 1. Introduction

In the last couple of SA4 meetings, a concept on testing UE ambience transmission has been widely discussed. Multiple important aspects are raised, such as determining the applied UE noise suppression mode, different possible tests in high level, as well as applied ambience sound field simulation methods

In this contribution, proposals on the envisioned changes for 3GPP TS 26.260 [1] and 26.261 [2] are presented and proposed for ATIAS-2 permanent document, for further integration to the specifications.

# 2. Discussion

For successful definition of ambience transmission tests for IVAS UEs, at least following aspects need to be clarified and defined in the 3GPP TS 26.260:

- Ambience sound field simulation system

- Applied ambience signals

- UE arrangement in the ambient sound field test setup

- UE and reference client configuration in terms of applied capture type (noise suppression)

- Test methods for assessing the ambience transmission

- Corresponding requirements

# 2.1 Ambience sound field simulation, signals and UE mounting

Regarding the ambient sound field simulation, ETSI STQ indicated in their liaison statement [3] an interest to adapt existing background noise simulation system defined in TS 103 224 [4] for immersive UE testing. However, it is expected that there are no immediate changes planned for the specification within the ATIAS_ph2 timeline.

However, it is considered that the existing setup can be used for generating the simulated sound field for ambience transmission tests with IVAS based UEs. Test system and/or signals can be updated and modified accordingly, once any specific updates for immersive UE testing are available.

On the applied ambience sound fields, number of different noise conditions are defined in TS 103 224. Only ambient sound field that do not contain any background speech / bubble noise is proposed at this point, as such alignment is considered helpful for defining tests comprising both (near-end) speech and background ambience in the future.

For the UE mounting to the ambient sound field setup, currently mounting directly to the geometric center of the simulation setup is seen viable. Further definitions for mounting of specific UE types could be considered.

# 2.2 UE configuration

When testing the ambient transmission of the UE, the noise suppression will have a significant impact on the obtained behavior. It is anticipated that IVAS based UEs may support multiple different capture types in terms of applied amount of noise suppression. Thus, it is important to determine how much noise suppression the UE is expected to apply. For this determination, SDP based approach has been discussed. Efforts for defining such SDP parameter are currently ongoing in the other SA4 Audio work items. Alternatively, manufacturer may define/indicate how much noise suppression the UE applies.

In the following figure, an example for negotiating the applied capture type for testing is illustrated. In the left-hand side possible options of UE SDP offer for applied capture type are presented. Based on the indicated support and preference, reference client can answer with the desired capture type for anticipated test. Requirements are then checked against the corresponding test and negotiated capture type.

Figure 1 Illustration of possible SDP negotiation for assessed capture type. Options a)-f) examples of possible different offers by UE under test.

# 2.3 Test methods and requirements

Regarding the relevant test methods, in [5] possible different tests for assessing the ambience transmission were presented in high level. For robust assessment, simple and predictable conditions and metrics should be applied. Thus, at least loudness, frequency response and temporal characteristics of transmitted ambience can be reliably assessed. Further considerations on the sufficient assessment method for testing the spatial characteristics of the transmitted ambience are under consideration. It is possible, that more accurate sound field simulation method may be required for conducting such assessments reliably.

Requirements for the corresponding test methods are also formulated, based on the assessed capture type. For now, all the requirements are left TBD. However, initial views on the relative differences between the requirements for different capture types are included.

Based on the above discussion, following changes are proposed to be included to the ATIAS-2 permanent document [6] for further integration to the 3GPP TSs 26.260 and 26.261.

# 2.4 Proposed changes to ATIAS-2 permanent document

Change for TS 26.260:

Start change

## 3.1	Definitions

Capture type: Indication on the applied noise suppression type by the UE.

Transparent capture type: TBD

Nominal capture type: TBD

Suppressed capture type: TBD

5.x.x.x Setup for ambient sound field simulation

A setup for simulating realistic ambience sound field in a lab-type environment is described in ETSI TS 103 224 [x]. If not stated otherwise this setup is used in all measurements where ambient sound field simulation is required. The

following ambient sound signals of ETSI TS 103 224 [x] shall be used.

Table X Applied ambient sound field signals in hand-held hands-free or binaural mode as specified in TS 103 224

| Name | Description | Length (HHHF/Binaural) | Hands-free levels | Binaural levels |
| --- | --- | --- | --- | --- |
| Crossroad noise (Crossroadnoise) | HATS and microphone array standing outside near a crossroad | 30s/35s | 1: 69,9 dB 2: 69,6 dB 3: 69,6 dB 4: 69,9 dB 5: 69,6 dB 6: 69,5 dB 7: 69,6 dB 8: 69,7 dB | 1: 68,3 dB 2: 68,6 dB 3: 68,3 dB 4: 67,7 dB 5: 66,8 dB 6: 66,4 dB 7: 66,7 dB 8: 66,7 dB |
| TBD | TBD | TBD | TBD | TBD |

NOTE: The used test system for generating the ambient sound field is anticipated to be replaced/improved, once such system for immersive UE testing is available.

For inserting ambient sound field signal to electrical interface with Binaural input, binaural signals from clause 8.2 in ETSI EG 202 396-1 [x] shall be used. The input signal shall be calibrated to [-26 LFKS], as described in clause 5.5.1. For inserting ambient sound field signal to electrical interface in other input formats is TBD.

x.x.x Capture type configuration

UEs may support multiple capture types in terms of noise suppression in sending direction. All supported capture type shall be tested in send direction supported by the UE, which is selected according to the following priority:

- Supported capture types, as indicated during negotiation in SDP

- Supported capture types specified by the manufacturer

The tested capture types shall be documented in the test report.

### 5.3.2	System simulator and reference client

The system simulator configuration and radio conditions on the air interface shall be as specified in clause 4.2 of 3GPP TS 26.132 [27]. Unless otherwise specified for the respective test, the system simulator shall provide an error-free radio connection to the UE under test.

The UE shall be connected to a system simulator and test equipment supporting the IVAS codec [28]. Unless specified otherwise, 48 kHz sampling rate and fullband mode shall be used.

Since UEs may provide different capabilities for sending (capture) and receiving (rendering) direction, a bidirectional communication may not be symmetric regarding IVAS Coded Formats and corresponding bitrates.

During negotiating and exchange of session parameters via Session Description Protocol (SDP), the UE advertises its supported and preferred Coded Formats and capture types in sending and receiving direction, as well as corresponding bitrate ranges.

-	The reference client shall advertise via SDP the envisioned IVAS audio format and capture type for testing sending and receiving direction along with corresponding bitrate as specified in Table 1 as the preferred and only capabilities.

-	For sending direction, the decoder in the reference client shall be configured for the EXT decoder output format, if not specified otherwise by any test method.

-	For receiving direction, the encoder in the reference client shall be configured for the Coded Format envisioned for testing.

5.6.5 Loudness (Ambience)

- The default arrangement for an ambient sound field generation as defined in clause [5.x.x.x] is set up. The UE under test and the reference client are connected and configured as described in the clause 5.3.2.

- Each ambient noise condition described in Table X of clause [5.x.x.x] is measured.

- For acoustic measurements, the UE is mounted in the geometric center of the loudspeaker array. Further information about the UE mounting is described in clause 5.4.2. The test signal is played back via the acoustic source.

For electrical interface, the test signal is played back via insertion into the electrical interface.

- The renderer in the reference client shall be configured to Mono output.

- The UE loudness in sending direction is obtained by Send Loudness Rating (SLR) according to ITU-T P.79 [13] in wideband mode. The calculation is carried out as specified in clause 8.2.3.1 of 3GPP TS 26.132 [27], except that the spectrum at the position of UE under test and the actual level at the position of the UE under test (measured in 1/3-octaves) are used as references to determine the sending sensitivity SmJ.

5.6.6 Frequency response (Ambience)

- The default arrangement for an ambient sound field as defined in clause [5.x.x.x] is set up. The UE under test and the reference client are connected and configured as described in the clause 5.3.2.

- Each ambient noise condition described in Table X of clause [5.x.x.x] is measured.

- For acoustic measurements, the UE is mounted in the geometric center of the loudspeaker array. Further information about the UE mounting is described in clause 5.4.2. The test signal is played back via the acoustic source.

For electrical interface, the test signal is played back via insertion into the electrical interface.

- The frequency response of the decoded output signal shall be calculated according to the format-specific definitions of the clause 5.6.3.2. For all formats, 1/12th octave intervals as given by the R40 series of preferred numbers in ISO 3 [6], for frequencies from 100 Hz to 12 kHz (inclusive) apply. The reference magnitude spectrum is the 1/12th octave spectrum of the mono downmix of the test signal.

    - Temporal characteristics (Ambience)

- The default arrangement for an ambient sound field as defined in clause [5.x.x.x] is set up. The UE under test and the reference client are connected and configured as described in the clause 5.3.2.

- Each ambient noise condition described in Table X of clause [5.x.x.x] is measured.

- For acoustic measurements, the UE is mounted in the geometric center of the loudspeaker array. Further information about the UE mounting is described in clause 5.4.2. The test signal is played back via the acoustic source.

For electrical interface, the test signal is played back via insertion into the electrical interface.

- The renderer in the reference client shall be configured to Mono output.

- The decoded and rendered signal is referred to the mono downmix of the test signal as level versus time analysis according to IEC 61672 [x], using an integration time of [250 ms]. The result represents the temporal level differences between the transmitted and reference ambient sound field.

]	End change

Change for TS 26.261:

Start change	[

5.3	Loudness

5.3.1 Loudness for single source

Existing text from 5.3

5.3.2 Loudness for ambient source

The nominal values of SLR to the POI shall be with transparent capture type:

SLRTransparent = [TBD ± 3 dB]

The nominal values of SLR to the POI shall be with nominal capture type:

SLRNominal = [(TBD + 6) ± 3 dB]

The nominal values of SLR to the POI shall be with suppressed capture type:

SLRSuppressed > [(TBD + 9) dB]

Compliance shall be checked by the relevant tests described in TS 26.260.

5.4 Frequency response

5.4.1 Frequency response for single source

5.4.2 Frequency response for ambient source

5.4.1.2 Acoustical interface

The requirements for acoustical interface testing with Transparent capture are given in Table 5.4.2.1a

Table 5.4.2.1a: Sending sensitivity/frequency requirements table for acoustical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 4000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

The requirements for acoustical interface testing with Nominal capture are given in Table 5.4.2.1b

Table 5.4.2.1b: Sending sensitivity/frequency requirements table for acoustical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 4000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

No requirements for Suppressed capture.

5.4.1.3 Electrical interface

TBD

Compliance shall be checked by the relevant tests described in TS 26.260.

5.6 Temporal characteristics

Transparent capture: The differences [shall] be less than [± TBD dB]

Nominal capture: The range of differences [shall/should] be less than [± (TBD+3) dB]

Suppressed capture: The range of differences [shall/should] be higher than [± (TBD+3) dB]

]	End change

# 3. Conclusion

In this document, an overview of required changes to the 3GPP TS 26.260 and TS 26.261 specifications are discussed. Based on the discussion, changes in section 2 are proposed to be integrated to the ATIAS-2 permanent document [6].

# 4. References

[1]	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive audio systems”, Release 18

[2]	3GPP TS 26.261: “Terminal audio quality performance requirements for immersive audio services”, Release 18

[3]	S4-250497: “Reply LS on extending ETSI TS 103 224 for testing immersive UEs”, ETSI TC STQ

[4]	ETSI TS 103 224: “Speech and multimedia Transmission Quality (STQ); A sound field reproduction method for terminal testing including a background noise database”

[5]	S4-250205: “On testing UEs in ambience”, Nokia

[6]	S4-250082: “Permanent Document ATIAS-2 v0.2”