Source:	Nokia

Title:	On applied test signal for AEC assessment

Document for:	DISCUSSION & AGREEMENT

Agenda item:	7.7 – ATIAS_Ph2

# 1. Introduction

One of the Work Item objectives for ATIAS_Ph2 was definition of test methods for assessing the acoustic echo control. Initial views and proposal on the test method for assessing terminal coupling loss were discussed on [1] and [2]. Based on these documents, a candidate test method proposal was integrated to the ATIAS-2 permanent document [3].

Further considerations on dynamic movement within the test signal was left for further considerations. In this document, an alternative set of applied source directions for the applied test signal in proposed Acoustic echo control test. Instead of earlier proposed static directions, more diverse spatial scene including both static directions and dynamic movement is proposed to be considered.

# 2. Discussion and proposal

# Currently, the proposed test signal for Acoustic echo control test utilizing Terminal coupling loss includes only static directions per talker. While these directions are rather well covering immersive aspects (e.g., realistic talker directions changes and realistic spatial directions), considerations regarding the dynamic movement were discussed during the integration of the proposal to the ATIAS-2 permanent document [3].

# Current approach in proposed Acoustic Echo Control is to use compressed British-English single talk sequence described in the ITU-T p.501 with virtually positioning the input according to the applied input format, as defied in clause 5.5.2. in TS 26.260. A set of static source directions are applied for each sentence of the test signal. The initially proposed directions can be found from the Table T1 in ATIAS-2 permanent document [3].

# To further improve immersion of the applied test signal, dynamic movement should be included to the applied input of the test. On the other hand, different static directions per talker and abrupt direction changes between talkers are seen equally relevant spatial events within an immersive call, which the UE should be able to handle.

# Thus, following test signal positions presented in the Table 1 are proposed to be applied for the test input:

Table 1: Source direction of each sentence in test signal

| Direction | Talker | Source direction in azimuth [°] |
| --- | --- | --- |
| 1 | M1, M4 | 0 |
| 2 | M2, M5 | Linear movement from 0 to 90 |
| 3 | M3, M6 | 90 |
| 4 | F1, F4 | Linear movement from 90 to -90 |
| 5 | F2, F5 | -90 |
| 6 | F3, F6 | 180 |

# Similarly as in the initial proposal, the different source directions and dynamic movements would be applied to individual sentences (talkers) in the test signal. Movement would be simulated by fitting a line between the start and end directions of the sentence. For example, when considering Direction 2 in the table (Talkers M2 and M5), the applied direction (talker path) per frame of the input signal would be estimated from the linearly interpolated line starting from 0° and ending at 90° direction in azimuth. Further details on the exact sentence start and end time instances may be included to the table above. The relation between presented talker path and compressed British-English single talk sequence are visualized in the Figure 1 below.

Figure 1 Compressed British-English single talk sequence and proposed talker path with dynamic movement

# 3. Conclusion

# In this document further considerations on the source directions for test signal of previously proposed Acoustic Echo Control test are presented. New source directions including dynamic movement are proposed to be included to the ATIAS-2 permanent document [3] as the utilized source directions of the test input.

# 4. References

[1]	S4-242015: “On acoustic echo control testing and on UE type combinations”

[2]	S4-241840: “Test methods for UE acoustic echo control”

[3]	S4-250082: “Permanent Document ATIAS-2 v0.2”

[4]	ITU-T P.501 (05/2020): "Test signals for use in telephonometry".