Agenda item: 	10.7

Source: 	Qualcomm Inc.

Title: 	[AvCall-MED] SDP Signaling

Document for	Discussion and Agreement

# Introduction

A user that wishes to offer its avatar during an AR call, can do so in 2 different ways:

- through a shared scene description, where it adds its avatar as a node to the scene graph,

- through SDP signaling for the case where a shared scene is not available

In this contribution, we propose the SDP signaling enhancements to allow a UE to offer its avatar and animation streams to other participants in a call.

# Proposal for SDP Signaling

## 7.3 	SDP Negotiation and Signaling of Avatars

An AR-MTSI client in terminal may include a data channel media description for an application data channel that provides the following data:

- offer an avatar with an indication of the selected base avatar

- offer avatar animation streams

When establishing the application data channel for avatar communication, the following applies:

- the dcmap sub-protocol shall be identified as “3gpp-ar-metadata”

- the req-app-id parameter of the “a=3gpp-req-app” shall be set to “urn:3gpp:ar:avatar:appid”

- The “a=dcsa” attribute shall be present and shall include the “base-avatar-id=” parameter, which provides a temporary base64 encoded identifier of the base avatar, including the asset selections.

The base-avatar-id is obtained after the discovery and configuration of the base avatar and shall be assigned by the IMS network (e.g. BAR or MF).

The answering UE that supports avatar communication and that desires to accept the offer shall do so by keeping the accepted “a=3gpp-req-app”. Otherwise, it shall reject the SDP m= line media description by setting port to 0 on that m=line.

The answering UE may use the received “base-avatar-id” to request the download of the base avatar of the offering UE by directly issuing an HTTP download request to the MF.

Editor’s Note: The security aspects of this download request are to be defined by SA3.

# Proposal

We propose agree the content of section 2 to the base CR.

# References

[1]	 	3GPP TS 26.264, IMS-based AR Real-Time Communication